3GPP TSG-RAN WG4 Meeting #113	R4-2418841

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.6.2

Source:            	Google

Title:                	Discussion on methods for reducing the number of UE Rx chains for fragmented carriers

Document for:	Approval

# Introduction

In RAN#104, a new SID for “Study on NR FR1 DL Fragmented Carriers” was approved as one of Release 19 work items [1]. The objective in the SID is to identify methods for reducing the number of UE Rx chains for two CCs DL NCCA in the single band with frequency span smaller than 100MHz, which is shown as follows. In this contribution, we will focus on the non-RF requirement related yellow part for the fragment carriers study.

### 4.1 Objective of SI or Core part WI or Testing part WI The objective of this study is as follows: - Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering: - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - Other requirements are not precluded, if identified - The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2]; - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4]; - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4] - Impacts on DL performance [RAN4] ; - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2]. NOTE 1: No RAN1 impact is foreseen NOTE 2: RAN2 work, if necessary, will be triggered by RAN4 LS NOTE 3: This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers. - Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In RAN4#112bis, the methods for reducing the number of UE Rx chains for fragmented carriers have been discussed and some open issues are captured in the approved WF [2]. In the WF, the open issues for signaling related and semi-statically switch hardware resources have not been well discussed. In this paper, we would like to share our view in the following.

# Open issues (for information) Open issues are to be discussed in the meetings later on. Other issues are note precluded to be further disucssed #### Issue 2-3: Signaling related #### Issue 2-4: Discussion on semi-statically switch hardware resources #### Issue 3-3: Observations of impacts on DL Rx requirements RAN4 discuss if following general aspects can be agreeable: - The largest interferer power in-gap shouldn’t larger than -25dBm for fragmented carriers using a single Rx RF chain - It is proposed to consider only same SCS for both CCs to ease receiver design for the common RF chain - ΔRIBNC for SCC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination. Furthermore, ΔRIBNC needs to apply to PCC too, although the value of ΔRIBNC can be different for PCC and SCC - RAN4 shall not consider requirements for fragmented carriers which will result in performance degradation as compared to legacy NC IB CA or – only in case of a strong in-gap interferer – single CC operation

# Discussion

In the SID [1], the objective is to study whether the separated Rx RF chain for each CC can be replaced with fully shared Rx RF chain for two CCs in a single DL band under the frequency span smaller than 100MHz. The issue is raised by some companies that the meaning of the shared Rx RF chain is not very clear. Probably, it can be referred to partially-shared Rx RF chain, i.e., with the same external-LNA and separated internal-LNA/mixer/LO/analog BB LPF, or fully-shared Rx RF chain, i.e., with the same LNA/mixer/LO/analog-BB LPF. After two meeting’s discussion, the study scope of the shared Rx RF chain is identified as the fully shared Rx RF chain and the terminology of one Rx RF chain is captured in the following agreement.

In our understanding, two CC NCCA is usually implemented by the shared antenna/BPF/external-LNA, different internal-LNA/mixer/LO/analog-BB LPF as the partially shared Rx RF chain. In other word, supporting fully shared Rx RF chain means that the UE can save more mixer/LO/analog-BB LPF to extend the maximum supporting CC number for high order CA. However, depending on the different RFIC implementation, the mixer/LO/analog-BB LPF allocation rule for a configured CA combination would be different. Hence, how many Rx chains can be reduced in a configured CA combination is usually limited by the RFIC capability.

# Ad hoc meeting agreements #### Merge issue 2-0/2-1 discussion: Agreement: The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

Signalling

One objective in the SID is to discuss the method for a UE to inform the network about which CA configuration it can support with the adjusted RF requirements, i.e., the RF requirement for the fully shared RF chain. In our understanding, one bit indication to support fully shared RF chain for fragmented carriers can be per-UE capability signalling, per-band capability signalling, per-BC capability signalling or per-band per-BC capability signalling.

For per-UE or per-BC capability signalling, it would mean that the UE needs to support fully-shared RF chain for all supported RF bands or for a configured band combination. Based on the above discussion, it would be less flexible for UE RFIC implementation. Considering the following example band combination CA_n3(2A)-n7(2A) in TS38.101-1, it is observed that there are two bands with the fragmented carriers in the listed NCCA band combination. Depending on the different optimization for the mixer/LO/analog-BB LPF allocation rule, there may be a possibility that the UE may use the fully shared Rx RF chain for NCCA CA_n3(2A) and non-fully shared Rx RF chain for NCCA CA_n7(2A) in the configured CA band combination. Hence, in our view, it seems that the per-band or per-band per-BC capability signalling is more suitable since it can offer more UE implementation flexibility.

[38.101-1] [V18.7.0]

Table 5.5A.3.1-1c: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3(2A)-n7(2A) | CA_n3A-n7A | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | CA_n7(2A)_BCS0 |  |

Table 5.5A.3.4-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (five bands)

| NR CA configuration | Uplink configuration or single uplink carrier 2 | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3(2A)-n7(2A)-n8A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n8A CA_n1A-n78A CA_n3A-n7A CA_n3A-n8A CA_n3A-n78A CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

In another example, take the high order CA band combination CA_n1A-n3(2A)-n7(2A)-n8A-n78A as an example. It is also observed that NCCA CA_n3(2A) and NCCA CA_n7(2A) are within the CA band combination. For these two examples, if per-band per-BC capability signalling is indicated, the UE may need to report two times for each band combination. Hence, in our view, the per-band capability signalling is more suitable due to the less signalling overhead for the multiple higher order CA combination including the same fragmented carrier scenario. For example, if the per-band capability bit is indicated in NR band n3 and the CA configuration is CA_n3(2A)-n7(2A), it can be interpreted as that the fully shared Rx RF chain is supported for CA_n3(2A) but not for CA_n7(2A). Therefore, if the capability signalling for fragmented carriers is agreed to be introduced in RAN4, it is preferred to use one-bit per-band capability signaling to indicate the supported CA configuration within a CA band combination.

Proposal 1: If the capability signalling for fragmented carriers is agreed to be introduced in RAN4, it is preferred to use one-bit per-band capability signaling to indicate the supported CA configuration within a band combination.

In addition, we think it is not quite clear how the network would use the reported capability information for supported fragmented carrier in the field. However, it may be helpful for the test equipment to differentiate the new introduced RF requirement for fragmented carriers from the existed NCCA RF requirements. Hence, if the capability signalling for fragmented carriers is agreed to be introduced in RAN4, it is preferred to use the capability signaling only for the testing purpose.

Proposal 2: If the capability signalling for fragmented carriers is agreed to be introduced in RAN4, it is preferred to use the capability signalling only for the testing purpose.

Semi-statically switch hardware resources

Another objective in the SID is to discuss the semi-statically switching hardware resources, i.e., the switching from fully-shared Rx RF chain (e.g., FC-mode) to non-fully shared Rx RF chain (e.g., non-FC-mode). The discussion is raised in the previous meetings that it is not very clear for the expected UE behavior in the FC-mode when the higher interference is detected inside the gap between two non-contiguous CCs. In our understanding, how to measure the interference inside the gap in the FC-mode and how to determine the threshold for the interference from FC-mode to non-FC mode needs to be further clarified.

For UE perspective, the one possibility is that the UE in FC-mode can monitor interference measurement inside the gap by itself and determine whether it should transfer to non-FC mode by itself when the hardware resources is sufficient. If the hardware resource is not sufficient, the UE can stay in FC-mode and depend on the network decision by the legacy scheduling mechanism to do any reconfiguration or fallback to single carrier. Hence, it is observed that whether the UE is in FC-mode or non-FC-mode can be transparent to the network.

From network perspective, the one possibility is that the network may configure the reference signal, e.g., NZP-CSI-RS or ZP-CSI-RS, inside the gap to request the UE to report RSRP/RSRQ/SINR/CQI. Based on the reported values, the network can decide whether to do any reconfiguration or fallback to single carrier. In this scenario, the UE is in FC-mode or non-FC-mode can also be transparent to the network.

In our view, it is straightforward that the Rx RF performance in non-FC mode is generally better than in FC-mode. If the hardware resource is sufficient, the UE may default use non-FC-mode for the NCCA configuration. Only in high order CA configuration, the FC-mode may be triggered to support more CCs DL configuration. Hence, the scenario can be applied to the FC-mode may be very limited. In addition, based on the above discussion, it seems that the UE can still indicate high order CA band combinations which only supported in FC-mode in the legacy UE capability signaling, and the network is not necessarily to know the exact information for which band combination supporting fully shared Rx RF chain. Therefore, considering the UE supporting the fully shared Rx RF chain for fragmented carriers can be transparent to the network, it is proposed that semi-statically switch hardware resources for fragmented carriers can be left to the UE implementation.

Proposal 3: Considering the UE supporting the fully shared Rx RF chain for fragmented carriers can be transparent to the network, it is proposed that semi-statically switch hardware resources for fragmented carriers can be left to the UE implementation.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: If the capability signalling for fragmented carriers is agreed to be introduced in RAN4, it is preferred to use one-bit per-band capability signaling to indicate the supported CA configuration within a band combination.

Proposal 2: If the capability signalling for fragmented carriers is agreed to be introduced in RAN4, it is preferred to use the capability signalling only for the testing purpose.

Proposal 3: Considering the UE supporting the fully shared Rx RF chain for fragmented carriers can be transparent to the network, it is proposed that semi-statically switch hardware resources for fragmented carriers can be left to the UE implementation.

# Reference

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek, RAN#104

- R4-2417201, “WF on fragmented DL”, MediaTek, RAN4#112bis