# 3GPP TSG-RAN WG4 Meeting #113	R4-2418850

# Orlando, United States, 18th - 22nd November, 2024

Agenda Item:	7.12.2.2

Source: 		Qualcomm Incorporated

Title: 	Views on FR1 NTN devices OTA

Document for:	Approval

# Introduction

In RAN4#112bis, WF in [1] capturing agreements and open issues for FR1 NTN OTA testing. In this paper, we provide our further views on OTA test method and performance metric for FR1 NR NTN and IoT NTN devices.

# 2. Discussion

For usage scenarios and performance metric for NR-NTN and IoT-NTN, the following agreements were made in [1].

## 2.2 Usage scenarios and UE performance metric

Issue 3-1-1: Usage scenarios for NR-NTN mobile handheld UE Agreements: - NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode. Issue 3-1-2: Check whether the UE orientation is arbitrary or directional during NR-NTN communication Agreements: - For NR-NTN, the target usage of NR-NTN UE is close to normal NR UE. Issue 3-2-1: Performance metric for NTN device Agreements: - For NR-NTN UEs, take a whole sphere measurement with TRP and TRS as the performance metrics for all usage modes as baseline. Other performance metric can be considered if issue is identified. - For IoT-NTN UEs, FFS performance metric.

Specifically, NR-NTN and IoT-NTN handheld UE have two main usage scenarios.

- Talk mode

  - Talk mode with hand+head

  - Talk mode with Hand only (via speaker)

- Browsing mode

  - Browsing mode with hand only

Satellite operators support the usage of NR-NTN UE should be close to normal NR UE. That means both talk mode with hand+head and talk mode with hand-only scenarios should be considered for NTN UE with legacy TRP metric. Hence it was agreed to take a whole sphere measurement with TRP and TRS as the performance metrics for all usage modes as baseline.

Table 1: TRP and TRS limits defined in Rel-18 for band n1 and n28

| Requirements | Talk mode TRP |  | Talk mode TRS |  | Browsing mode TRP |  | Browsing mode TRS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n1 PC3 | n28 PC3 | n1 TRS | n28 TRS | n1 PC3 | n28 PC3 | n1 TRS | n28 TRS |
| Limits in Rel-18 | 10.6 | 7.2 | -87 | -80 | 11.6 | 10 | -88.5 | -83.5 |
| Limits adopted in March 2026 (RAN#111) | N/A | 7.7 | N/A | N/A | N/A | 10.5 | N/A | N/A |

Table 1 shows the TRP and TRS limits defined in Rel-18 for band n1 and n28 (FDD band). If we take n1 (2GHz) TRP as an example, the antenna efficiency is -12.4dB for talk mode and -11.4dB for browsing mode. Note that TRP here are based on a whole sphere measurement.

In Rel-17 NR NTN coexistence study, the UL Geometry SINR were evaluated and the values from 9 companies’ input are shown in Table 2. If we look at the mean SINR @5% (cell edge UE), the mean SINR are -20.76dB for GEO, -10.27dB for LEO1200, and -6.74dB for LEO600. It should be noted that in the simulations, a reference antenna with a gain of 0dBi is assumed for handheld UE. However, the actual antenna gain can’t meet 0dBi in each direction considering the antenna efficiency which is not 100%.

So if we further consider the antenna efficiency derived from TRP limits defined in Rel-18, over -10dBi antenna gain need to be added which will make the system doesn’t work anymore. Even though, 3GPP are working on the UL coverage enhancements for NTN UE including NTN HPUE, PUSCH/PUCCH reception, the gap is still very big if antenna efficiency of over -10dB is taking into account.

Observation 1: Considering a whole sphere measurement for TRP/TRS as the OTA performance metric for NTN UE will lead to a very bad coverage or even outage for NTN.

It is well understated that operator’s motivation of introducing a whole sphere TRP. It could be considered in future release with coverage enhancements solutions.

Proposal 1: RAN4 should consider defining a proper performance metric, e.g., Peak EIRP/EIS and/or partial Spherical coverage CDF in this release for NR-NTN. The TRP/TRS with a whole sphere measurement could be consider in future release.

Proposal 2: FFS on whether to define the different performance metric for talk mode (hand+head and/or hand only) and browsing mode.

Table 2: Calibration summary for NTN UL Rural cases in TR 38863

| Calibration metrics |  | UL Coupling Loss |  |  | UL Geometry SINR |  |  | UL UE Tx Power |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDF percentile |  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| LEO-600 | Samsung | 123.60 | 125.25 | 126.98 | -5.51 | -3.07 | -0.70 | 23.00 | 23.00 | 23.00 |
|  | Qualcomm | 123.64 | 125.37 | 127.14 | -5.43 | -2.92 | -0.42 | 23.00 | 23.00 | 23.00 |
|  | CATT | 123.68 | 125.47 | 127.29 | -7.22 | -1.78 | 0.42 | - | - | - |
|  | THALES | 123.75 | 125.84 | 127.90 | -6.16 | -3.76 | -1.31 | 23.00 | 23.00 | 23.00 |
|  | Huawei | 123.54 | 125.63 | 129.16 | -5.12 | -1.31 | 0.60 | 23.00 | 23.00 | 23.00 |
|  | Xiaomi | 123.66 | 125.22 | 126.92 | -5.14 | -2.60 | 0.18 | 23.00 | 23.00 | 23.00 |
|  | ZTE | 123.83 | 125.65 | 127.46 | -5.45 | -3.04 | -0.55 | 23.00 | 23.00 | 23.00 |
|  | Ericsson | - | - | - | -14.64 | -8.63 | -3.87 | - | - | - |
|  | FhG | 123.85 | 125.27 | 127.12 | -5.98 | -3.47 | -1.38 | 23.00 | 23.00 | 23.00 |
|  | Variance | 0.01 | 0.04 | 0.48 | 8.19 | 3.96 | 1.63 | 0.00 | 0.00 | 0.00 |
|  | Mean | 123.69 | 125.46 | 127.50 | -6.74 | -3.40 | -0.78 | 23.00 | 23.00 | 23.00 |
| LEO-1200 | Samsung | 129.62 | 131.27 | 132.99 | -9.44 | -7.60 | -5.83 | 23.00 | 23.00 | 23.00 |
|  | Qualcomm | 129.56 | 131.18 | 132.89 | -9.40 | -7.61 | -5.78 | 23.00 | 23.00 | 23.00 |
|  | CATT | 129.72 | 131.31 | 132.85 | -8.84 | -6.97 | -5.28 | - | - | - |
|  | THALES | 129.68 | 131.51 | 133.28 | -9.46 | -7.68 | -5.75 | 23.00 | 23.00 | 23.00 |
|  | Huawei | 129.48 | 131.28 | 133.83 | -9.48 | -6.85 | -5.18 | 23.00 | 23.00 | 23.00 |
|  | Xiaomi | 129.61 | 131.23 | 132.93 | -7.97 | -6.09 | -4.20 | 23.00 | 23.00 | 23.00 |
|  | ZTE | 129.60 | 131.38 | 133.25 | -9.27 | -7.32 | -5.63 | 23.00 | 23.00 | 23.00 |
|  | Ericsson | - | - | - | -19.05 | -13.42 | -9.84 | - | - | - |
|  | FhG | 129.68 | 131.28 | 133.06 | -9.51 | -7.69 | -5.98 | 23.00 | 23.00 | 23.00 |
|  | Variance | 0.01 | 0.01 | 0.09 | 9.87 | 4.03 | 2.16 | 0.00 | 0.00 | 0.00 |
|  | Mean | 129.62 | 131.31 | 133.14 | -10.27 | -7.91 | -5.94 | 23.00 | 23.00 | 23.00 |
| GEO | Samsung | 138.15 | 139.85 | 141.59 | -20.31 | -18.54 | -16.85 | 23.00 | 23.00 | 23.00 |
|  | Qualcomm | 138.06 | 139.73 | 141.47 | -20.21 | -18.47 | -16.78 | 23.00 | 23.00 | 23.00 |
|  | CATT | 138.07 | 139.61 | 141.26 | -19.86 | -18.24 | -16.70 | - | - | - |
|  | THALES | 138.23 | 140.07 | 141.90 | -20.37 | -18.55 | -16.68 | 23.00 | 23.00 | 23.00 |
|  | Huawei | 139.45 | 140.61 | 142.00 | -17.89 | -16.73 | -15.61 | 23.00 | 23.00 | 23.00 |
|  | Xiaomi | 138.10 | 139.59 | 141.16 | -18.98 | -17.45 | -15.92 | 23.00 | 23.00 | 23.00 |
|  | ZTE | 137.76 | 140.33 | 142.91 | -18.35 | -15.72 | -13.37 | 23.00 | 23.00 | 23.00 |
|  | Ericsson | - | - | - | -30.59 | -25.37 | -21.63 | - | - | - |
|  | FhG | 138.33 | 139.76 | 141.56 | -20.26 | -18.49 | -17.06 | 23.00 | 23.00 | 23.00 |
|  | Variance | 0.22 | 0.12 | 0.27 | 12.84 | 6.56 | 4.15 | 0.00 | 0.00 | 0.00 |
|  | Mean | 138.27 | 139.94 | 141.73 | -20.76 | -18.62 | -16.73 | 23.00 | 23.00 | 23.00 |

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 NTN devices. The following proposals were made:

Observation 1: Considering a whole sphere measurement for TRP as the OTA performance metric for NTN UE would lead to a very bad coverage or even outage.

Proposal 1: RAN4 should consider defining a proper performance metric, e.g., Peak EIRP/EIS and/or partial Spherical coverage CDF in this release for NR-NTN. The TRP/TRS with a whole sphere measurement could be consider in future release.

Proposal 2: FFS on whether to define the different performance metric for talk mode (hand+head and/or hand only) and browsing mode.

# 4.	References

- R4-2413603, WF for [112][333] TRP_TRS_MIMO_OTA, vivo