# 3GPP TSG-RAN WG4 Meeting #113	R4-2418851

# Orlando, United States, 18th - 22nd November, 2024

Agenda Item:	7.12.2.3

Source: 		Qualcomm Incorporated

Title: 	Discussion on FR1 dynamic MIMO OTA test methodology

Document for:	Approval

# Introduction

In RAN4#112bis, FR1 dynamic MIMO OTA test methods were discussed and the WF was agreed in [1]. In this paper, we provide our considerations on FR1 dynamic MIMO OTA test methodology.

# 2. Discussion

## 2.1 Channel model

CDL channel model was initially introduced in OTA testing started from FR1/FR2 MIMO OTA. In TR 38.827, the NLOS channel coefficient for time instant t, Rx antenna/beam u, Tx beam s, and cluster n is defined by following equation:

Figure 2: CDL channel model in TR 38.827 with simplifications

From the equation above, we can observe that to generate the channel with spatial characteristics, the key parameters including Tx/Rx antenna configurations, Tx/Rx antenna array beamforming, and initial phase and coupling of ray angles for sub-rays, etc.

In [1], the following agreements were approved regarding the dynamic MIMO OTA channel model generation.

Issue 1-1-1: Dynamic MIMO OTA channel model generation Agreements: - Encourage CE vendors to provide more details on how to handle the fixed channel parameters in dynamic MIMO OTA channel model, e.g., the coupling of ray angles, in dynamic MIMO OTA channel model for continuous channel modelling.

It was agreed that CE vendors are encouraged to provide more details on parameters in dynamic MIMO OTA channel model, e.g., the coupling of ray angles. Initial phases are also important but were not added in the WF due to proprietary issue. Besides the coupling of ray angles and initial phases, Tx antenna configurations and beamforming gain are also have impact to the CDL channel model in FR1 dynamic OTA testing.

In [2], no specific beamforming is included in the antenna radiation pattern and 4 antenna ports with dual-polarized 2* 2 antenna are configured.

Figure 1 Base station antenna array configuration in [2].

For 2x2 and 4x4 FDD configurations, it is reasonable to use PMI-feedback based MIMO precoding by the BS emulator. While for 4x4 TDD, RAN4 could discuss whether to consider the Tx beamforming to emulate more realistic environment.

Proposal 1: RAN4 to discuss whether to consider the Tx beamforming to emulate more realistic environment for 4x4 TDD configurations.

## 2.2 Maximum number of HARQ transmission

In RAN4#112, the following agreements were made maximum number of HARQ transmissions [1].

Issue 1-2-1: The maximum number of HARQ transmissions Agreements: - Take the following test parameters as the starting point and further check the feasibility and potential impact on dynamic MIMO OTA testing. - Maximum number of HARQ transmission: [4] - Number of HARQ processes for TDD bands: [8] - Number of HARQ processes for FDD bands: [4] - Measurements can be performed to compare the measurement results with Maximum Number of HARQ Transmission = 1 vs. 4.

In [2], the maximum number of HARQ transmission is set as 1, and the number of HARQ processes for TDD and FDD bands are set as 8 and 4 respectively.

In [4], the maximum number of HARQ transmission is set as 4, and the number of HARQ processes for TDD and FDD bands are set as 8 and 4 respectively.

To verify UE’s performance in the realistic scenario, RAN4 can take the following test parameters as the starting point and further discuss the feasibility and impact on the UE’s performance.

- Maximum number of HARQ transmission: [4]

- Number of HARQ processes for TDD bands: [8]

- Number of HARQ processes for FDD bands: [4]

It is noted that the number of HARQ processes will not have impact to the channel model validation. Therefore, it would not create new testability issues compared to a smaller number of HARQ transmission, e.g., 1.

Proposal 2: RAN4 can take the following test parameters as the starting point and further discuss the feasibility and impact to the UE’s performance based on measurement data.

- Maximum number of HARQ transmission: [4]

- Number of HARQ processes for TDD bands: [8]

- Number of HARQ processes for FDD bands: [4]

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 dynamic MIMO OTA testing. The following proposals were made:

Proposal 1: RAN4 to discuss whether to consider the Tx beamforming to emulate more realistic environment for 4x4 TDD configurations.

Proposal 2: RAN4 can take the following test parameters as the starting point and further discuss the feasibility and impact to the UE’s performance based on measurement data.

- Maximum number of HARQ transmission: [4]

- Number of HARQ processes for TDD bands: [8]

- Number of HARQ processes for FDD bands: [4]

# 4.	References

- R4-2416579, WF for [112bis][324] MIMO_OTA_Ph3, CAICT

- R4-2413536, TP on Dynamic MIMO Aspects, Keysight Technologies, Spirent Communications

- TR 38827

- TS 38101-4