3GPP TSG-RAN WG4 Meeting #113	R4-2418892

# Orlando, USA, 18 Nov. – 22 Nov., 2024

Source: 	vivo

Title: 	On RRM for LP-WUR

Agenda Item:	7.23.4.2

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2] and RAN #105 [3].

The objectives of WI are duplicated as below:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

The initial discussion for this topic was initially discussed at RAN4 110bis meeting. The WF on this WI of RAN4 111 and 112 meeting can be found at [4]. The topic summary of RAN4 112bis can be found at [6].

In this contribution we provide our further consideration on this topic.

Discussion

The following agreements were achieved at previous RAN4 meeting for RRM core part.

Issue 1-1-1-1: For case 3, whether to combine MR and LP-WUR samples when serving cell measurement when results from both MR and LP-WUR are available

Agreement:

From RAN4 requirement perspective, combine measurement results from different receivers shall not be considered

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

Agreement

RAN4 specifies relaxed UE MR measurement requirements for both serving cell and neighbour cell measurements at idle/inactive mode for UE supporting LP-WUR

Issue 1-1-8: LP-WUR operating carrier frequency

Agreement:

RAN4 will further discuss whether any technical issues regarding the LS from RAN1.

From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2.

Issue for further discussion based on contribution driven:

- For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

Issue 1-2-5: Nubmer of Rx for LP-WUR requirements

Agreement

Single Rx is the baseline for LR-WUR RRM measurement.

Issue 1-2-6: LP-SS based RRM requirements periodicity and filtering

Agreement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

At RAN2 #127 meeting, regarding procedure and configuration of LP-WUS and RRM measurement relaxation and offloading, RAN2 had the following related conclusions:

- Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

- RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

At RAN2 #127bis meeting, RAN2 had the following related conclusions:

- Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements.

- RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

- Working assumption

For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

General aspects

Issue 1-1-2: Measurement requirements to be specified for LP-WUR

- Proposals

  - P1: RAN4 suspends study on measurement requirements for OFDM based LP-WUR based on LP-SS signal at idle/inactive state until more RAN1 conclusions; RAN4 shall focus on following cases:  (xiaomi vivo Apple oppo)

    - Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

    - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

  - P2: RAN4 needs more progress from RAN1 on whether to have any work on “OFDM-based LP-WUR serving cell measurement requirements based on LP-SS on overlaid OFDM sequences(s) ” scenario. (vivo Apple)

  - P3: RAN4 to define requirements for OFDM-based LR serving cell measurement based on LP-SS. (Huawei)

  - P4：RAN4 shall define the serving cell measurement requirements based on LP-SS. However, we deem that the time window like SMTC may not be introduced since LP-SS will not perform neighbor cell measurement. (ZTE)

  - P5: RAN4 to discuss LP-WUR measurement requirements starting from SSS based measurement and wait for more progress from RAN1 for LP-SS based measurement. For SSS based LP-WUR measurement, the existing intra-frequency cell reselection requirements can be used as baseline with the side condition to be further discussed. (CATT)

At RAN1 118bis meeting the following agreements are available:

Agreement

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

- Based on RAN1 agreement, overlaid OFDM sequences(s) for LP-SS is supported however there is no intention to introduce any new requirements. Therefore if OFDM based LP-WUR is used to detect LP-SS, its measurement requirements can be the same as that of OOK based LP-WUR based on LP-SS. It is suggested to have the following update.

- At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK-based and OFDM-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state. For OFDM based LP-WUR serving cell measurement based on LP-SS, requirements of OOK based LP-WUR based on LP-SS will be used.

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

Issue 1-1-1: Cases/states to be considered for RRM relaxation

The case to be considered for LP-WUR has been discussed, case 1 and case 3 have already been agreed.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

The remaining listed cases at RAN4 111 meeting are case 2 and case 4 where case 4 is covered by higher priority frequency search.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

Based on figure 1, the key issue to have case 2 or not is whether a new threshold will be defined for entry/exit for case 1 or the legacy threshold to trigger neighbour cell measurement (intra or inter) is reused as the threshold for entry/exit of case 1. Apparently for the latter scenario, case 2 needs not be supported since case 1 and case 3 will transfer to each other directly.

Figure 1 Illustration on fully offloading and relaxed cases of LP-WUR

One concern regarding bypass case 2 is the threshold triggering neighbour cell measurements could be near the edge of the LP-SS coverage, as illustrated in figure 2. Under this scenario, if case 1 is triggered by using the legacy threshold for neighbour cell measurement, the performance of LP-WUR, such as accuracy, maybe not at ideal stage since it is close to LP-SS coverage.

- When the threshold to trigger the neighbour cell measurement is near the edge of LP-SS coverage, the performance of LP-WUR, such as accuracy, maybe not at ideal stage hence transforming directly to case 1 under this scenario where only LP-WUR serving cell measurement is available, should be carefully.

Neighbour cell measurement threshold

LP-SS coverage

S Measure

Figure 2 Illustration on possible issue when bypassing case 2

If a new threshold for case 1 is introduced, as illustrated in figure 1, case 2 simply is a natural extension of case 3 when neighbour cell measurement is not needed. Compared with case 1, case 2 create a different trade-off point between power consumption and mobility performance, since the measurement of MR on the serving cell may also help mobility performance.

On the other hand, if case 2 is not introduced in the end, whether using same or different legacy threshold to trigger neighbour cell measurement for non LP-WUR capable UE and LP-WUR capable UE, needs be investigated.

Hence, for the RRM relaxation for MR and serving cell RRM measurement offloaded to LP-WUR, besides case 1 and case 3, it is preferred to consider case 2.

- For the “Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements” in the WID and cases to be considered for RRM relaxation, besides agreed case 1 and case 3, it is preferred to consider case 2.

Issue 1-1-4: Threshold for switch between case 1 and case 3

- Proposals

  - P1: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1). LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). (Apple)

  - P2: (Huawei)

  - For switch from Case #1 to Case #3 based on LR measurement.

    - Option 1: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ

    - Option 2: Use existing thresholds for SS-RSRP and SS-RSRQ, and define offsets for LP-RSRP and LP-RSRQ

  - For switch from Case #3 to Case #1 based on LR or both LR and MR measurements.

    - Option 1: LR measurement only

    - Option 2: both LR and MR measurements, and UE stops neighbor cell measurement when both measurements are below their corresponding thresholds

  - P3: RAN4 to discuss whether and how to differentiate the entry/exit conditions for Case#1 and Case#3. The following options can be considered (MTK)

    - Option 1: Multiple thresholds: separate entry/exit thresholds are defined for different cases, Case#1 and Case#3.

    - Option 2: Single threshold: single entry/exit threshold is defined for both cases, Case#1 and Case#3, in which UE enters Case#3 (RRM relaxation) first, then enters Case#1 (RRM offloading) after some confidence period T.

For issue 1-1-4, to transfer from case 1 to case 3 or case 2 (to be determined), a LR-based threshold should to be defined since MR measurement is not available anyway. The alternative method proposed for switching from case 1 to other cases by using existing thresholds for SS-RSRP and SS-RSRQ plus offset(s) is not preferred since the offset should also be configured by NW and the total efforts is not reduced compared with introducing a new LP-based threshold. Introducing new thresholds based on LP-RSRP and LP-RSRQ is also consistent with the following RAN2 #127 meeting agreement:

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

- Introducing new LP-based threshold(s) based on LP-RSRP and LP-RSRQ used for switch/exit from case 1 to other cases is necessary. Other proposals at issue 1-1-4 are up to whether case 2 will be introduced or not.

Issue 1-1-7: On jointly consideration on issue 1-1-5 and 1-1-6

- Proposals

  - P1-1: Define same entry conditions for LP-WUS monitoring and RRM measurement relaxed cases (case 3). (vivo Huawei).

  - P1-2: Define same entry/exit condition for LP-WUS monitoring and LP-WUR serving cell measurements (xiaomi Huawei MTK)

  - P1-3：the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring.(Apple)

  - P2: The LR-based LP-WUR measurement threshold can be less than or equal to the LR-based LP-WUS monitoring threshold. (LG)

  - P3: The threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbour cell measurement triggering. (Apple):

  - P5: RAN4 shall be involved on criterion and scenario (entry/exit condition) design for LP-WUR measurement and MR RRM measurement relaxation together with  RAN1 and RAN2. (Samsung)

    - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package

  - P6: (Ericsson)

  - When UE checks the LR entry threshold

    - UE is required to evaluate both MR and LR RRM quality if both configured from NW side;

    - UE is optionally to only evaluate MR quality if only MR evaluation criteria is configured from NW side.

  - P7: RAN4 to work on the following issues once RAN2 has made progress (Nokia)

  - Criteria (entry/exit conditions) for LP-WUR serving cell measurement

  - Criteria (entry/exit conditions) for MR RRM measurement relaxation

  - Criteria (entry/exit conditions) for LP-WUS monitoring

Recommendations:

Focus on:

The entry point between the start of LP-WUS monitoring and the start of case 3 [case 1]

The exit point between the exit of LP-WUS monitoring and the exit of case 3 [case 1]

- For this issue, based on RAN2 127 and 127bis agreement, for the LP-WUR monitoring, the entry conditions and the exit conditions are the following:

- Entry conditions: based on MR-based thresholds and optional LP-based thresholds (RAN2 127)

- Exit conditions: LP-based threshold (RAN2 127bis “The LPWUS monitoring exit condition does not include MR measurements”)

- The entry/exit conditions for serving cell offloading (case 1) has been determined at RAN2 127 and RRM relaxation (case 3) has not yet been fully determined by RAN2. To our understanding, although the detail of criteria will be designed by RAN2, RAN4 still could discuss the constraints or the application conditions for these criteria from the top level on how they work together. For the scenario when the LP-WUR is triggered for LP-WUS monitoring whereas the MR still performs serving cell and neighbour cell RRM measurement without any RRM relaxation, to our understanding, this scenario does not bring any benefit to both NW and UE side and should be avoided since the paging occasions is monitored through LP-WUR however there is no power saving gain. Hence, he entry point for LP-WUS monitoring, should be not early than or the same as that of RRM relaxation, or not early than or the same as that of the fully offloading whereas the paging indicator monitoring through LP-WUR is justified by power consumption saving either through RRM relaxation or fully offloading. Actually it is more preferred to have entry/exit conditions of the LP-WUR monitoring to be the same as that of RRM relaxation otherwise the benefit of MR serving cell RRM relaxation is counteracted by paging monitoring of MR.

- Same entry/exit conditions can be used for LP-WUS monitoring and RRM measurement relaxed cases, i.e., case 3.

Issue 1-1-8: LP-WUR operating carrier frequency

Agreement:

- RAN4 will further discuss whether any technical issues regarding the LS from RAN1.

- From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2.

Issue for further discussion based on contribution driven:

- For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

In addition, a LS related to this issue [8] was also sent to RAN4 by RAN1 and the content was copied below for reference.

For the issue when LR and MR work at different bands. To our understanding from UE perspective, it is a typical case where LR is not implemented as many bands as that of the MR. In addition from NW perspective, for a particular band, it also may not support LP-WUR operation even both LP and MR are implemented for that band.  To extend the usability of this feature to ensure more UE could obtain power saving gain and better justify the implementation cost of this feature, we think reasonable scenario when LR and MR work at different bands should be considered. For example, when a LP-WUR is implemented at a low-frequency band, in the idle/inactive mode, the MR will or is preferred to camp on higher frequency band through higher priority frequency layer search. Hence the LP-WUR feature may not be used in the cell center of the low-frequency cell due to higher frequency layer search if inter-band operation is not supported, i.e., the potential of the LP-WUR feature still may not be fully exploited.

- The LP-WUR will have limited user scenarios and the potential of the LP-WUR feature may not be fully exploited if only LP and MR working at the same band is supported.

Regarding reasonable scenario when LR and MR work at different bands to be considered, firstly co-located scenario for different bands is the most straightforward scenario. This scenario could be further identified to:

1. Co-located case, when cells co-located have similar coverage

2. Co-located case, when cells co-located have notable different coverage

For the first case, the sub-features of LP-WUR WI from RAN4 perspective, such as LP-WUR monitoring, serving cell offloading and MR RRM relaxation, the design for LR and MR work at the same band can be reused straightforward. For the second case, in principle the design for LR and MR work at the same band can still be used however should be limited to the cell with the smallest coverage.

- For the LR and MR work at different band, co-located scenario could be considered. Regarding the time line on when it should be considered, further discussion is needed. When cells co-located have similar coverage, the design for LR and MR work at the same band can be reused. When cells co-located have notable different coverage, the design for LR and MR work at the same band can still be used however will be limited to the cell with the smallest coverage.

From RAN4 RRM perspective, the LP-WUR measurement requirements are defined based on the LP-WUR measurement/evaluation performance on corresponding reference signal for LP-WUR, either LP-SS or SSS. Hence the LP-WUR measurement requirements are not related to MR working band, i.e., LP-WUR measurement requirements is not depend on whether LP-WUR and MR work at the same band or not. Based on this point, there is no extra specification work for RAN4 LP-WUR measurement requirements if LP-WUR and MR work at the different band.

- However for the requirements for offloading and RRM relaxation, currently the final mechanism when MR and LR work at different band, if to be defined, is not clear. There will be impact on the corresponding requirements for offloading and RRM relaxation, since RAN4 at least needs re-evaluate corresponding requirements if there is any new solutions defined for these two items. For example, RAN4 currently discusses the interruption period which could be defined at the duration from LP-WUS received the paging indication to MR ready for paging monitoring. If this interruption is to be defined by RAN4, there will be impact on the requirements when MR and LR work at different band.

- LP-WUR requirements depends on LP-WUR band, but is not dependent on whether MR work at the same band or different band from LP-WUR. And there is no extra specification work on RAN4 LP-WUR measurement requirements if LP-WUR and MR work at the different band.

- There will be impact on requirements on offloading and RRM relaxation, depending on solutions for these two items when MR and LR work at different band. For example, the interruption period, e.g., the duration from LP-WUS received the paging indication to MR ready for paging monitoring, if to be defined by RAN4, will be impacted if LR and MR work at different bands.

Issue 1-1-9: LP-WUR status at legacy state

- Proposals

  - P1: LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring (Apple)

    - Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement

    - Alt 2 (preferred): MR is ON with RRM measurement on serving cell and neighbour cell (if any). But whether or not LP-WUR is ON for RRM measurement is up to UE implementation.

  - P2: RAN4 needs consider latest RAN2 agreement when determines LP-WUR status before entering/after exiting LP-WUS monitoring. (xiaomi vivo)

  - P3: Postpone the discussion on LR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring. RAN4 to informs RAN2 to revisit the working assumption on entry condition of LP-WUS monitoring: LR based thresholds are not considered for LP-WUS monitoring entry condition. (Huawei)

- For the status of LP-WUR after exiting LP-WUR monitoring, RAN2 has the following working assumption at RAN2 #127 and #127bis meeting:

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements

- Since the entry conditions may include LP-based threshold, the LP-WUR should be ON to evaluate the LP-based threshold. In practice if the UE enters MR RRM relaxation or fully offloading cases early than enter into LP-WUS monitoring, there is no issue since the LP-WUR is already ON.

- However, one issue is when before starting LP-WUS monitoring, the UE is in the legacy scenario performing legacy serving cell or serving and neighbour cell measurement (either legacy Rel-15 scenario or legacy Rel-16 UE power saving scenario), then the LP-WUR still need “ON”, or at least to be “ON” some time before the entry conditions, following RAN2’s agreement. It is a little bit strange since LR should be “ON” firstly to check whether it needs be “ON” or “OFF”. It also means that the understanding where the LP-WUR can be “OFF” all the time when the UE is in the legacy state (neither at LP-WUR monitoring state nor in case 1, 2 and 3) is not true any more. However this is endurable since the LP-WUR only needs “ON” some time to perform measurement used to compare with LP-based threshold. Since how long the LP-WUR is “ON” is up to UE implementation under this scenario, it is suggested no requirement is defined for this scenario.

- At legacy state, whether or not LP-WUR is ON for RRM measurement is up to UE implementation. When LP-WUR measurement is needed to compare with a LP-based threshold, LP-WUR needs be “ON” at least some duration early before the comparison, no requirement will be defined for this duration.

Issue 1-1-10: Considerations on higher priority frequency layer

- Proposals

  - P1: MR perform relaxed higher priority frequency layer search in RRM relaxation case#1. Open to further discuss which higher priority layer measurement requirement should be applied for case#3 (CMCC)

  - P2: MR perform relaxed higher priority frequency layer search in RRM relaxation case#1 and case#3 (xiaomi vivo CT)

  - P3: Higher priority layer neighbor cell measurement shall also be OFF for case #1 (Apple vivo)

  - P4: UE should perform measurement for higher priority frequency layers, if configured (Huawei)

    - If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements

    - If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed.

  - P5: High priority frequency layer measurement requirements shall use the corresponding requirements of the legacy Rel-16 RRM relaxation WI as the base (ZTE)

    - P5-1: RAN4 shall study the relaxed scaling factor or additional scenarios (ZTE)

  - P6: No need to define new requirements for higher priority frequency layer (Nokia, Ericsson)

For higher priority frequency layer search, it could be stopped in case 1 since the MR is off in case 1. However since the frequency of higher priority frequency layer search is not high, especially considering the further relaxation defined in Rel-16, it is fine that the higher priority frequency layer search is performed in relaxed mode similar to what has been defined in Rel-16 UE power saving WI. For relaxed cases 3, higher priority frequency layer search could be performed in relaxed mode when relaxed conditions are satisfied.

In Rel-16, when the serving cell quality is higher than the threshold for inter-frequency neighbour cell measurement, relaxation on higher priority frequency layer search is performed as below and similar approach can be reused.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60.

- For case 1, higher priority frequency layer search could be OFF. Alternatively, for case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: Accuracy requirements

- Proposals

  - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (xiaomi LG Apple CT Huawei Ericsson MTK)

  - P2: Define LP-WUR measurement accuracy in dedicated section. Whether/how to reflect it in the core requirements can be further discussed after the simulation by taking the results into account (CATT)

Regarding the accuracy requirements, no dedicated accuracy requirement could be fine since it follows traditional RAN4’s methodology. However currently in RAN4 specs these margins are only defined in the sections for intra/inter/inter-RAT measurement requirements and there is no margin defined for the serving cell measurement. For LP-WUR, it is possible to introduce a margin to reflect accuracy performance when LP-WUR serving cell measurement results is used to compare with a threshold since it is a new introduced receiver.

However for MR, margin is not defined for serving cell when serving cell measurement is used to compared with a threshold. Hence there are a few options:

When P1, “introduce a margin to reflect accuracy performance of LP-WUR when LP-WUR serving cell measurement result is used to compare with a threshold and no dedicated accuracy requirement is defined” is used for LP-WUR, for MR, the following options could be considered:

Option 1: For MR, margin is also introduced when MR serving cell measurement result comparing with a threshold, this margin will be based on legacy MR accuracy requirements.

Option 2: For MR, margin is not also introduced when MR serving cell measurement result comparing with a threshold as the legacy specs.

Alternatively P2 is used for issue 1-2-1.

- It is fine to use P1, “introduce a margin to reflect accuracy performance of LP-WUR in core specs”, when LP-WUR serving cell measurement results is used to compare with a threshold. If P1 is used, for MR, the following options could be considered:

Option 1: For MR, margin is also introduced when MR serving cell measurement result comparing with a threshold, this margin will be based on legacy MR accuracy requirements.

Option 2: For MR, margin is not also introduced when MR serving cell measurement result comparing with a threshold, as that of the legacy specs.

If no consensus on how to handle MR when P1 is used, P2 should be considered.

Issue 1-2-2: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

- Proposals

  - P1: Define evaluation period for evaluating entry/exit threshold for LP-WUS monitoring; Define evaluation period for evaluating entry/exit threshold for fully offloading (case 1) or MR RRM relaxation (case 3). (vivo CATT LG MTK)

  - P1-1: The evaluation period is the same as that of the corresponding measurement period. (vivo)

  - P1-2: The evaluation period is based on LP-WUR measurement period, and further discuss whether to define measurement filtering (e.g., number of samples). (CATT)

  - P1-3: RAN4 to discuss the delay requirements to evaluate the entry criteria by using MR measurement results only and to evaluate exit criteria by using LR measurement results only. (MTK)

  - P2: RAN4 specify following requirements for MR RRM relaxation and offloading and existing requirements for S criteria or relaxed measurement criterion can be considered as starting point (Samsung)

    - LR measurement period and accuracy

    - MR measurement relaxation and the evaluation period for entry/exit criteria threshold evaluation under different states (case 1 fully offloading, and case 3 MR relaxation)

  - P3-1: Do not need a dedicated evaluation delay requirement (e.g., in how long time the MR/LR measurement results can meet the condition). Use the MR measurement period and/or LR measurement period to check the criteria for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, i.e., MR measurement delay or LR measurement delay are sufficient for criteria checking; (Apple)

  - P3-2: RAN4 to define requirements for serving cell evaluation against the entry/exit condition for LP-WUS monitoring, including Measurement delay (number of samples); Measurement accuracy (margin for evaluation); Measurement filtering (number of samples and separation between samples) (Huawei)

  - P4: RAN4 specifies the following for entry/exit criteria evaluation for WUS paging monitoring: (QC)

    - The number of measurement samples needed for filtering.

    - The number of consecutive times the UE needs to meet the entry/exit criteria

Recommendations:

- For both MR and LP-WUR, for checking whether the thresholds for entry/exit WUS paging monitoring/LP-WUR measurement/MR RRM relaxation are fulfilled or not,

- Option 1: Use the measurement period of LR or MR (Note: the measurement period of LR has already been agreed to be specified as issue 1-1-2)

- Option 2: Define another evaluation delay period. Evaluation is longer or the same as that of measurement period.

For issue 1-2-2, we discuss some general considerations on the requirements for evaluating entry/exit criteria for LP-WUR monitoring or MR RRM relaxation firstly.

4.2.2.2	 Measurement and evaluation of serving cell

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second,

-	otherwise M1=1.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least eDRX_IDLE cycle/2, if the UE is configured with eDRX cycle ≤ 10.24s; otherwise DRX cycle/2.

According to the [5], the rate of measurement SS-RSRP/RSRQ and evaluation the cell selection criterion S (a particular threshold) is the same, i.e., M1*N1 DRX. For FR1 and when M1=1, the SS-RSRP/RSRQ measurement and evaluation whether a threshold (here it is the cell selection criterion S) is satisfied or not is executed per DRX cycle. It is further defined the samples set used for measurement/evaluation (filtering requirement), i.e., within the set of measurements used for the filtering at least two measurements shall be spaced by, at least eDRX_IDLE cycle/2.

The specification further specifies requirements on when cell selection criterion S is satisfied, how and when the corresponding actions can be executed, i.e., an action is taken after Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S.

If the UE is not configured with eDRX_IDLE cycle and the UE has evaluated according to Table 4.2.2.2-1 or Table 4.2.2.2-4 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

For intra/inter-frequency measurement, requirements for measurement, evaluation and detection are defined. Detection related requirements are not suitable for LP-WUR case 1 or case 3. The requirements for

4.2.2.3	Measurements of intra-frequency NR cells

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5) for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

Basically the methodology of intra/inter-frequency measurement is similar to that of serving cell measurement, i.e., a measurement period with corresponding filtering requirements are defined. And another requirement, evaluation period, usually with a long time, are defined to specify the end point where the corresponding action shall happen.

However the UE shall perform intra-frequency measurement when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ, the duration to evaluate whether Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ is satisfied or not should be the serving cell measurement period, although it is not explicitly specified.

At Rel-16 UE power saving WI, the fulfilment of the not at cell edge criteria is defined in [7],

The evaluation duration is not explicitly specified however it was agreed in [10] that:

- The evaluation rate for measurement relaxation mode triggering shall be the same as current serving cell evaluation rate defined in 38.133.

Hence the evaluation rate is also the serving cell measurement rate.

- In the current specs, the evaluation duration to check whether the serving cell measurement fulfills a particular threshold or not, is either

- The same as that of the serving cell measurement duration, for thresholds such as SSearchThresholdP/SSearchThresholdQ or SIntraSearchP/SIntraSearchQ

- Longer than that of the serving cell measurement duration, such as Nserv for cell selection criterion S evaluation.

This observation is further illustrated in the following figure:

Figure 3 Illustration on methods for evaluating whether a threshold is satisfied or not

Considering thresholds used for LP-WUS monitoring or for MR serving (neighbor cell) measurement relaxation is more like the existing thresholds such as SSearchThresholdP/SSearchThresholdQ or SIntraSearchP/SIntraSearchQ.  Hence follow the legacy methodology, we propose:

- For issue 1-2-2, support P3-1, i.e., use the MR and/or LP-WUR measurement period to check the threshold for either LP-WUR monitoring or MR RRM relaxation is satisfied or not; P1 is also ok providing P1-1 is used.

Issue 1-2-2-1: Detail on LP-WUR/MR evaluation/measurement requirement

- Proposals

  - P1: (vivo LG)

  - For the evaluation period for evaluating entry threshold for LP-WUS monitoring, defined separately as:

    - the MR serving cell measurement period

    - the LP-SS based LP-WUR measurement period or SSB based LP-WUR measurement period if LR-based threshold is configured. (Nokia)

  - For the evaluation period for evaluating exit threshold for LP-WUS monitoring, defined as either the LP-SS based LP-WUR measurement period or the SSB based LP-WUR measurement period. (Nokia)

  - For the evaluation period for evaluating entry/exit threshold for fully offloading or MR RRM relaxation, defined separately as:

    - the MR serving cell measurement period

  - LP-WUR measurement period if needed, including LP-SS based LP-WUR measurement period or SSB base LP-WUR measurement period respectively. (Nokia)

  - P2: Depending on different cases, UE may use MR measurement, or LR measurement, or MR+LR measurement to check the criteria; and followings are considered (Apple):

    - If only MR is ON for measurement, the delay requirements for entry/exit criteria evaluation shall be same as legacy MR serving cell measurement delay requirement.

    - If only LR is ON for measurement, RAN4 to discuss the delay requirements for entry/exit criteria evaluation.

    - If both LR and MR is ON for measurement (MR is in relaxation mode), RAN4 to discuss the MR/LR serving cell measurement delay requirements for entry/exit criteria evaluation

  - P3: RAN4 to discuss the delay requirements to evaluate the entry criteria by using MR measurement results only and to evaluate exit criteria by using LR measurement results only. (MTK)

Recommendations:

- For this issue, based on latest RAN2’s agreements, sometimes both MR + LR measurement results will be used for evaluating whether a threshold is satisfied or not, sometimes measurement results from only one receiver, either MR or LR, are used to evaluating corresponding thresholds. For the scenario when both MR and LR measurement results are used for evaluation whether a threshold is satisfied, defined MR and LR requirements for entry/exit evaluation separately as:

    - the MR serving cell measurement period, either legacy or relaxed measurement period

    - the LP-SS based or SSB based LP-WUR measurement period

- For issue 1-2-2-1, For the scenario when both MR and LR measurement results are used for evaluation whether an entry/exit condition is satisfied or not, either for LP-WUR monitoring or MR RRM relaxation, defined MR and LR requirements for entry/exit evaluation separately as:

- the MR serving cell measurement period, either legacy or relaxed measurement period

- the LP-SS based or SSB based LP-WUR measurement period

Issue 1-2-3: On interruption related requirements for LP-WUR WI

- Proposals

  - P1: Interruption requirements could be defined for the duration from LP-WUS received paging indication to MR ready for paging monitoring (CMCC vivo Samsung Docomo CT)

  - P2: Interruption requirements may need to be specified for the following:(CMCC)

    - Between LP-WUR measurement result fulfills exit criteria and MR to start measurement

    - Between MR measurement result fulfills entry criteria and LP-WUR to start measurement

  - P3: RAN4 only needs to define the wake-up delay requirement, if necessary, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring (Apple)

  - P4: Wait for more RAN1 progress on PO monitoring before discussing the possible interruption requirements related to LR (Huawei)

  - P5: RRM requirements for interruption should be based on the RAN1 agreements for wake-up delay (NTT Docomo)

  - P6: RAN4 shall clarify the timeline from decoding LP-WUS to MR wake-up in order to monitor legacy PO. (ZTE)

Recommendations:

Suggest company provide which components should be considered/included in the interruption period

- Regarding the “interruption” to be defined, firstly the intention of this requirement is to have some limitation on time from the MR is triggered till MR is ready for particular actions. The term “interruption” maybe not fully appropriate since there is no process A is interrupted by process B however is actually one action is triggered by another action. Hence it is suggested to use the term “MR activation” time as the title of the requirements. For the interruption duration from LP-WUR exits fully offloading case (case 1) till MR is ready for measurement, the concern is whether a separately “interruption” requirement is needed for this case since this case could be covered by the paging interruption case.

- Suggest to use the term “MR activation” time to replace “interruption” on paging. The interruption duration from LP-WUR exits fully offloading case (case 1) till MR is ready for measurement, could be covered by the paging interruption case and may need not a separate requirement.

Regarding the components to be considered in this “activation” requirements, one component should be definitely included is ramping time which has been discussed in RAN1. Based on the RAN1 118bis meeting agreements as below, multiple values for ultra-deep sleep based on RAN1 agreements should be considered in RAN4 requirements. To our understanding, this ramping time has already included any uncertainty from LP start to trigger till MR get this message.

Agreement

Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state.

Another component to be considered is the time for a MR to get synchronous after triggered from ultra-deep mode, this component will consist of at least the time for AGC, raw timing sync and other related procedure if necessary. Typically 2 SSBs (2 samples) are needed for AGC and raw timing sync respectively. For the related procedure to be considered, the worst case is that the MR may immediately need a cell reselection procedure after being waking up. Under this scenario, two components should be considered. The first one is the component for cell detection requirements, i.e., how many samples a UE needs to detect a newly cell. The number of samples corresponding to DRX 1.28, i.e., 25 samples and the typical 20ms SSB periodicity can be used as a base. The other component is the maximum paging interruption defined currently in the current spec, the TSI-NR + 2*Ttarget_cell_SMTC_period can be reused.

For normal case where a cell reselection procedure is not needed after waking up, the “MR activation” duration could be:

“MR activation” duration = Ramping duration + (2 + 2) samples * 20ms

For a case where a cell reselection procedure is needed, although these cases may be rare, it is still through these cases should be covered. The components to be considered for this case has been provided above. The components are considered to be sequential implemented for this case and the whether the “MR activation” duration is:

“MR activation” duration =  Ramping duration + (2 + 2 + 23) samples * 20ms +  TSI-NR + 2*Ttarget_cell_SMTC_period

In RAN1’s discussion, the ramping time is either 800ms or 400ms.

- Defining interruption requirement for the normal case and worst case where a cell reselection is included in the worst case, detailed conditions are FFS. The “MR activation” time for the normal and worst case are:

- “MR activation” duration = Ramping duration + (2 + 2) samples * 20ms

- “MR activation” duration = Ramping duration + (2 + 2 + 23) samples * 20ms + TSI-NR + 2*Ttarget_cell_SMTC_period

Note: Ramping duration refers to RAN1 conclusion

Issue 1-2-4: On LP-WUR cell selection evaluation requirements for case 1

- Proposals

  - P1: RAN4 to discuss whether to define evaluation requirements for cell selection criterion based on LR measurement for fully offloading case (CATT)

  - P2: No need to define the LR based cell selection criterion evaluation in case#1. (Apple vivo Huawei)

  - P3: Define LP-WUR based serving cell selection criterion S evaluation requirement (Ericsson MTK)

    - P3-1: If cell selection criterion S is not fulfilled by LR within X measurement cycles, the UE (MR) shall initiate the measurements of all neighbour cells indicated by the serving cell by triggering the MR to wake up (i.e., exit from Case#1 and activate MR). (MTK)

For the cell selection evaluation requirement for case 1, the cell selection for LP-WUR is used to trigger MR to perform neighbor cell measurement, as indicated in P3-1, however the exit conditions of case 1 serve the same purpose and may even with a higher exit threshold hence the necessity to have a cell selection evaluation requirements for LP-WUR is low.

- For issue 1-2-4, support P2, i.e., no need to define the LR based cell selection criterion evaluation in case#1.

Periodicity for SSB based LP-WUR measurement delay requirements

Option 1: Based on DRX configured for MR (vivo)

Option 2: follow the same periodicity of LP-SS if LP-SS periodicity is configured (CMCC, Xiaomi, vivo, HW, Apple, QC, MTK)

Option 3: A fix value (like 320ms), FFS on the value (Apple - if LP-SS periodicity is not configured, HW- if LP-SS periodicity is not configured, LGE - if LP-SS periodicity is not configured, )

Option 4: max of option 1 and 2 (QC)

Option 5: max of option 2 and 3 (OPPO)

Other options are not precluded.

For LP-SS based LP-WUR, the following agreement is achieved at RAN4 112bis for the periodicity for measurement requirement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

For SSB based LP-WUR measurement requirements, the issue is the SSB periodicity could be quite small (like 20ms) and cannot be used directly. In legacy the periodicity for serving cell measurement based on SSB is based on DRX, although DRX will not be configured for LP-WUR, the DRX periodicity can still be used as the periodicity for SSB-based LP-WUR measurement period, however this method may be complex. Option 2 definitely works and when LP-SS is not configured, option 3 (defining a fixed value) can be used as a complement for option 3 and the fixed value could be FFS.

- For periodicity for SSB based LP-WUR measurement delay requirements, use option 2 + option 3, i.e., follow the same periodicity of LP-SS if LP-SS periodicity is configured, use a pre-defined fix value (like 320ms) if LP-SS periodicity is not configured, detail value is FFS.

MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

  - P1: Use relaxation/scaling factor for MR serving/neighbour cell relaxation(xiaomi vivo  CMCC vivo Huawei Samsung)

    - P1-1: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor (Samsung CT ZTE MTK)

    - P1-2: Relaxation factor =8 as the starting point (CMCC)

    - P1-3: Relaxation factor >=16 (Apple vivo Huawei MTK)

    - P1-4: At least the scaling factor should be larger than that of legacy Rel-16 UE power saving WI. (vivo)

    - P1-5: The scaling factor can be further discussed after RAN4 have concrete agreements on the MR RRM relaxation cases. (xiaomi)

  - P2: For MR neighbor cell measurement relaxation, the existing relaxed requirements in 4.2.2 can be used as baseline. For MR serving cell measurement relaxation, a scaling factor can be introduced. Further discuss whether to use same scaling factor as neighbor cell measurement (CATT, Ericsson)

  - P3: RAN4 to further discuss scaling factor for MR RRM relaxation for serving and/or neigbhor cell after scenario and detailed criterion for measurement relaxation have been clarified (LG)

  - P4: The existing relaxation for scaling factor can be the starting point for MR RRM relaxation (NTT Docomo)

  - P6: The legacy intra-/inter-frequency and inter-RAT neighbour cell measurement requirements can be the baseline and RAN4 can study the relaxed scaling factor (ZTE)

  - P7: Multiple values can be considered depending on conditions. (Samsung MTK)

Issue 1-3-2: On Neighbour cell and serving cell relaxation factor

MR RRM relaxation had been discussed at RAN4 112bis meeting and at the online session, the following aspects were discussed.

Aspect 1: Agree on relaxation/scaling factor(s) for MR serving/neighbour cell relaxation as the starting point

- Support: vivo, ZTE, MTK, Samsung, Apple, HW, China Telecom, QC

- Discuss the number later: Nokia, E///, OPPO, Xiaomi, LGE, CATT

Aspect 2: The same or different relaxation/scaling factors for MR serving and neighbour cell relaxation

- Same: MTK, vivo, QC, China Telecom, HW, OPPO, Xiaomi, CMCC, ZTE, Samsung, Apple

- Different: CATT, Nokia

  - FFS Smaller factor for serving cell or neighboring cell

Exact relaxation factor as the starting point

- Option 1: Only consider the scaling factor(s) larger than that of legacy Rel-16 UE power saving WI (i.e., 3) (QC, CMCC, LGE, ZTE, HW, China Telecom, Apple, vivo, MTK, Xiaomi, Samsung)

- Option 2: 3 is included as one of the factors (E///, Nokia)

For the scaling factor(s) larger than 3, which number can be used as starting point.

- 8: CMCC, China Telecom, vivo, Samsung, Ericsson, MTK, QC, Nokia

- >=16: vivo, QC, HW, Apple, Samsung, MTK

However there is no consensus on any topic at RAN4 112bis meeting. Since it is clearly indicated in the WID that “Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements” and case 3 had been agreed at previous RAN4 meeting, more progress on this topic is needed.

As we analysed before, investigation on the power saving gain when MR enters RRM relaxation mode compared with I-DRX with or without PEI has been carried out at SI phase for LP-WUS duty-cycled monitoring and continuously monitoring, the results are captured in the section 8.1.1.5 and 8.1.1.6 in [9] and summarized in the following table:

Table 1 Power saving gain against MR relaxation times

| MR RRM relaxation times | mean power saving gain average (LP-WUR duty-cycled mode) | mean power saving gain average (LP-WUR continuously mode) |
| --- | --- | --- |
| MR relaxed < 8 times | average: 4%, | average: 22% |
| 8 times<= MR relaxed <=16 times | average: 40%, | average: 32% |
| MR relaxed > 16 times | average: 60%, | average: 57% |
| MR offload RRM to LR | average: 83%, | average: 74% |

From the above outcome apparently the fully offloading mode where MR does not perform any RRM measurement achieves the maximum power saving gain when the serving cell RRM measurement is offloaded to LP-WUR. It is also observed a significant power saving gain increase happens when the relaxation times of MR changes from less than 8 to between 8 and 16 for the LP-WUR duty-cycled mode. In general, 8 relaxation times is needed to achieve a reasonable 40% power saving gain base on the table 1.

The relaxation factor for various RRM relaxation schemes at idle/inactive state introduced since Rel-16 are also summarized in the following table.

Table 2 Exiting RRM measurement criteria and corresponding relaxation

|  | Rel-16 low mobility criteria | Rel-16 Not-at-cell-edge criteria | Rel-16 Both low mobility and Not-at-cell-edge criteria | Rel-17 stationary criteria (Redcap) | Rel-17 stationary and not-at-cell edge criteria (Redcap) |
| --- | --- | --- | --- | --- | --- |
| Measurement relaxation factor for neighbour cell measurement (intra/inter frequency measurement and inter-RAT measurement when applicable) | 3 times | 3 times | 1 hour | 6 times | 4 hours |

For neighbour cell measurement relaxation for case 3, regarding criteria, RAN2 has the following agreements.

- Working assumption

For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Based on RAN2’s agreement, RAN4 could at least consider further RRM relaxation based on “not at cell edge” criteria. To our understanding at least for not at the cell edge criteria, a new threshold, which will be tighten compared with that of Rel-16, may be defined for case 3 as illustrated in figure 1, which enable a larger relaxation factor, such as 8 or 16, can be used for neighbour cell measurement relaxation. More important, besides the serving cell measurement performed by the MR, the LP-WUR also performs serving cell measurement which enable a UE has a quick fallback to normal measurement status when the serving cell quality is not good enough, this extra benefit introduced by LP-WUR also justifies a larger scaling factor can be used for neighbour cell measurement. It also should be noted that 3 times relaxion in Rel-16 is already a compromise.

For the serving cell measurement relaxation, considering the fully offloading mode where no MR serving cell measurement is acceptable, in case 3 both LP-WUR and MR perform serving cell measurement, hence a relaxed factor from 8 to 16 is reasonable in order to achieve the power saving gain indicated in table 1.

- A larger scaling factor such as 16 can be considered for serving cell and neighbor cell measurement since either a new threshold, which is tighten compared with that of Rel-16, may be introduced for case 3 or the RRM relaxation mode can be left more promptly compared with previous UE power saving scheme thanks to the fact that besides MR, one extra receiver (LP-WUR) is used to check whether the relaxation mode should be left or not.

In addition, different consideration on the scaling factor for serving and neighbor cell are proposed, we prefer to have the same scaling factor for both the serving cell and neighbor cell since the UE power saving is determined by the minimum scaling factor. For example when the serving or neighbour cell measurement relaxation is 8, it does not make a lot sense to have neighbour or serving cell relaxation as 16 since it will not have a lot power saving gain.

- The amount of power saving gain is mainly determined by the minimum scaling factor among serving cell or neighbour cell measurement relaxation.

- Consider relaxation factors such as 16 for the relaxation factor for the MR RRM relaxation for both serving and neighbour cell measurements.

- Consider only one relaxation factors for the MR RRM relaxation for both serving and neighbour cell measurements.

Regarding proposal to consider multiple value for scaling factor, it needs to clarify whether the intention is to have scaling factors with different value for serving and neighbour cell measurement or have scaling factors with different value for other purpose whereas the serving cell and neighbour cell always have the scaling factor with the same value although that value can be multiple. In addition how to use multiple value also needs be clarified. It should be noted that having multiple value on scaling factors has already been discussed in Rel-16 UE power saving WI and was not used in the end. To our understanding, multiple value on scaling factor should be avoided unless there is a clear purpose and implementation method.

- Regarding multiple value for scaling factors, the serving cell and neighbour cell will still have the scaling factor with the same value although that value can be multiple. Multiple scaling factor was discussed in Rel-16 UE power saving WI and was not used in the end.

- Multiple value on scaling factor should be avoided unless there is a clear purpose and a clear way on how to implement it.

Issue 1-3-3: Accuracy for relaxed MR measurement

- Proposals

  - P1: The legacy accuracy for relaxed MR measurement should be reused if Case#2 and/or Case#3 introduced. (CMCC vivo Huawei)

For the accuracy requirements for relaxed MR measurement, the legacy requirements are used during Rel-16 UE power saving WI. It is straightforward to the use same principle, i.e., use the legacy accuracy requirements for relaxed MR measurement.

- For relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

LP-WUR connected mode

The part corresponding to CONNECTED mode from the WID are copied below:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

The RAN4’s RRM task for RRC_Connected mode was discussed at RAN4 111 meeting under the following issue:

Issue 1-4-1: LP-WUR at CONNECTED mode

- Proposals

  - P1: No RRM objectives is expected for connected mode in this WI. (xiaomi vivo CATT LG Apple ZTE)

  - P2: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress. Paging reception requirement impact can be discussed after further input from RAN2 and RAN1 (Samsung)

  - P3: Postponed until more progress in other groups (Ericsson, Huawei)

  - P4: RAN4 to discuss interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. (Qualcomm)

Recommendations:

The majority view is there is no RAN4 actions on CONNECTED state. However it is also possible for issues being identified depending on RAN1/2 progress. Hence the discussion can be postponed until more progress.

Regarding P4, the assumption is that MR and LR works at different time, when the WUS monitoring occasion collides with a MG, one possible solution is the MR uses the MG to perform measurement and LR monitors LP-WUR after MG. However the detailed design could be up to RAN1 design and since there is no RAN4 scope on CONNECTED mode, it is suggested to postpone the discussion on this issue.

- For issue 1-4-1, postpone the discussion.

Other issues

Issue 1-5-2: LR based RRM with EMR in IDLE/inactive mode

- Proposals

  - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple)

Recommendations:

Check the principle of P1 is agreeable or not

Regarding EMR related issue, to our understanding the principle P1 in issue 1-5-2 is reasonable. The issue itself is not crucial for Rel-19 requirements point of view and can be discussed later.

- For issue 1-5-2, the principle of P1 is reasonable. The issue can be discussed later.

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

- When the threshold to trigger the neighbour cell measurement is near the edge of LP-SS coverage, the performance of LP-WUR, such as accuracy, maybe not at ideal stage hence transforming directly to case 1 under this scenario where only LP-WUR serving cell measurement is available, should be carefully.

- In the current specs, the evaluation duration to check whether the serving cell measurement fulfills a particular threshold or not, is either

- The same as that of the serving cell measurement duration, for thresholds such as SSearchThresholdP/SSearchThresholdQ or SIntraSearchP/SIntraSearchQ

- Longer than that of the serving cell measurement duration, such as Nserv for cell selection criterion S evaluation.

- A larger scaling factor such as 16 can be considered for serving cell and neighbor cell measurement since either a new threshold, which is tighten compared with that of Rel-16, may be introduced for case 3 or the RRM relaxation mode can be left more promptly compared with previous UE power saving scheme thanks to the fact that besides MR, one extra receiver (LP-WUR) is used to check whether the relaxation mode should be left or not.

- The amount of power saving gain is mainly determined by the minimum scaling factor among serving cell or neighbour cell measurement relaxation.

- Regarding multiple value for scaling factors, the serving cell and neighbour cell will still have the scaling factor with the same value although that value can be multiple. Multiple scaling factor was discussed in Rel-16 UE power saving WI and was not used in the end.

- At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK-based and OFDM-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state. For OFDM based LP-WUR serving cell measurement based on LP-SS, requirements of OOK based LP-WUR based on LP-SS will be used.

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- For the “Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements” in the WID and cases to be considered for RRM relaxation, besides agreed case 1 and case 3, it is preferred to consider case 2.

- Introducing new LP-based threshold(s) based on LP-RSRP and LP-RSRQ used for switch/exit from case 1 to other cases is necessary. Other proposals at issue 1-1-4 are up to whether case 2 will be introduced or not.

- Same entry/exit conditions can be used for LP-WUS monitoring and RRM measurement relaxed cases, i.e., case 3.

- The LP-WUR will have limited user scenarios and the potential of the LP-WUR feature may not be fully exploited if only LP and MR working at the same band is supported.

- For the LR and MR work at different band, co-located scenario could be considered. Regarding the time line on when it should be considered, further discussion is needed. When cells co-located have similar coverage, the design for LR and MR work at the same band can be reused. When cells co-located have notable different coverage, the design for LR and MR work at the same band can still be used however will be limited to the cell with the smallest coverage.

- LP-WUR requirements depends on LP-WUR band, but is not dependent on whether MR work at the same band or different band from LP-WUR. And there is no extra specification work on RAN4 LP-WUR measurement requirements if LP-WUR and MR work at the different band.

- There will be impact on requirements on offloading and RRM relaxation, depending on solutions for these two items when MR and LR work at different band. For example, the interruption period, e.g., the duration from LP-WUS received the paging indication to MR ready for paging monitoring, if to be defined by RAN4, will be impacted if LR and MR work at different bands.

- At legacy state, whether or not LP-WUR is ON for RRM measurement is up to UE implementation. When LP-WUR measurement is needed to compare with a LP-based threshold, LP-WUR needs be “ON” at least some duration early before the comparison, no requirement will be defined for this duration.

- For case 1, higher priority frequency layer search could be OFF. Alternatively, for case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search.

- It is fine to use P1, “introduce a margin to reflect accuracy performance of LP-WUR in core specs”, when LP-WUR serving cell measurement results is used to compare with a threshold. If P1 is used, for MR, the following options could be considered:

Option 1: For MR, margin is also introduced when MR serving cell measurement result comparing with a threshold, this margin will be based on legacy MR accuracy requirements.

Option 2: For MR, margin is not also introduced when MR serving cell measurement result comparing with a threshold, as that of the legacy specs.

- For issue 1-2-2, support P3-1, i.e., use the MR and/or LP-WUR measurement period to check the threshold for either LP-WUR monitoring or MR RRM relaxation is satisfied or not; P1 is also ok providing P1-1 is used.

- For issue 1-2-2-1, For the scenario when both MR and LR measurement results are used for evaluation whether an entry/exit condition is satisfied or not, either for LP-WUR monitoring or MR RRM relaxation, defined MR and LR requirements for entry/exit evaluation separately as:

- the MR serving cell measurement period, either legacy or relaxed measurement period

- the LP-SS based or SSB based LP-WUR measurement period

- Suggest to use the term “MR activation” time to replace “interruption” on paging. The interruption duration from LP-WUR exits fully offloading case (case 1) till MR is ready for measurement, could be covered by the paging interruption case and may need not a separate requirement.

- Defining interruption requirement for the normal case and worst case where a cell reselection is included in the worst case, detailed conditions are FFS. The “MR activation” time for the normal and worst case are:

- “MR activation” duration = Ramping duration + (2 + 2) samples * 20ms

- “MR activation” duration = Ramping duration + (2 + 2 + 23) samples * 20ms + TSI-NR + 2*Ttarget_cell_SMTC_period

Note: Ramping duration refers to RAN1 conclusion

- For issue 1-2-4, support P2, i.e., no need to define the LR based cell selection criterion evaluation in case#1.

- For periodicity for SSB based LP-WUR measurement delay requirements, use option 2 + option 3, i.e., follow the same periodicity of LP-SS if LP-SS periodicity is configured, use a pre-defined fix value (like 320ms) if LP-SS periodicity is not configured, detail value is FFS.

- Consider relaxation factors such as 16 for the relaxation factor for the MR RRM relaxation for both serving and neighbour cell measurements.

- Consider only one relaxation factors for the MR RRM relaxation for both serving and neighbour cell measurements.

- Multiple value on scaling factor should be avoided unless there is a clear purpose and a clear way on how to implement it.

- For relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

- For issue 1-4-1, postpone the discussion.

- For issue 1-5-2, the principle of P1 is reasonable. The issue can be discussed later.

- References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

[2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

[3] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 105, vivo, NTT DOCOMO

[4] R4-2416861,	WF on RRM requirements for NR_LPWUS, RAN4 112bis, vivo

[5] TS38.133, V18.4.0

[6] R4-2415664	Topic summary for [112bis][217] NR_LPWUS, RAN4 112bis,vivo

[7] TS38.304, V18.0.0

[8] R1-2407559, LS on LP-WUS operation in IDLE/INACTIVE mode, RAN1, RAN1 118

[9] TR 38.869 V18.0.0

[10] R4-2009265, WF on RRM measurement relaxation for Power Saving, RAN4 96e, CATT