3GPP TSG-RAN WG4 Meeting #113	R4-2418895

# Orlando, USA, 18 Nov. – 22 Nov., 2024

Source: 	vivo

Title: 	Consideration on RRM impacts for SBFD operation

Agenda Item:			7.20.3.2

Document for:	   Discussion

- Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The updated objective in [1] which are related to RRM are copied below for reference:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles)

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

    - At least aperiodic reporting

    - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - CLI measurement accuracy requirement

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we provide our considerations on RRM requirements for SBFD.

- Discussion

This WI was initially discussed at RAN4 112 meeting. At previous meeting, based on the WF [2], the following agreements are available:

Issue 1-2-1: Requirements for SSB based measurement

- Agreements (from online session)

  - Down-select one of the two options:

    - Option 1:

      - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

      - This can be revisited if it proves to be conflicting with further RAN1 agreements.

    - Option 2:

      - For inter-frequency measurement without gap, UL transmission is prioritized.

      - Follow option 1 in other cases.

Issue 1-2-2: Requirements for CSI-RS based measurement

- Agreements (from online session)

  - RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations.

    - CSI-RS colliding with dynamically scheduled UL transmission

    - CSI-RS puncturing in frequency domain in SBFD symbols

    - CSI-RS distributed over SBFD and non-SBFD symbols

  - Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI.

  - RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

Issue 1-2-9: Limits on the maximum number of DL/UL switching

- Agreements (from online session)

  - RAN4 not to discuss limits on the maximum number of DL/UL switching unless requested by other WGs.

The following topics are open for discussion.

#### Issue 1-2-1: Requirements for SSB based measurement

- Agreements (from online session)

  - Down-select one of the two options:

    - Option 1:

      - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

      - This can be revisited if it proves to be conflicting with further RAN1 agreements.

    - Option 2:

      - For inter-frequency measurement without gap, UL transmission is prioritized.

      - Follow option 1 in other cases.

For SSB based management, regarding option 2, for the inter-frequency measurement without gap, it will happen when the measurement bandwidth is within the current active BWP or the UE has a spare RF chain. However this does not mean the UE has the capability to have the UL transmission while the downlink SSB based measurement. Option 1 is preferred for issue 1-2-1.

Proposal 1: For issue 1-2-1, support option 1.

Issue 1-2-2: Requirements for CSI-RS based measurement

- Proposals related to CSI-RS colliding with dynamically scheduled UL transmission

  - Proposal 1 (vivo, Nokia, ZTE):

    - CSI-RS measurement period should be extended when dynamic UL transmission is scheduled to be colliding with the CSI-RS resource.

      - Option 1: counting the lost measurement occasions as in Rel-16 NR-U,

      - Option 2: using a scaling factor as in Rel-17 con-MG.

  - Proposal 2 (CATT):

    - For CSI-RS-based serving cell measurements,

      - UL transmission is cancelled if cancellation timeline is met when semi-statically configured UL transmission collides with dynamically scheduled DL reception.

      - UE does not receive DL channel/signal when dynamically scheduled UL transmissions collide with semi-statically configured DL reception.

  - Proposal 3 (HW):

    - RAN4 to discuss the impact of CSI-RS measurement colliding with dynamically scheduled UL on SBFD symbols

For the CSI-RS based measurement, when dynamic UL transmission is scheduled to be colliding with the CSI-RS resource, the measurement occasion will be dropped and the measurement period should be extended accordingly. The solution for this case could be the measurement requirements are extended. Currently there are two methods to extend the measurement period, one is from Rel-16 NR-U where the measurement period is extended by counting the number of lost measurement occasions. The other way is to use a scaling factor to extend the measurement period like what has been done in Rel-17 concurrent gap WI. To our understanding either way works and RAN4 can select one from the two solutions to determine how to extend the CSI-RS measurement requirements.

Proposal 2: CSI-RS based measurement period should be extended under SBFD scenario when it is impacted. On how to extend the CSI-RS measurement period, the method for extending the measurement period, either by counting the lost measurement occasions introduced in Rel-16, or by using a scaling factor introduced in Rel-17 can be considered.

Issue 1-2-4: Requirements for RACH requirements

- Proposals

  - Proposal 1 (vivo, CATT, Samsung, ZTE):

    - RAN4 to confirm there is no impact on RACH requirements due to SBFD operation.

  - Proposal 1a (Samsung):

    - Further discuss whether new test cases with updated PRACH configuration during RRM performance phase which also pending on detailed RAN1/RAN2 specification update.

  - Proposal 2 (E///):

    - Wait for the further RAN1 conclusion and decide whether there is RRM impacts

- Recommended WF

  - As baseline, there is no impact on RACH requirements due to SBFD operation.

  - This can be revisited if impacts are identified based on further RAN1 agreements

For the requirements for RACH requirements, we support the recommended WF at as baseline, there is no impact on RACH requirements due to SBFD operation.

Proposal 3: For RACH requirements, as baseline, there is no impact on RACH requirements due to SBFD operation.

Issue 1-2-6: Requirements for MG and BWP switch

- Proposals

  - Proposal 1 (E///):

    - For UL slot definition and wording update within the TS 38.133, the issue can be addressed during the CR phase.

  - Proposal 2 (HW, QC):

    - Include transmission in SBFD slots in addition to the UL slots, wherever applicable in the specification, e.g. in MG and BWP switch requirements.

- Recommended WF

  - Update the wording in the spec to include transmission in SBFD slots in addition to the UL slots, wherever applicable.

  - Try to identify the list of impacted requirements, and detailed wording update can be left to CR phase.

Proposal 4: Support the recommended WF.

Issue 1-2-7: Requirements for UL resource muting

- Proposals

  - Proposal 1 (vivo, LGE, Samsung, HW):

    - RAN4 to confirm there is no RRM impact due to UL resource muting.

  - Proposal 2 (E///):

    - Wait for the further RAN1 conclusion and decide whether there is RRM impacts

- Recommended WF

  - As baseline, there is no RRM impact due to UL resource muting.

  - This can be revisited if impacts are identified based on further RAN1 agreements.

We do not see any impact on RACH requirements and any impact due to UL resource muting.

Proposal 5: There is no impact on RACH requirements and no impact on RRM due to UL resource muting.

- Conclusions

In this contribution we provided our initial considerations on RRM requirements for SBFD and have the following proposals:

Proposal 1: For issue 1-2-1, support option 1.

Proposal 2: CSI-RS based measurement period should be extended under SBFD scenario when it is impacted. On how to extend the CSI-RS measurement period, the method for extending the measurement period, either by counting the lost measurement occasions introduced in Rel-16, or by using a scaling factor introduced in Rel-17 can be considered.

Proposal 3: For RACH requirements, as baseline, there is no impact on RACH requirements due to SBFD operation.

Proposal 4: Support the recommended WF.

Proposal 5: There is no impact on RACH requirements and no impact on RRM due to UL resource muting.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2416858, WF on RRM requirements for NR_duplex_evo, RAN4 112bis, Huawei