3GPP TSG-RAN WG4 Meeting #113	R4-2418898

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	7.17.2.1

Source:	Intel Corporation

Title:	Views on AI/ML testability and interoperability for CSI compression use case

Document for:	Discussion

# Introduction

In Rel-18 RAN WG4 conducted detailed studies on the testability and interoperability aspects of AI/ML enabled techniques for NR air interface as a part of FS_NR_AIML_Air SI [1] and the conclusions are summarized in TR 38.843 [2]. A new WI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN #102 meeting with the latest version available in [3] with the following RAN4 objectives (the study stage objectives related to  RAN4 are marked in yellow). Furthermore, in accordance with the agreements in RAN #105 [4] the RAN4 work on the testability and interoperability was extended till the end of Rel-19.

### Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: … - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: … - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: … - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. … Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - … - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. ### Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

In this contribution we provide views and proposals on the AI/ML testability and interoperability aspects for two-sided AI/ML models for CSI compression and CSI compression use case.

# Discussion

## Background

In the SI stage RAN4 discussed feasibility of testing 2-sided AI/ML models for CSI compression use case and several candidate options on how to test encoder/decoder implementation at the test equipment side were identified [2].

Following the above principles, the considered options of test decoder are listed below - Option 1: DUT provides the decoder - Option 2: Infra vendor provides the decoder - Option 3: Full decoder specification in standard - Option 4: TE vendor provides the decoder

In the RAN4 #110 (Feb 2024) meeting, a decision was made to focus on establishing a standardized reference decoder for two-sided AI/ML models, effectively narrowing the scope to options 3 and 4 [5].

Issue 4-2: Testing options for 2-sided model - RAN4 to further discuss only options 3 and 4

In RAN4#110bis (Apr 2024) a discussion on the potential next steps took place and a high-level procedure was presented in [6] although not much technical progress was made.

In RAN4 #111 (May 2024) meeting and a subsequent email discussion the following progress was reached regarding the analysis of feasible testing options for two-sided AI-ML models [7]:

- It was agreed that companies should present results for a pair of encoder/decoders under Option 3. From these, one decoder will be selected for further evaluation to determine if UE-vendor-specific encoders can be effectively trained to achieve comparable performance and complexity (or other evaluation criteria).

- The simulation parameters for assessing the feasibility of defining harmonized models across companies were established based on the assumptions made by RAN1 during the study item phase.

- For the initial feasibility analysis, RAN4 agreed to utilize simple AI/ML models (CNNs ResNet). However, for assessing performance requirement stage, the development of a new Test Decoder model is anticipated.

- The specific parameters for the simplified AI/ML model to be used for feasibility analysis were determined through an email discussion in June 2024.

In RAN4 #112 (Aug 2024) meeting the following progress was reached regarding the analysis of feasible testing options for two-sided AI-ML models [8-9]:

- The next steps to conclude on Option 3 analysis were identified

- Simulation results were collected and agreed to perform another round of alignment until RAN4 #112bis with refined simulation parameters agreed in [9]

- Further agreements and sub-options for Option 4 methodology were identified

In RAN4 #112bis (Oct 2024) meeting the following progress was reached regarding the analysis of feasible testing options for two-sided AI-ML models [10]:

- Option 3 analysis

  - Simulation results were collected with a reasonably good alignment achieved

  - Interested companies were encouraged to share decoder and encoder ML models and dataset in next meeting (RAN4#113)

  - Additional data to be submitted was identified:

    - CDF of SGCS per subband

    - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

- Option 4 analysis

  - Interested companies were invited to bring further proposals for the next steps for the feasibility study of Option 4 (for any of the sub-options of Option 4) including analysis of commonality with option 3 (whether any steps or data generated for Option 3 can be reused)

## Inter-vendor training collaboration framework

The concurrent discussion with respect to inter-vendor training collaboration of AI/ML-based CSI compression is taking place in RAN1. In RAN1 #116 (Feb 2024) the following agreements were reached:

Agreement To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options: - Option 1: Fully standardized reference model (structure + parameters) - Option 2: Standardized dataset - Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Option 5: Standardized model format + Reference model exchange between NW-side and UE-side Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc. Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

In the following meetings additional agreements on details of respective options were made, but no final conclusions were reached and almost all options are under consideration. In accordance with the RAN1 #118 agreements the following general directions are considered:

- Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) … - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b) … - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1) …

As acknowledged in RAN1 discussion the RAN1 Option 1 (Direction C) closely resembles RAN4 Options 3/4, with the primary distinction being the potential inclusion of a reference encoder in RAN1 approach. If RAN1 Option 1 / Direction C is adopted, then the respective models can be potentially reused for RAN4 testing. Meanwhile, other RAN1 directions are reflecting a broader range of considerations and if any of the options are adopted, then additional RAN4 discussions may be required. RAN1 discussions on reference models may have significant implications for RAN4 work on defining reference decoder options and decisions made by RAN1 will influence the direction of RAN4 standardization efforts. Given the interdependency of RAN1/4 discussions, it is recommended for RAN4 to continue analyzing both Options 3 and 4 concurrently and wait for RAN1 conclusions on how to facilitate inter-vendor collaboration before deriving final conclusions. The respective proposal is in-line with the common understanding in RAN4 #110bis (“RAN4 continues to discuss Option 3 and 4 and can review/discuss RAN1 agreements on interoperability if there will be any”).

Proposal #1:	Further align the AI/ML test methodology with RAN1's conclusions on inter-vendor training collaboration for AI/ML-based CSI compression.

## Option 3: Fully standardized reference/test decoder

### AI-ML model and training assumptions

The simplified model agreed during the June RAN4 email discussion is clear enough. In RAN4 #112 assumptions on the model and training were agreed in [9]. Below, we provide more details on used dataset, model and training parameters used in simulations:

Table 1. AI-ML model and training assumptions

| Parameter/Configurations | Options/Values | Comments |
| --- | --- | --- |
| Dataset related |  |  |
| Sub-band channel averaging | Average of covariance channel matrices per PRBs in the sub-band 1 sample per PRB is selected | Same as [9] |
| Any other pre-processing | Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector L2 norm of the eigenvector is equal to 1. | Same as [9] |
| Data collection assumptions | Multiple channels realizations are collected from multiple UEs and multiple drops of same UE. 10 realizations from one UE with 1000 ms time separation |  |
| Training dataset size | 600K for training 120K for validation (20%) 60K for inference/testing (10%) |  |
| Dataset quantization | Float32 | Same as [9] |
| Model related |  |  |
| Quantization/Dequantization | Results provided for 1) No quantization 2) Scalar 2-bit quantization with fixed grid with cluster centres of [0.125, 0.375, 0.625, 0.875] Quantization-aware training |  |
| Normalization | For the results with quantization use a sigmoid function in the encoder for normalization and inverse sigmoid after de-quantization |  |
| Training related |  |  |
| Use and configuration of drop-out | No | Same as [9] |
| Use of regularization | No L1/L2 regularization |  |
| Training loss function | (1-SGCS) | Same as [9] |
| Optimizer | Adam | Same as [9] |
| Learning rate | 0.0001 | Same as [9] |
| Batch size | 256 | Same as [9] |
| (Max) Number of epochs | 150 | Same as [9] |
| Stopping criteria | No improvement over 25 epochs over validation dataset | Same as [9] |
| Weight initialization | Default | Same as [9] |
| Kernel Parameters | Default | Same as [9] |
| Other | Layer-specific training is performed for each MIMO layer (i.e., training is performed for layer 1 dataset only) |  |

### Simulation results

The modelling results for SGCS for the agreed model for the test dataset are provided below. The results include:

- AI CSI feedback with RAN4 CNN model w/ 64 bits per layer

  - No quantization

  - Fixed grid quantization

- eTypeII CSI feedback with 62 bits for 1 layer case and 113 bits for 2 layers

Note: results for SGCS are same as in RAN4 #112bis

The table below provided the results for the SGCS across all datasets and all subbands within the dataset (no sub-band averaging applied for CDF) and figures below provide a more details CDF statistics of the SGCS. The results show that SGCS for AI CSI feedback with RAN4 models is slightly better than eType II CSI feedback for layer 1 and layer 2. Given that average SGCS metric may not provide full insights on the SGCS statistics, it is recommended to additionally use CDF statistics (e.g., 5/50/95% points) for the model alignment and criteria for the feasibility analysis.

Table 2. Simulation results summary

| 2GHz / 15kHz SCS / 10MHz BW / layer 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | SGCS |  |  |  |
|  | Avg | 5% CDF | 50% CDF | 95% CDF |
| eTypeII CSI feedback | 0.690 | 0.329 | 0.716 | 0.965 |
| AI CSI feedback (no quant) | 0.740 | 0.273 | 0.797 | 0.990 |
| AI CSI feedback (fixed quant) | 0.691 | 0.222 | 0.742 | 0.969 |
| 2GHz / 15kHz SCS / 10MHz BW / layer 2 |  |  |  |  |
|  | SGCS |  |  |  |
|  | Avg | 5% CDF | 50% CDF | 95% CDF |
| eTypeII CSI feedback | 0.553 | 0.170 | 0.551 | 0.958 |
| AI CSI feedback (no quant) | 0.614 | 0.142 | 0.640 | 0.984 |
| AI CSI feedback (fixed quant) | 0.560 | 0.108 | 0.575 | 0.958 |

| Figure 1. Simulation results – SGCS CDF statistics |  |
| --- | --- |

To facilitate better alignment, we also include additional alignment data in excel format:

- CDF of SGCS values per subband

- CDF of the eigen values of the channel matrix of training data for each subband (layer 1 – the 1st strongest eigenvalue; layer 2 – the 2nd strongest eigenvalue):

Note: The data is presented in dB scale and corresponds to the channel matrix for 1 SC per each subband. No RSRP normalization is applied and values include both large-scale and small-scale propagation effects.

### Decoder and dataset selection criteria

In RAN4 #112 meeting the following options on the Decoder selection criteria were proposed:

Issue 4-3: Decoder selection criteria - Proposals - Option 1: Best performance (SGCS) and/or mean performance of corresponding eType II CSI feedback - Option 2: select multiple and work in parallel - few decoders can be selected and a subset of companies check feasibility on one decoder - Option 3: random selection, as long as performance is similar it does not matter which decoder is selected - Option 4: decoder trained with the highest number of sample - Option 5: To use an aggregated dataset of channel realizations/eigenvectors {V} for comparison of the performance of encoders/decoders provided by the companies. - Option 6: other criteria

From our point of view the selection criteria for the feasibility analysis is not a critical aspect and a random selection can be considered. Meantime, it may be beneficial to select more than one decoder to check the feasibility. For instance, 2-3 candidate decoders can be considered to ensure that the feasibility can be confirmed for different companies’ implementations. In accordance with the feasibility analysis procedure companies may need to train the encoders based on own and external datasets. The external datasets for feasibility studies shall be selected corresponding to the selected decoders.

Proposal #2: 	Randomly select 2-3 candidate decoder implementations for further feasibility analysis.

## Option 4: Partially standardized reference/test decoder

The option 4 is defined as follows in [2]:

For option 4, the following aspects should be considered: - TE vendor should be able to develop the decoder based on the specifications - Test repeatability should be ensured (variation among TE vendor implementations should be bound) - Other vendors should also be able to develop such a decoder and which can deliver similar performance - Interoperability should be ensured based on the parameters that need to be specified - Parameters that need to be specified are FFS - Candidate parameters/conditions that may be considered for defining test decoder include - Training data set for TE decoder training - Model structure (Activation function is included in the model structure) - Performance parameters for the TE decoder (e.g. cosine similarity, loss function, etc) - Maximum FLOPs allowed for the test decoder - Maximum number/size of model parameters - Compression ratio of decoder (output size/input size) - Quantization level - Other parameters are not precluded and to be further discussed. - Note: Feasibility of definition of parameters needs further investigated.

Additional agreements were made in RAN4 #112 [9]:

Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

The eventual goal of AI/ML test feasibility analysis is to make ensure test methodology provides a framework to provide repeatable results at the TE side. From this point of view, there may be no fundamental difference between Option 3 and Option 4 approaches in terms of feasibility if sufficient details of Option 4 model are provided such that TE vendors can come up with models providing aligned results.

In our interpretation the intention of Option 4 is to focus on specifying datasets and principles, which can be used to ensure that TE implementation is sufficiently characterized. In our view, the main potential advantage of Option 4 is that it may not limit the implementation of encoder/decoder models to certain assumptions. Therefore, Option 4a is preferred without specifying a reference encoder. We assume that to enable Option 4a RAN4 needs to specify datasets at the input/output of test decoder and the following information required:

- Channel dataset (encoder input / decoder output)

  - Same dataset as for Option 3

  - Used for latent space dataset generation

  - Used for TE decoder training

- Latent space dataset (encoder output / decoder input)

  - Generated via simulations from companies for the given reference encoder/decoder models. The intermediate results from Option 3 feasibility analysis can be reused.

  - Used for TE decoder training

- Reference encoder/decoder

  - High-level description is sufficient and there is no need to capture the full model in specifications

  - Reference encoder/decoder are used to generate the latent space dataset

In addition, a specific procedure to ensure that TE decoder is good enough needs to be specified. For instance, SGCS metric requirement can be provided to ensure that target SGCS is within certain limits of the target value.

Proposal #3: 	Further study Option 4a under assumption that the following information is specified

- Channel dataset (encoder input / decoder output)

- Latent space dataset (encoder output / decoder input)

- Reference encoder/decoder

Deprioritize Option 4b.

The following steps for Option 4a feasibility analysis procedure are proposed (steps with reuse of Option 3 assumptions/data are marked in green):

- Agree on simulation assumptions for Channel dataset generation (reuse Option 3 assumptions)

- Agree on reference AI-ML models (encoder/decoder) for Latent dataset generation (reuse Option 3 assumptions)

- Jointly train encoder/decoder models and generate Latent space dataset

  - The dataset can be generated as a byproduct of joint training of encoder/decoder used for Option 3 analysis

- Share companies’ datasets

  - Channel dataset (reuse Option 3 dataset)

  - Latent dataset

- Select one or more datasets for further analysis

- Each company brings results for training of own decoder / encoder

  - The decoder is trained based on the selected dataset. The decoder models based on reference and own implementation assumptions shall be analysed.

  - The encoder is trained based on the selected dataset and own dataset. The encoder models based on reference and own implementation assumptions shall be analysed.

  - The results can be compared to the case of joint training of encoder/decoder

  - The results can be compared to the case of using of own dataset

- Conclude on overall feasibility of Option 4

  - FFS on what the feasibility criteria

Proposal #4: 	Consider the following steps for Option 4a feasibility analysis:

- Agree on simulation assumptions for Channel dataset generation

- Agree on reference AI-ML models (encoder/decoder) for Latent dataset generation

- Jointly train encoder/decoder models and generate Latent space dataset

- Share companies’ datasets (channel and latent)

- Select one or more datasets (channel and latent) for further analysis

- Each company brings results for training of own decoder / encoder

- Conclude on overall feasibility of Option 4

## Two-sided models complexity

Due to a diverse nature of AI/ML models implementations, there may be a significant variation in their complexity and performance, which is illustrated by Rel-18/19 RAN1 studies. The summary AI/ML models used for RAN1 evaluations for 2-sided AI/ML models for CSI compressions is provided in TR 38.843 [2] with the complexity in the range from 10M to 1100M FLOPs and with 1M to 17M parameters for the decoder (CSI reconstruction) part:

The complexity metric in terms of FLOPs and number of parameters of AI/ML models adopted in the evaluations of CSI compression are summarized in Figure 6.2.2.1-1, where the complexity for the CSI generation part and the complexity for the CSI reconstruction part are illustrated separately. - A majority of 25 sources adopt the CSI generation model subject to the FLOPs from 10M to 800M, and 26 sources adopt the CSI reconstruction model subject to the FLOPs from 10M to 1100M. - A majority of 21 sources adopt the CSI generation model subject to the number of parameters from 1M to 13M, and 22 sources adopt the CSI reconstruction model subject to the number of parameters from 1M to 17M. - Results refer to Table 1 of clause 7.3, R1-2310450. Figure 6.2.2.1-1: Complexity of AI/ML models from evaluation results in terms of FLOPs and number of parameters for CSI compression

In the figure below we illustrate the dependency of SGCS performance depending on the AI/ML model complexity based on results in [11]. It can be observed that AI/ML model complexity has a direct impact on the overall performance and that the complexity of the methods may substantially exceed the complexity of legacy PMI-based CSI feedback schemes.

Figure: SGCS and complexity for eType II PMI codebook and different transformer-based AI/ML models.

The RAN4 reference models for testing and performance requirements definition are expected to set the basis for both UE and NW implementations and serve as reference decoders for performance requirements definition. Therefore, RAN4 needs to further discuss on associated complexity for Options 3/4 considering RAN1 analysis on the performance as well as practical considerations on the algorithm implementation complexity including computational complexity and memory consumption. Additional RAN4 analysis on complexity should be performed to ensure that reference models represent practical implementations in terms of complexity and power consumption. The respective bounds on the maximum complexity shall be considered for both Option 3 and Option 4 methods.

Proposal #5:	Further discuss and define the upper bound complexity including the number of computations and number of parameters for test/reference encoders/decoders for the purpose of performance requirements definition.

# Conclusion

In this paper we provide our views on general aspects of AI/ML testability and interoperability aspects, and, in summary, make the following proposals:

Proposal #1:	Further align the AI/ML test methodology with RAN1's conclusions on inter-vendor training collaboration for AI/ML-based CSI compression.

Proposal #2: 	Randomly select 2-3 candidate decoder implementations for further feasibility analysis.

Proposal #3: 	Further study Option 4a under assumption that the following information is specified

- Channel dataset (encoder input / decoder output)

- Latent space dataset (encoder output / decoder input)

- Reference encoder/decoder

Deprioritize Option 4b.

Proposal #4: 	Consider the following steps for Option 4a feasibility analysis:

- Agree on simulation assumptions for Channel dataset generation

- Agree on reference AI-ML models (encoder/decoder) for Latent dataset generation

- Jointly train encoder/decoder models and generate Latent space dataset

- Share companies’ datasets (channel and latent)

- Select one or more datasets (channel and latent) for further analysis

- Each company brings results for training of own decoder / encoder

- Conclude on overall feasibility of Option 4

Proposal #5:	Further discuss and define the upper bound complexity including the number of computations and number of parameters for test/reference encoders/decoders for the purpose of performance requirements definition.

# References

- RP-221348, “SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #96, June 2022

- 3GPP TR 38.843 “Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface”

- RP-242399, “WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #105, September 2024

- RP-242387, “Moderator's summary on offline discussion about non-RAN2 aspects on REL-19 WI NR_AIML_Air”, RAN #105, September 2024

- R4-2403712, “WF on AI/ML”, Qualcomm, RAN4 #110, February 2024

- R4-2406617, “WF on AI/ML”, Qualcomm, RAN4 #110bis, April 2024

- R4-2410570, “WF on AI/ML”, Qualcomm, RAN4 #111, May 2024

- R4-2414323, “WF on AI/ML”, Qualcomm, RAN4 #112, August 2024

- R4-2414447, “Alignment of model training and data set parameters for AI/ML-based CSI compression”, Nokia, RAN4 #112, August 2024

- R4-2417212, “WF on AI/ML”, Qualcomm, RAN4 #112bis, October 2024

- R1-2402147, “AI/ML for CSI compression”, Intel Corporation, RAN1 #116bis, April 2024