3GPP TSG-RAN WG4 Meeting #113	R4-2418905

Orlando, US, November 18th-22nd, 2024

Source: 	Ericsson

Title:  	Discussion on other RRM requirements for NTN for NR Phase 3

Agenda Item:	7.26.4.2

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #1: Downlink coverage enhancements

#### Issue 2-1-1: RRM impact of SSB periodicity enhancement

Recall RAN1 and RAN2 agreements in last meetings as follows:

RAN1:

Excerpted from RAN1 #116 meeting: For system level study based on analytical evaluation: - N1 beam footprints are in state “off” - These beam footprints are not served by any signal (no satellite service in this area) - N2 beam footprints are in state “common messages only” - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access. - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation - N3 beam footprints are in state “active traffic” - These beam footprints have X active (e.g. VoNR) users each. - These beam footprints are also served the necessary information for cell discovery and initial access

As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint

For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

RAN2:

1. From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) 2. From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. 3. RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity 4. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

Accordingly, RAN1 has agreed to study the SSB periodicity extension for initial access.

From RRM perspective, the initial access feature isn’t in the scope, however, some RRM requirements are relevant with the default SSB periodicity, as shown below.

- Interruption time for handover and conditional handover:

- Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

- RRC Connection Release with Redirection:

- Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: - the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, - there is no requirement if the SSB transmission periodicity is larger than 20ms.

- Initial transmission timing error:

When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms.

A valid question arises: shall we revise or maintain the present definitions of Trs for handover, conditional handover and RRC Connection Release with Redirection?

Observation 1:  SSB periodicity extension may impact the present definitions of Trs defined for

- Handover  and conditional handover

- Connection Release with Redirection

In the same manner, the requirements: transmit timing requirement, timing advance adjustment accuracy also may be influenced by the extension of the SSB periodicity. As the content quoted in the above, Te is applicable provided that at least one SSB is available at the UE during the last 160 ms.

Observation 2: SSB periodicity extension may impact the present the present 160ms defined for Te requirement.

It appears that, due to the extension of the SSB periodicity and the underlying extension of SMTC and/or DRX, the measurement time delay is scaled up, which may subsequently degrade the measurement accuracy for the requirements including cell reselection, L1/L3 measurements, especially for the earth-moving cell case.

Observation 3: RRM requirements may be impacted by the scaled measurement time delay due to the extension of SSB/SMTC/DRX periodicity. The measurement herein may comprise cell reselection, L1/L3 measurements.

According to RAN2 observations, the rule of cell-reselection could be changed for the cells operating with DL CE, depending on the details of the RAN1 solution.

Observation 4: RRM requirements for cell-reselection on the cells (serving cell and/or neighbour cells) operating with DL CE may be impacted by DL coverage enhancement, depending on RAN1 and RAN2 progress.

Proposal 1: Below requirements may be impacted, depending on RAN1 and RAN2 progress.

- Trs defined for

  - Handover  and conditional handover

  - Connection Release with Redirection

- Te

- Measurement delays

- Cell-reselection

# Conclusion

Observation 1:  SSB periodicity extension may impact the present definitions of Trs defined for

- Handover  and conditional handover

- Connection Release with Redirection

Observation 2: SSB periodicity extension may impact the present the present 160ms defined for Te requirement.

Observation 3: RRM requirements, e.g., measurement accuracy, may be impacted by the scaled measurement time delay due to the extension of SSB/SMTC/DRX periodicity. The measurement herein may comprise cell reselection, L1/L3 measurements.

Observation 4: RRM requirements for cell-reselection on the cells (the serving cell and/or neighbour cells) operating with DL CE may be impacted by DL coverage enhancement, depending on RAN1 and RAN2 progress.

Proposal 1: Below requirements may be impacted, depending on RAN1 and RAN2 progress.

- Trs defined for

  - Handover  and conditional handover

  - Connection Release with Redirection

- Te

- Measurement delays

- Cell-reselection

# References

- R4-2416866, The WF on RRM requirements for Rel-19 NR NTN phase3, QC