3GPP TSG-RAN WG4 Meeting #113	R4-2418906

Orlando, US, November 18th-22nd, 2024

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Agenda Item:	7.15.2.1

Document for:	Discussion

# Introduction

In the light of the WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [2].

This contribution outlines our point of view on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor as follows.

# Discussion

## Topic #1: FR2-1 SSB based L3 measurement delay reduction for connected mode

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF:

Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

The conditions of FBS activation and deactivation shall be studied separately because the concerns are different.

For the FBS activation, we understand there shall be an event-triggered procedure including receiving, processing and reporting from UE perspective.

Similar to the requirements for CHO, RAN4 needs to define the requirements for FBS activation delay. Firstly, recall the relevant CHO delay in TS38.133 in the below.

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover Tmeasure is the measurements time stated in clause 6.1.4.2.2. TCHO_execution is the conditional execution preparation time in clause 6.1.4.2.3. Tinterrupt is the interruption time stated in clause 6.1.4.2.4.

We understand that the FBS activation delay shall refer to it with updates of FBS switching.

Proposal 1: FBS activation delay shall at the least includes below items.

When the UE receives a RRC message implying FBS the UE shall be ready to start the L3 measurements with FBS operation within DFBS seconds from the end of the last TTI containing the RRC command.

DFBS = TRRC + TEvent_FBS + Tmeasure + Tprocessing_FBS

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

TEvent_FBS is the delay uncertainty which is the time from when the UE successfully decodes the RRC command until a condition exists at the measurement reference point which will trigger the FBS.

Tprocessing_FBS is for UE processing and preparing FBS operation before UE applying FBS.

As shown in the outlined equation, another issue to be addressed is whether the UE shall report the FBS validity after activating FBS with respect to the NW RRC signaling. Since it wasn't covered in the previous meeting, we would want to express our opinions here. Since the purpose of the RRC signaling is to mandate the UE to activate FBS once certain condition is met, the NW shall be aware of whether the UE is doing FBS or not, which is helpful for the NW managing the mobility or scheduling work more efficiently. Otherwise, if no report to the NW, the RRC signaling is meaningless since anyway activating or deactivating BFS is purely up to UE implementation.  Preparing the UE mobility beforehand is also beneficial to the NW.

Proposal 2: UE shall report FBS validity to NW after activating FBS with respect to the NW RRC signaling.

Regarding the condition to activate FBS, it is common understanding that the main scenario of FBS is facilitating the rapid assessment of mobility, particularly when a UE experiences a low level or poor signal from the serving cell, necessitating a transition to a different serving cell. Given the above, we think existing events for L3 measurement ﻿report triggering can represent the condition when the UE is at cell edge. It appears that the event condition for FBS may be different from event condition for CHO, typically the event condition for FBS shall be earlier to be trigged.

In last meeting, at least one company proposed to reuse cellEdgeEvaluation or similar for relaxed measurements in the low activity RRC state, which is duplicated in the below table, as the condition to activate FBS.

##### Relaxed measurement criterion for UE not at cell edge The relaxed measurement criterion for UE not at cell edge is fulfilled when: - Srxlev > SSearchThresholdP, and, - Squal > SSearchThresholdQ, if SSearchThresholdQ is configured, Where: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB).

Instead of reusing cellEdgeEvaluation, we prefer to reuse the existing events for L3 measurement ﻿report. Especially, the mobility of the UE in the target scenario isn’t only relevant to the quality of the serving cell but also the quality of the neighbor cell to switch, which could not be reflected by cellEdgeEvaluation. Moreover, defining the conditions with respect to the UE location/mobility additionally necessitates the introduction of extra RRC Information Elements (IEs) in the signaling provided by the NW.

Proposal 3: Use one of events for L3 measurement ﻿report triggering as the condition to activate FBS.

For the deactivation of FBS, as per our understanding, at the least one or more than one of the below conditions need be studied.

- After the UE have activated FBS:

  - The condition of activating FBS is invalid.

    - As an example, when the UE moves from cell edge to non-cell edge, it’s not rationale to still maintain FBS.

    - To avoid too frequent ping-pong between FBS and non-FBS, a minimal during time in FBS/ non-FBS shall be defined. Before the end of the minimal during time, even the condition is changed, the FBS shall not be deactivated or the FBS shall not be activated if non-FBS is activated.

  - The condition of activating FBS still is valid.

    - L3 measurement with FBS has conducted for a time period.

      - As an example, the condition of activating FBS still is valid, but the UE is keeping a stationary at cell edge or there’s no qualified neighbour cell to switch. In this case, FBS may be disabled.

    - ﻿Internal overheating is detected.

- Meet activation conditions but not activate FBS yet:

  - Internal overheating is detected or other limitation from UE perspective.

Proposal 4: RAN4 to study the below FBS deactivation conditions:

- After the UE have activated FBS:

  - The condition of activating FBS is invalid.

    - Define a minimal during time for FBS/non-FBS.

  - The condition of activating FBS still is valid but,

    - L3 measurement with FBS has been conducted for a time period with respect to a pre-defined measurement period or time.

    - Internal overheating is detected or other limitation from UE perspective.

- Meet activation conditions but not activate FBS yet:

  - Internal overheating is detected or other limitation from UE perspective.

Proposal 5: UE shall report NW of deactivating FBS once the deactivation condition is met.

A further problem is if the UE deactivates FBS and then the UE determines that the deactivation condition is invalid or the activation condition is met again, can the UE re-activate FBS or the UE has to wait for the NW to send the RRC signaling to enable the event-triggered activation procedure?

Proposal 6: RAN4 to study the UE behaviour, provided the UE deactivates FBS and then the UE determines that the deactivation condition is invalid or the activation condition is met again. There are two exemplary behaviors as follow:

- UE directly re-activates FBS.

- UE keeps FBS deactivated and awaits until the new RRC signaling enabling the event-triggered activation procedure.

Yet another problem is, even it is agreed that FBS shall be applied during Tsearch during handover, which is duplicated in the below table, it still is a little unclear for CHO.

For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch).

In the CHO delay requirements, Tmeasure is the measurements time stated in clause 6.1.4.4.2 of TS38.133, it is the measurement delay from the time instant when the condition for handover is met. In this manner, it’s reasonable to adopt FBS to speed up the measurement in CHO as well.

Proposal 7: For CHO, FBS is applied during Tmeasure after the condition for handover is met.

We also observe the FBS for serving the mobility purpose may be triggered in an implicit way. For example, relaxed radio Link Monitoring (RLM) is designed for a UE to be able to operate these relaxation features when it is under good radio conditions and/or at a low mobility state. In other words, it means the radio conditions are stable and no need to request the UE to perform fast beam sweeping or to shorten the measurement delay time on the neighbor cells. In this sense, there could be some relationships between BFD and/or RLM and FBS.

On other hand, if beam failure or radio link failure is detected, FBS may be applied to speed up potential procedures. Since we have agreed that RRC re-establishment shall apply BFS, it actually reflects that radio link failure shall trigger BFS.

Proposal 8: BFD and/or RLM status may trigger UE to enable FBS.

- If the UE meets the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE hasn’t the demand to conduct FBS. On the contrary, if the UE doesn’t meet the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE may have the demand to conduct FBS.

- If beam failure or radio link failure is detected, FBS may be applied.

Yet another question is, since measurement delay is shorten,  whether the existing ReportInterval  shall be extended to contain a smaller value, if ﻿ the UE is configured with periodical reporting may impact the NW decision on handover command.

Proposal 9: RAN4 to study whether the existing ReportInterval  shall be extended to contain a smaller value for the measurement with FBS enabled.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC

A UE may store the SSB samples and post-process them for a full L3 measurement before starting receiving the SSB samples for the consecutive L3 measurement or other procedure.

In legacy, a processing time serving the function called as ﻿Tmargin  is allocated in the equation outlining handover interruption time, where Tmargin can reach a maximum of 2 milliseconds. To the contrary, the definitive duration for the processing time is not allocated in the time delay of cell identification, even the UE shall also process the received SSB samples in cell identification.

6.1.1.4 NR FR2- NR FR2 Handover The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR FR2 cell to NR FR2 cell. 6.1.1.4.1 Handover delay When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover ms from the end of the last TTI containing the RRC command. Where: Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.1.4.2. 6.1.1.4.2 Interruption time The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: Tsearch is the time required to search the target cell when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - If the target cell is a known intra-frequency cell, then Tsearch = 0ms. - If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. - If the target cell is a known inter-frequency cell, then - if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0ms; - if the measured SSB and the target SSB for handover belong to the same NR target cell and if the UE supports ncd-SSB-BWP-Wor-r18, Tsearch = Trs ms provided any one of the following conditions is satisfied: - CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in first active DL BWP is the target SSB for handover - NCD-SSB in a DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for handover - Both measured SSB and the target SSB for handover are NCD-SSB within different DL BWPs - Otherwise, the target cell is an unknown inter-frequency cell and Tsearch = N*3* Trs ms if the target cell Es/Iot≥-2 dB. Where N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. Tprocessing is time for UE processing. Tprocessing can be up to 20ms. Tmargin is time for SSB post-processing. Tmargin can be up to 2ms. T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cell.

However, L3 measurement requirements don’t reserve such processing time in the corresponding context, which is repeated in the follows.

9.2.5 Intrafrequency measurements without measurement gaps 9.2.5.1 Intrafrequency cell identification The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms Where: TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection - For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TPSS/SSS_sync_intra is given in Table 9.2.5.1-11; otherwise, TPSS/SSS_sync_intra is given in Table 9.2.5.1-2. - For UE indicating no-gap-no-interruption, TPSS/SSS_sync_intra is given in Table 9.2.5.1-1 for FR1 and Table 9.2.5.1-2 for FR2. For UE indicating no-gap-with-interruption, TPSS/SSS_sync_intra is given in Table 9.2.5.1-17 for FR1 and Table 9.2.5.1-18 for FR2. - Otherwise, TPSS/SSS_sync_intra is given in table 9.2.5.1-1, 9.2.5.1-2, 9.2.5.1-4 (deactivated SCell) or 9.2.5.1-5 (deactivated SCell) or 9.2.5.1-9 (deactivated SCell) or 9.2.5.1-11 or 9.2.5.1-12 (deactivated PSCell) or 9.2.5.1-13 (deactivated PSCell). TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured

To our understanding, considering processing time differently is dependent on the use cases.

- For handover, Tmargin shall be reserved explicitly in the handover interruption time requirement since of no headroom for the UE to flexibly arrange the SSB processing time.

- By contrast, for L3 measurements, in terms of the practical UE implementation, it is conceivable to embed the processing time in the overall cell identification delay, as the cell identification procedure consumes multiple SMTC/DRX occasions/intervals and is able to provide adequate opportunity for processing SSB samples while maintaining receiving SSBs.

Proposal 10: Extra processing time can be applied to handover interruption time, e.g. Tmargin, if necessary. No extra processing time for cell identification delay.

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX

The below is summary in the last meeting.

- Option 1 (Xiaomi): The SSB based L3 measurement delay reduction based on FBS can be dependent with DRX configuration. - Option 2 (CATT, CMCC, Samsung, CTC, NTT DCM, Nokia): L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case. - Option 3 (Ericsson): - L3 measurement enhancement is applicable to UE configured with DRX. It can be studied if some DRX cases really impact performance. - When L3 measurement enhancement is enabled, DRX can be ignored during the fast L3 measurement from tech. perspective. [Moderator]: Is option 2 agreeable?

Actually, our opinion is same as Option 2. Moreover, we’d like to check if the UE shall skip/ignore the DRX configuration for measurement when FBS is valid, i.e., the measurement delay requirement ignores DRX periodicity, if FBS is enabled.

Proposal 11: L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX.

Proposal 12: RAN4 to study whether DRX can be ignored when FBS is valid.

Issue 1-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Until now, it is concerned that the number of RX beam sweeping factor is dependent on UE capability.

In principle, FBS enables the simultaneous reception on multiple (2 typically or as default) panels. That’s the reason we proposed in the last meeting that the RX beam sweeping factor = 4. However, we’re open to the RX beam sweeping factor less than 4, if the UE can apply less beams for L3 measurements on the serving cell and neighbor cells.

While, we have concerns on directly applying the UE capability [TBD - multi-rx fast beam switching capability] for the L3 measurement delay reduction, since L1 measurement adopting [TBD - multi-rx fast beam switching capability] may have different beam shapes and/or beam scheme from L3 measurement. Reusing the same UE capability may result in the loss of flexibility of RX beam sweeping implementation.

Proposal 13: RX beam sweeping factor can be defined with respect to UE capability.

Proposal 14: Introduce a new UE capability instead of reusing UE capability [TBD - multi-rx fast beam switching capability] for L3 measurement delay reduction.

Issue 1-4-1: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement

- Proposal (ZTE): - The requirement at transition between the normal L3 measurement and FBS L3 measurement should be identified. There are two options: - Option 1: Reuse similar solution as the requirements at transition between measurement within gap and measurement outside gap, i.e. the longer delay is the period when transition happens. - Option 2: Once the transition happens, apply the requirements after the transition, no matter it is the normal L3 measurement or the FBS L3 measurement. - Recommended WF - TBA

We may need to take into account two switch directions, from non-FBS to FBS and from FBS to non-FBS.

- For the case of switching from single panel reception (non-FBS) to multiple panel reception (FBS), the UE may need transition time to modify/update the processing.  Since we have defined that the UE shall enable FBS once the condition for FBS is met, it is fair to specify that a delay is expected from the time instant when the condition is met.

- For the case from FBS to non-FBS, we have not discussed the fallback solution yet. It’s unclear that whether it’s really necessary to explicitly highlight the delay in requirements.

Proposal 15: Regarding the transition time,

- For the case from non-FBS to FBS, RAN4 specify a delay from the time instant when the condition is met.

- For the case from FBS to non-FBS, the necessity of a delay during transition is unclear since the fallback solution isn’t fixed.

# Conclusion

Proposal 1: FBS activation delay shall at the least includes below items.

When the UE receives a RRC message implying FBS the UE shall be ready to start the L3 measurements with FBS operation within DFBS seconds from the end of the last TTI containing the RRC command.

DFBS = TRRC + TEvent_FBS + Tmeasure + Tprocessing_FBS

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

TEvent_FBS is the delay uncertainty which is the time from when the UE successfully decodes the RRC command until a condition exists at the measurement reference point which will trigger the FBS.

Tprocessing_FBS is for UE processing and preparing FBS operation before UE applying FBS.

Proposal 2: UE shall report FBS validity to NW after activating FBS with respect to the NW RRC signaling.

Proposal 3: Use one of events for L3 measurement ﻿report triggering as the condition to activate FBS.

Proposal 4: RAN4 to study the below FBS deactivation conditions:

- After the UE have activated FBS:

  - The condition of activating FBS is invalid.

    - Define a minimal during time for FBS/non-FBS.

  - The condition of activating FBS still is valid but,

    - L3 measurement with FBS has been conducted for a time period with respect to a pre-defined measurement period or time.

    - Internal overheating is detected or other limitation from UE perspective.

- Meet activation conditions but not activate FBS yet:

  - Internal overheating is detected or other limitation from UE perspective.

Proposal 5: UE shall report NW of deactivating FBS once the deactivation condition is met.

Proposal 6: RAN4 to study the UE behaviour, provided the UE deactivates FBS and then the UE determines that the deactivation condition is invalid or the activation condition is met again. There are two exemplary behaviors as follow:

- UE directly re-activates FBS.

- UE keeps FBS deactivated and awaits until the new RRC signaling enabling the event-triggered activation procedure.

Proposal 7: For CHO, FBS is applied during Tmeasure after the condition for handover is met.

Proposal 8: BFD and/or RLM status may trigger UE to enable FBS.

- If the UE meets the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE hasn’t the demand to conduct FBS. On the contrary, if the UE doesn’t meet the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE may have the demand to conduct FBS.

- If beam failure or radio link failure is detected, FBS may be applied.

Proposal 9: RAN4 to study whether the existing ReportInterval  shall be extended to contain a smaller value for the measurement with FBS enabled.

Proposal 10: Extra processing time can be applied to handover interruption time, e.g. Tmargin, if necessary. No extra processing time for cell identification delay.

Proposal 11: L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX.

Proposal 12: RAN4 to study whether DRX can be ignored when FBS is valid.

Proposal 13: RX beam sweeping factor can be defined with respect to UE capability.

Proposal 14: Introduce a new UE capability instead of reusing UE capability [TBD - multi-rx fast beam switching capability] for L3 measurement delay reduction.

Proposal 15: Regarding the transition time,

- For the case from non-FBS to FBS, RAN4 specify a delay from the time instant when the condition is met.

- For the case from FBS to non-FBS, the necessity of a delay during transition is unclear since the fallback solution isn’t fixed.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2416853, WF for NR_RRM_Ph5, Apple