# 3GPP TSG-RAN WG4 Meeting #113	R4-2418911

Orlando, Florida, US, 18th – 22nd November, 2024

Source:			Ericsson

Title:			NTN Ku-band – FR1-TN vs FR2-TN

Agenda item:		7.9.3

Document for:		Discussion

# Introduction

In RAN#104, a new WI ([1]) has been approved to specify new NTN Ku band(s). One of the WI objectives is to enable 125 MHz in UL and 250 MHz, considering either FR1-TN range or FR2-TN range.

This contribution is further discussing the different approaches, based on the agreed WFs ([2] and [3]) from last RAN4#112 and RAN4#112bis meetings.

# Discussion

## WI objectives

In last RAN4#112 meeting, the following agreements ([2]) related to the FR choice for the new NTN Ku-band(s) were made:

## Efficient channel utilization

As we have shown in our previous contribution ([4]), FR1 channel bandwidths would be more efficient for narrow bandwidth, i.e. 36 MHz, while FR2 channel bandwidths would be more suitable for wider channel bandwidth like 250 MHz avoiding CA configuration with 3 carriers.

The following table compares the spectrum utilization for the targeted aggregated bandwidth, using FR1 or FR2 channel bandwidth.

| Targeted aggregated channel BW | FR1 channel BW |  | FR2 channel BW |  |
| --- | --- | --- | --- | --- |
|  | Spectrum utilization (MHz) | BW Combination / SCS | Spectrum utilization (MHz) | BW Combination / SCS |
| 36 MHz | 33.84 | 35 MHz / 15kHz | NA | NA |
| 72 MHz | 68.04 | 70 MHz / 30kHz | 47.52 | 50 MHz / 60kHz |
| 250 MHz DL | 244.44 | 2*100 + 50 MHz / 30 kHz | 237.6 | 200 + 50 MHz / 60 kHz |
| 125 MHz UL | 121.68 | 100 + 25 MHz / 30 kHz | 95.172 | 100 MHz / 60 kHz |

Observation1: FR1-NTN would be more efficient for narrow channel bandwidth (e.g. 36MHz) while FR2-NTN would be more efficient for wider channel bandwidth (e.g. 250 MHz).

## Misc

Also, if NTN KU-band(s) should be part of FR1-NTN, it should be noted that, currently, the RF and demodulation requirements for VSAT type of UE are only specified for bands belonging to FR2-NTN. This would require considerable effort to specify VSAT requirement for the Ku-band(s) then, comparable to the effort spent on the Ku-band.

Observation2: RF and demodulation requirements are specified for NTN UE types 1-5 when operating in bands belonging to FR2-NTN. It would require considerable effort to develop again those requirements for  FR1-NTN.

As it was highlighted in last RAN4#112bis meeting, SAN operating above 10 GHz are most likely to operate simultaneously in several NTN bands, i.e. Ku and Ka-band. Also, by browsing on the net, it seems quite common nowadays to find VSAT devices supporting operation both in Ka and Ku-band. It would be costly and complex to design a SAN or a VSAT that should support both FR1-NTN and FR2-NTN numerology. It would be then future proof to consider that the Ku-band belongs to the same frequency range than the Ka-band, i.e. FR2-NTN.

Observation3: SAN operating above 10 GHz might support both Ku and Ka-band.

Observation4: It seems common that a VSAT device supports operation both in Ku-band and Ka-band.

Based on above the above observation, we would make the following proposal:

Proposal: Consider the NTN Ku-band(s) as part of FR2-NTN frequency range.

# Conclusion

In this contribution, we further analyzed the impacts of considering the NTN Ku-band(s) as part of FR1-NTN or FR2-NTN. We made the following observations and proposal:

Observation1: FR1-NTN would be more efficient for narrow channel bandwidth (e.g. 36MHz) while FR2-NTN would be more efficient for wider channel bandwidth (e.g. 250 MHz).

Observation2: RF and demodulation requirements are specified for NTN UE types 1-5 when operating in bands belonging to FR2-NTN. It would require considerable effort to develop again those requirements for  FR1-NTN.

Observation3: SAN operating above 10 GHz might support both Ku and Ka-band.

Observation4: It seems common that a VSAT device supports operation both in Ku-band and Ka-band.

Proposal: Consider the NTN Ku-band(s) as part of FR2-NTN frequency range.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Eutelsat

- R4-2416141, NTN Ku-band – FR1-TN vs FR2-TN, Ericsson