# 3GPP TSG-RAN WG4 Meeting #113	R4-2418915

Orlando, Florida, US, 18th – 22nd November, 2024

Source:				Ericsson

Title:					Lower band consideration for AAS BS

Agenda item:		4.3

Document for:		Approval

# Introduction

In last RAN4#112bis meeting, a way forward ([1]) was agreed to clarify from which frequency an AAS BS could be considered. Based on this common understanding, RAN4 could then update the BS specifications and have a consistent approach when considering any NR band.

Based on this WF, we are sharing our view in this contribution and list the impacted TSs to be updated

# Discussion

## Background

The way forward ([1]) encourages companies to share their view on the AAS low frequency limit, from which Release this should be applicable and to propose a list of specifications to be updated:

## AAS Low frequency limit

We propose 600MHz as the low frequency limit for AAS BS. From this 600MHz frequency, we think it would be possible to build an AAS BS.

Proposal1: Consider 600MHz as the AAS low frequency limit (AAS_low_limit).

## Preferred release

Ideally, such limit should be applicable from Rel-15, or even earlier release when AAS was introduced. Nevertheless, to reduce the drafting effort, we would propose to adopt this limit from Rel-17 only.

Proposal2: Implement the clarification on AAS low frequency limit from Rel-17.

## List of impacted specifications

To our view, the following specifications might be impacted and need to be checked based on the agreed AAS low frequency limit:

- TS 38.141-2 (NR conformance radiated)

- TS 37.105 (AAS BS core specification)

- TS 37.145-2 (AAS conformance radiated)

- TS 38.176-2 (IAB conformance radiated)

It should also be clarified that, below this frequency threshold, BS type 1-H and BS type 1-O are not considered as supported in all AAS and NR specifications (TS 37.105, TS 37.145-1, TS 37.145-2, TS 38.104, TS 38.141-1, TS 38.141-2, TS 38.106, TS 38.115-1, TS 38.115-2, TS 38.174, TS 38.176-1, TS 38.176-2).

This is especially needed for specifications where only conducted requirements are considered (e.g. TS 38.141-1), to avoid any ambiguity with BS type 1-H support.

Proposal3: Based on the agreement on the AAS lower frequency limit, the following specifications should be checked and possibly updated: TS 38.141-2, TS 37.105, TS 37.145-2 and TS 38.176-2.

Proposal4: Add a note in all AAS and NR specifications clarifying that, below the AAS lower frequency limit, BS type 1-H and BS type 1-O are considered as not supported.

## BS type 1-H and type 1-C

As RAN4 is discussing lower frequency limit for AAS BS, it might also be a good opportunity to discuss the upper frequency limit for BS type 1-H and type 1-C, this would avoid useless discussion every time a new band is introduced.

The same principles used to select the frequency limit for AAS BS could be used, selecting the maximum proposed frequency by companies.

Proposal5: RAN4 to agree on the upper frequency limit for which a BS could be of type 1-C. The selected value may be derived using the maximum value of all proposals.

Proposal6: RAN4 to agree on the upper frequency limit for which a BS could be of type 1-H. The selected value may be derived using the maximum value of all proposals.

# Conclusion

In this contribution, we discussed the AAS lower frequency limit based on the RAN4#112 bis way forward ([1]) and made the following proposals:

Proposal1: Consider 600MHz as the AAS low frequency limit (AAS_low_limit).

Proposal2: Implement the clarification on AAS low frequency limit from Rel-17.

Proposal3: Based on the agreement on the AAS lower frequency limit, the following specifications should be checked and possibly updated: TS 38.141-2, TS 37.105, TS 37.145-2 and TS 38.176-2.

Proposal4: Add a note in all AAS and NR specifications clarifying that, below the AAS lower frequency limit, BS type 1-H and BS type 1-O are considered as not supported.

Proposal5: RAN4 to agree on the upper frequency limit for which a BS could be of type 1-C. The selected value may be derived using the maximum value of all proposals.

Proposal6: RAN4 to agree on the upper frequency limit for which a BS could be of type 1-H. The selected value may be derived using the maximum value of all proposals.

# Reference

- R4-2417198, Way Forward for [112bis][110] on low bands consideration for AAS BS, Huawei