# 3GPP TSG-RAN WG4 Meeting #113	R4-2418916

Orlando, Florida, US, 18th – 22nd November, 2024

Source:				Ericsson

Title:					LS Reply to RAN1 on the clarification of FR2-NTN frequency range definition

Agenda item:		8.2

Document for:		Approval

# Introduction

RAN1 sent a LS [4] to RAN4 mentioning they have completed the work to support NR NTN for the frequency bands defined as part of FR2-NTN, highlighting RAN4 should better clarify FR2-NTN definition.

This contribution is discussing this aspect, based on the agreed WF ([4]) in RAN4#112bis, proposing a reply to RAN2 LS in annex.

# Discussion

In the LS ([1]) sent to RAN4, RAN1 asks RAN4 to consider updating the note on FR2-NTN definition in table 5.1-1.

We agree this note might be confusing, the reader might not understand if RAN4 is referring to FR2-1 and/or FR2-2.

In last RAN4#112bis meeting, it was agreed that FR2-NTN follows the FR2-1 procedures and the following options were captured in the WF when discussing updating the notes in the FR tables ([4]):

To our view, we can’t just remove the notes, at least as long as similar notes is not captured in RAN1 specifications. That would be extremely confusing as reader would not understand the applicability of those new NTN FR ranges.

So, option 1 and option 2 are not acceptable for us at this stage.

Regarding the other options:

- Option 3a is the one we proposed in last meeting, by submitting corresponding draft CRs.

- Option 3b slightly changes the wording in note 2, which doesn’t look necessary.

- Option 3c is more precise, mentioning FR2-NTN should be considered as FR2 in specifications where FR2 is mentioned, and it should be considered as FR2-1 when there is a distinction made between FR2-1 and FR2-2. This option would be acceptable to us.

- Option 3d doesn’t propose any change really to the existing note.

Proposal1: update the note 2 in table 5.1-1 (TS 38.108 and TS 38.101-1) according to option 3a or eventually option 3c.

We propose so to update this note in both TS 38.101-5 and TS 38.104, as suggested in our companion CRs ([2] and [3]), and reply to RAN1 LS to acknowledge such requested update.

Proposal2: Approve the LS reply to RAN1 in Annex and endorsed our companions CRs ([2] and [3]).

# Conclusion

In this contribution, we discussed the RAN1 LS and proposed a reply in Annex.

Proposal1: update the note 2 in table 5.1-1 (TS 38.108 and TS 38.101-1) according to option 3a or eventually option 3c.

Proposal2: Approve the LS reply to RAN1 in Annex and endorsed our companions CRs ([2] and [3]).

# Reference

- R1-2407406, LS on FR2-NTN inclusion to specifications, RAN1

- R4-2418917, CR to TS 38.101-5 – NTN frequency range and RAN1 LS, Ericsson

- R4-2418918, CR to TS 38.108 – NTN frequency range and RAN1 LS, Ericsson

- R4-2417185, WF on FR2-NTN inclusion to specifications, Apple

# Annex : LS reply to RAN1 on FR2-NTN frequency range definition

# 3GPP TSG-RAN WG4 Meeting #113	R4-241xxxx

Orlando, Florida, US, 18th – 22nd November, 2024

Title:	LS Reply to RAN1 on FR2-NTN inclusion to specifications

Response to:	R1-2407406

Release:	Release 18

Work Item:	NR_NTN_solutions-Core

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:		Dominique Everaere

- E-mail Address:	dominique.everaere@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for their LS R1-2407406.

RAN4 acknowledges that the note 2 in table 5.1-1 should be updated, clarifying that the band should be regarded as a FR2-1 band, and not a FR2-2 one, as highlighted below:

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 7125 MHz |
| FR2-NTN (Note 2) | 17300 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications. |  |

The attached CRs were agreed in next RAN4#113 meeting.

2. Actions:

To RAN WG1.

ACTION: 	RAN4 respectfully requests RAN1 to take into consideration the above information.

3. Date of Next TSG-RAN WG4 Meeting:

RAN4#114	from 2025-02-17	to 2025-02-21		Athens, Greece