# 3GPP TSG-RAN WG4 Meeting #113	R4-2418919

Orlando, Florida, US, 18th – 22nd November, 2024

Source:			Ericsson

Title:			mmWave in NR: EESS protection – UE RF aspects

Agenda item:		6.21.2

Document for:		Discussion

# Introduction

In RAN#105 meeting, a new WI was adopted to consider China and EC Regulation related to the 24GHz band updates and the EESS protection. And in last RAN4#112bis, a way forward ([9]) was agreed based on preliminary discussion.

This contribution is further discussing the impacts on UE RF aspects based on this way forward.

# Discussion

## Background (reminder)

When RAN4 specified the mmWave NR bands n257 and n258, ITU-R regulation and WRC-19 Resolution 750 was considered.

On UE side, this was captured partly in TS 38.101-2, with the following additional spurious requirements handled via NS_202 and NS_203 as shown below:

Only the first stage limit (-29dBW/200MHz) from WRC-19 Resolution 750 is captured, the second stage is not mentioned.

Observation: TS 38.101-2 only captures the first stage limit from WRC-19 Resolution 750 (valid until 1 September 2027 when following ITU-R RR).

## Europe situation

In April 2020, the EU Decision 2019/784 ([2]) was amended with the Commission Implementing Decision 2020/590 ([3]). Based on CEPT recommendation ([4]), the EU decided to set the transition to the second stage limit to an earlier date (1 January 2024) , this to better protect EESS in the 23.6-24.0 GHz frequency range while early enabling 26GHz deployment. Such decision is binding to all EU countries.

## TS 38.101-2 impacts

### NS_202

In last RAN4#112bis meeting, the following way forward [9] related to NS_202 was agreed:

NS_202 was initially specified to handle ERC 74-02 spurious emission limits ([6]). It was then updated ([7]) to consider potential EU emission limits of +1dBm/200MHz discussed in RSCOM but, as far as we investigated, no EC Decision was published based on such discussion. Rather, as we mentioned previously, the amended EU Decision specified a limit of -5dBm/200 MHz which was not considered in TS 38.101-2.

Considering that this -5dBm/200MHz is applicable from 1 January 2024 for any device sold on the EU market (even if it was produced earlier), our proposal is to update NS_202, changing the 1dBm/200MHz to -5dBm/200 MHz limit.

Usually, RAN4 doesn’t update any NS to avoid any NBC issue. But here, the situation is different as the U Regulation is already applicable from 1 January 2024, meaning that no device could be sold in a shop in Europe nowadays if it’s not compliant with this regulation. The NS_202 update is then needed.

Note that this could be done in the scope of this WI or in maintenance, as long as it’s done.

Proposal1: Update NS_202 (and CA_NS_202) changing the +1dBm/200MHz limit to -5dBm/200MHz.

### New NS

NS_203 was specified to handle the WRC-19 Resolution 750 ([8]). It specifies only the first stage limit of 1dBm/200MHz. To address China’s situation and anticipate the second stage limit which will be appliable from 1 September 2027, a new NS would be needed. This is even more justified that, as shown in [10] and [11], A-MPR would be required at least for PC1 to fulfil this more stringent limit.

Proposal2: Specify a new NS_205 to manage the second stage limit of -5dBm/200 MHz in the 23.6-24.0 GHz frequency range.  A corresponding new NS CA_NS_205 for CA should also be specified.

Also, to address the handover issue where only one NS could be indicated in the RRC configuration, RAN4 should adopt the same approach that was used for NS_203, indicating NS_205 support in the modifiedMPR-behavior field.

In last RAN4#112bis meeting, it was also proposed ([12]) that, if no additional A-MPR would be needed for the 27.4-27.8 GHz frequency range, it might be possible to specify this more stringent limit as a general requirement (i.e. UE coexistence) as well. According to [10], this would be the case except for PC1 UEs.

Observation: If no additional A-MPR is needed for the 27.4-27.8 GHz frequency range when applying the more stringent limit, it would be possible this more stringent limit as a general requirement and the new NS would not be needed anymore at least for 27.4-27.8 GHz.

Our companion draft CR ([5]) proposes an initial draft of expected changes. Note that this draft CR is made on Rel-18 specifications to highlight all possible changes but they should be done to earlier release as well.

# Conclusion

In this contribution, we analyzed the impacts of Chinese and European regulation related to the EESS protection in the 23.6-24.0 GHz frequency range. We made the following observation and proposal:

Observation: TS 38.101-2 only captures the first stage limit from WRC-19 Resolution 750 (valid until 1 September 2027 when following ITU-R RR).

Proposal1: Update NS_202 (and CA_NS_202) changing the +1dBm/200MHz limit to -5dBm/200MHz.

Proposal2: Specify a new NS_205 to manage the second stage limit of -5dBm/200 MHz in the 23.6-24.0 GHz frequency range. This NS CA_NS_205 should only be used from 1 September 2027. A corresponding new NS for CA should also be specified.

# References

- RP-242371, WID on mmWave UE spurious emssion, China Unicom, Deutsche Telekom, Vodafone, CATT, Huawei, HiSilicon, OPPO, Qualcomm, Xiaomi, ZTE

- EU Decision 2019/784, Commission Implementing Decision (EU) 2019/784 of 14 May 2019 on harmonisation of the 24,25-27,5 GHz frequency band for terrestrial systems capable of providing wireless broadband electronic communications services in the Union.

- EU Decision 2020/590, Commission Implementing Decision (EU) 2020/590 of 24 April 2020 amending Decision (EU) 2019/784 as regards an update of relevant technical conditions applicable to the 24,25-27,5 GHz frequency band.

- ECC(20) 055 Annex 18, CEPT letter dated 6 March 2020 ‘CEPT response on additional input regarding the impact of the WRC-19 outcome on the harmonised technical conditions for the 26 GHz band’

- R4-2418921, draft CR to TS 38.101-2 – EESS protection in the 23.6-24.0 GHz frequency range, Ericsson

- R4-1910261, dCR to 38.101-2: FR2 A-MPR updates, including ERC 74-01 changes, Qualcomm

- R4-2006785, CR to 38.101-2: NS_202 update after changes to EU regulations, Qualcomm

- R4-2016785, EESS protection related requirements for FR2 bands, Nokia, Nokia Shanghai Bell, NTT DOCOMO, INC

- R4-2417083, WF on UE RF requirements for EESS protection, Huawei, China Unicom

- R4-2415726, 24 GHz EESS band phase 2 requirements and UE compliance, Qualcomm

- R4-2000216, Impact of WRC19 resolutions on FR2, Qualcomm

- R4-2416194, Views on EESS protection for n257, NTT Docomo Inc