3GPP TSG RAN WG4 Meeting #112bis	R4-2418941

Orlando, Florida, U.S.A, 18th – 22nd November 2024

Source: 		Rohde & Schwarz

Title: 			Discussion on test setup for AI/ML based beam management

Agenda Item: 	7.17.4.2

Document for: 	Discussion

# Introduction

For AI/ML based beam management, core requirements and testability aspects are discussed together, as they have a direct interdependency. In the previous meetings, the channel model has been identified as one of the main key factors impacting the test equipment complexity. According to the WF agreed in [4], TE vendors need to provide more feedback on this. In the mean time there will be simulations for AI/ML BM performance from UE vendors, with different channel models, as agreed in the simulation assumptions in [5].

In this paper we provide some analysis on testing with CDL and TDL channel models and the respective test equipment complexity.

# Discussion

## CDL Channel model

From the WF in [4] following analysis is required with regard of CDL channel model:

- For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes.

  - Set A and B should be generated in the chamber for ground-truth extraction.

### Reference number of probes for spatial CDL

The basic framework for our analysis we provided already in [2], which we briefly summarize once again in this section. To conclude on the number of probes required to emulate spatially a CDL channel, a simulation with following assumption was done:

- Reference TR 38.827 [6] and TS 38.151 [7] (as used for MIMO-OTA / 3D MPAC)

- Channel model CDL-C UMi (TR 38.827 [6], Table 7.2.2-3) with 24 Clusters

- fixed delay, fixed angular spread, fixed coupling of ZoD/ZoA/AoD/AoA rays within one cluster

- BS code book with 128 Tx beams (16 in azimuth [-60°,60°] and 8 in elevation [-25°, 25°])

- BS with 8x16 antenna array (single panel)

- UE with 4x4 antenna (single panel)

- 28GHz

- We consider 8 beams (Set B) for BS Tx beam sweeping out of 128 beams (Set A)

BS Beams (B1-B128) and CDL-C UMi clusters (C1-C24) have been depicted in the figures below.

We can think of 4 typical scenarios of distribution / selection of the 8 beams as shown in Figure 1.

Scenario 1: No knowledge of channel 	  			Scenario 2: Partial knowledge of channel (Elevation)

Scenario 3: Best knowledge of channel	 	  			Scenario 4: Strongest cluster

Figure 1 – 8 BS Tx beams with different spatial distributions

After the simulation, the Spherical Power Spectrum (SPS) for each of the 8 beams and for each of the distribution scenarios, has been shown in Figure 2.

The power has been normalized for each individual beam to the strongest cluster, up to a dynamic of 30dB (i.e. 0dB to -30dB). The diagrams give also an impression of the total angular spread that needs to be emulated.

Scenario 1: No knowledge of channel 	  			Scenario 2: Partial knowledge of channel (Elevation)

Scenario 3: Best knowledge of channel	 	  			Scenario 4: Strongest cluster

Figure 2 – SPS at the UE end for 8 BS Tx beams

From the SPS diagrams we can roughly estimate the number of probes required to emulate each beam, following the approach agreed in [6] for MIMO-OTA.

We can take as example the Spatial distribution scenario 2. For some of the beams, the selection and the number of probes required for TE emulation is given in Figure 3.

Beam #87  6 Probes	  						Beam #85  13 Probes

Beam #83  20 Probes	  						Beam #81  24 Probes

Figure 3 – TE Probe locations (Spatial distribution scenario 2)

As a result, in order to simulate all 4 beams, we need 6+13+20+24 = 63, say roughly 50 Probes, which means for 8 beams roughly 100 Probes = 200 RF channels (considering V+H).

Even when following another approach (4-6 Probes per strong Cluster), the result will be similar.

Observation 1: Following the MIMO-OTA / 3D MPAC methodology, to emulate 8 BS Tx Beams with a CDL-C UMi channel requires a test system with ≥100 probes and ≥200 RF channels which is unfeasible.

According to the probe derivation approach above, the number of required probes depends on both, number of Tx beams, number of channel clusters, as well as their mutual spatial relation (i.e. beam spatial distribution, since the clusters are fixed). Thus, if we intend to reduce the system complexity and number of probes, we need to investigate in both areas.

Observation 2: For a feasible spatial (multi-AoA) emulation of the CDL channel, simplifications in terms of number of Tx beams, their spatial distribution and channel model need to be investigated.

### Dependencies of prediction performance from beams number and spatial distribution

Due to the short time between RAN4#112bis and RAN4#113, a further analysis on potential simplification of the channel model is not yet available. However, we focused on the other aspect considered as a potential factor for simplification: the number of Tx beams and their spatial distribution. We analyzed the relevance for the AI/ML BM requirement definition and testability discussion, following a top-down approach, i.e. investigating the impact of different Tx beam number and spatial distribution on the AI/ML beam prediction performance.

For this purpose, we performed simulations with the following configurations:

- General: 28GHz, SCS 120kHz, 8RBs, BS TX Array 8x4, UE Rx Array 4x1, CDL-C UMi

- AI/ML Model: CNN, DNN

- Codebook / Set A beams: 32, 64, 128

- Set B beams: 8, 16

We defined different combinations of Set B / Set A beams, including a variety of selection i.e. spatial distribution of Set B beams, as shown in Figure 4.

[8B, 32A] = 8 Set B Beams to predict 32 Set A Beams

[8B, 64A] =  8 Set B Beams to predict 64 Set A Beams 		[16B, 128A] = 16 Set B Beams to predict 128 Set A Beams

[8B, 128A] (1) = 8 Set B Beams to predict 128 Set A Beams (1) 	    [8B, 128A] (2) = 8 Set B Beams to predict 128 Set A Beams (2)

[8B, 128A] (3) = 8 Set B Beams to predict 128 Set A Beams (3)		[16B, 128A] = 16 Set B Beams to predict 128 Set A Beams

Figure 4 – Set B beams to predict Set A beams

For each of these cases we ran simulations with 2 different AI/ML models, CNN and DNN, to predict the Top3 and Top1 beams from Set A. The results (percentage of correct prediction) are summarized in the diagrams in Figure 5.

Figure 5 – Prediction accuracy probability

After comparing the results carefully, we can observe 2 main inter-dependencies.

First, it seems obvious, that prediction performance improves when ratio Set B / Set A increases.

Example from Figure 5: Performance for [8B, 32A] and [16B, 64A] are similar (for Top 3 beams) and both are better than [16B, 128A].

This means that – as expected – for a given Set A, we need to emulate more Set B beams for better prediction performance.

Observation 3:  AI/ML Beam prediction performance (e.g. Top 3, Top 1 Set AI beams) improves as the ratio Set B / Set A increases. For a given Set A, we need to emulate more Set B beams for better performance, especially for the prediction of the best (Top 1) beam.

Secondly, it can be observed, that the spatial distribution of Set B beams (as a result of the BS Tx Beams sweeping pattern) affects the prediction performance. A more scattered spatial distribution. is better for the prediction of more beams.

Example from Figure 5: [8B, 128A] (Distribution 1) and [8B, 128A] (Distribution 3) lead to better prediction of Top 3 beams, whereas the distribution [8B, 128A] (Distribution 2) rather for Top 1 beam.

This means that for the prediction of more beams, we need a wider spatial distribution of the Set B beams.

Observation 4:  The spatial distribution of Set B beams affects the prediction performance. For a better prediction of multiple beams (e.g. Top 3) a wider spatial distribution of Set B beams is required.

One of the potential ways to reduce the complexity of the test system which was proposed in last meetings, was to emulate multiple co-located Tx beams (same probe / AoA) with different power offsets. However, such new beams do not improve the spatial distribution and as per observation 4 would not contribute to a better prediction. In fact, these beams might have unexpected impact on a CDL trained AI/ML model, since they are co-located and introduce inappropriate spatial information.

Observation 5:  Emulating power-offset Set B beams, does probably not bring much value for prediction performance and might have unexpected impact on a CDL trained AI/ML model, since such beams are co-located and introduce inappropriate spatial information.

So far, the focus was on Set B beams.  However, according to the WF in [4], in case of CDL, we need to emulate all the Set A beams in order to extract the ground truth.

Observation 6:  Set A beams (up to 128) needs to be generated in the chamber for ground truth extraction.

The above analysis shows that the number of beams and their spatial distribution need to remain high for a good AI/ML prediction performance and ground truth extraction. At least with this regard, there seems to be no justified approach to significantly reduce the beam number, their spatial distribution and as such the complexity of the system.

Observation 7:  The overall number of beams and their spatial distribution needs to remain high for a good AI/ML prediction performance and ground truth extraction.

Considering Observation 7 and referring to the reference number of probes in Observation 1, we conclude that:

Observation 8: Since for a meaningful AI/ML performance the number of beams and their spatial distribution is required to be high (>8, ≤128), even significant simplification of the channel model (currently 8 beams requires 100 probes), would rather not downscale the number of probes to a feasible spatial (multi-AoA) emulation of CDL in TE.

### Single-AoA CDL

The different aspect impacting the end to end UE performance for AI/ML based BM can be described in our view as below in Figure 6 (top down; from BTS towards the UE)

Figure 6 – AI/ML BM Layers Model

From our perspective the main reason of considering a spatial emulation of the CDL channel, is to include the UE Rx Beam/Antenna pattern and UE Rx Beam sweeping performance in the AI/ML BM requirement.

Observation 9: Main reason for spatial (multi-AoA) emulation of CDL channel model is probably the inclusion of UE Rx Beam/Antenna pattern and UE Rx Beam sweeping performance in the AI/ML BM requirements.

However, as described in section 2.1.1+2., this leads currently to an unfeasible TE complexity. We can try to adapt and segregate the testing of these aspects, targeting a feasible test system and still achieving the AI/ML BM testing scope.

A spatial emulation of the CDL channel requires a multi-AoA/probe system. For simplicity, let us assume a 2AoA/probe environment with 2 Beams as in Figure 7.

Figure 7 – CDL from single AoA with knowledge of UE Rx Gains

In this case:

- Beam 1 from AoA1, reaches the UE RF/BB with a power B1(AoA1) + G(AoA1), where G(AoA1) is the UE Rx Beam Gain in AoA1 direction.

- Similarly, Beam 2 from AoA2, reaches the UE RF/BB with power B2(AoA2) + G(AoA2)

If we feed Beam 2 from AoA1, instead of from AoA2, by correcting the power to B2(AoA1) = B2(AoA2) + G(AoA2) - G(AoA1), it reaches the UE RF/BB again with unchanged power of B2(AoA2) + G(AoA2). Since for L1-RSRP measurement only the power at UE RF/BB is relevant, such a change would be transparent for the UE measurement.

As such we reduce the scenario from 2AoAs to only 1AoA and still keep L1-RSRP measurement as well as CDL characteristics of the signal for Beam 2 unchanged.

However, the above approach:

- Requires the knowledge of G(AoA1) and G(AoA2) i.e. of the UE Rx Beam/Antenna pattern.

- Does not consider the time aspect of the UE Rx Beam sweeping i.e. the fact that the UE might need time to sweep the UE Rx Beam from AoA1 to AoA2.

Regarding the UE Rx Beam/Antenna pattern, there are 3 alternatives:

- RAN4 defines a Reference UE Rx Beam/Antenna pattern which is used for AI/ML BM requirement definition and testing.

- UE vendor provides the UE Rx Beam/Antenna pattern to the TE during testing.

- TE measures the UE Rx Beam/Antenna pattern before starting the test (the probably well known Radiated Two Stage Method RTS)

Observation 10: With the knowledge of the UE Rx Beam/Antenna pattern, the CDL channel can be emulated in a practical and feasible single-AoA test setup, keeping the L1-RSRP and the CDL characteristics of the signal unchanged.

Observation 11: The UE Rx Beam/Antenna pattern for single-AoA CDL, can be defined as a reference for requirement definition and testing, or can be provided or measured before the testing.

With respect to the power dynamic range, routing all the beams to a single AoA and RF, is not expected to introduce limitation in the number of beams. It is true that in a multi-AoA system, each AoA has a dedicated RF with an independent adjustable power. However, RAN5 has already estimated the available SNR / power dynamic range for FR2 Demod testing in TS 38.521-4 [8], Table 7.1.1_1-1 (PC3) as shown in Figure 8. This is roughly 30dB and applies to all AoAs in the same way (in terms of absolute powers), since it is an effect of the maximum output power of the test system as well as the UE noise (either with or without added AWGN). As such, single AoA CDL emulation, does not impact the number of Tx beams.

Figure 8 – FR2 Power dynamic range (from TS 38.521-4 [8])

Observation 12: The supported power dynamic range for FR2 testing is circa 30dB. This applies the same to all AoAs in terms of absolute powers. Thus, moving from a multi-AoA to a single-AoA CDL emulation, does not impact the number of Tx Beams supported.

Regarding the time aspect of the UE Rx Beam sweeping, we think that it can be tested in a separate test optimized only for this purpose, and with less complexity in terms of number of beams and channel model.

In a real-life scenario, probably all the aspects described in Figure 6 are considered in the AI/ML learning model. However, for conformance testing we need to define requirement testable with feasible test systems, which as shown above would not be the case for a spatial CDL emulation.

In addition, conformance tests need to be debugged with confidence. Including all the aspects in the same test leads rather to an End-to-End test, the fail of which, cannot reveal much about the main aspect of interest (in our case AI/ML BM).

Also, we need to point out that a UE Rx Beam sweeping test is not yet specified for RRM conformance. Thus, it would be very challenging and probably counterproductive to introduce it for the first time within the complex package of AI/ML BM testing.

Observation 13: The single-AoA emulation of the CDL channel with the knowledge of the UE Rx Beam/Antenna pattern, does not consider the time aspect of the UE Rx Beam sweeping.

Observation 14: The time aspect of the UE Rx Beam sweeping can be tested in a separate multi-AoA test, optimized only for this purpose and with less complexity in terms of number of beams and channel model (e.g. TDL).

Based on all observations above about CDL, we propose:

Proposal 1: Segregate CDL testing for AI/ML BM in a first single-AoA test with CDL emulation with known UE Rx Beam/Antenna pattern (referenced / provided / measured) and necessary number of beams, and in a second multi-AoA test for UE Rx Beam sweeping (channel TBD).

## TDL Channel model

From the WF in [4] following analysis is required with regard of TDL channel model:

- For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes.

  - Set A and B should be generated in the chamber for ground-truth extraction.

    - For single AoA, FSS how to derive Set A

TDL channels for FR2 are already widely used in the specifications. For Demod and RRM test cases they are specified in TS 38.101-4 [9], Annex B.2.1.2 with Delay spread up to 60ns (TDLC60) and power dynamic up to 30dB roughly (TDLD10) as shown in Figure 9. These channels have been successfully implemented by the TE and are being used for testing, including 2AoA scenarios.

Figure 9 – Some FR2 TDL channels Demod / RRM testing (TS 38.101-4 [9])

In case that for AI/ML BM testing further Further extensions in terms of taps and dynamic are needed, this should probably not introduce feasibility issues for TE.

Observation 15: TDL channels are currently supported by TE and used for testing, including multi-AoA scenarios. Further extensions in terms of taps and dynamic should probably not introduce feasibility issues for TE.

The TDL channel consist of separate signal taps, with defined delays, attenuation and doppler. The taps do not contain explicit spatial information. Thus, in general the emulation of a TDL channel is done from a single direction / AoA / probe.

Observation 16: The emulation of a TDL channel is in general done from a single AoA as the signal taps do not contain explicit spatial information.

In case the signals are provided from different directions / AoAs, they go through independent TDL channels, since as mentioned above, due to the missing spatial information, there is no rule how different taps of the same TDL channel can be mapped to different directions / AoAs.

Regarding the content of the signals from different directions / AoA, so far this has been different i.e. Beam 1 / Cell 1 from AoA1 and Beam 2 / Cell 2 from AoA2 and so on, implying another BS or a main reflection from AoA2. Basically, this can be considered as a multi TRP scenario.

Observation 17: Currently, in multi-AoA tests with TDL, signals from different AoAs, go through separate TDL channels and have different content i.e. AoA(n) contains Beam(n) or Cell(n), which represent a multi-TRP scenario.

Scenarios where the same signal content (e.g. same Beam) goes through different AoAs with independent TDL channels, have not been considered so far. From a TE perspective this would be possible with feasible effort. Thus, if it would be sufficient to train and test the AI/ML BM model of the UE, it could be considered for AI/ML testing.

Observation 18: Multi-AoA scenarios, with same signal content per AoA i.e. single TRP (still with separate TDL per AoA), should be feasible from TE perspective and could be investigated for relevance for UE AI/ML BM requirements and testing.

Regarding the number of Tx beams (SSB / CSI-RS), since these are generated in the baseband, there are basically no restrictions for the emulation in the TE. Restrictions might apply mostly due to overall processing resources required, in terms of combination of high number of beams with high number of TDL channels with high number of probes with larger bandwidths and so on. But these details need to be evaluated at the level of test case parameters’ definition.

Observation 19: Number of in baseband generated BS Tx beams (SSB, CSI-RS) is in general not limited, but might be restricted by the overall resources required in the test case (combination of numbed of beams, TDL channels, AoAs, BWs). This can be assessed during the test case parameters' definition phase.

In terms of power dynamic range, as mentioned also in section 2.1.3, for FR2 it has been defined to be circa 30dB (TS 38.521-4 [8], Table 7.1.1_1-1), which applies the same to all AoAs (Observation 12).

For a given power dynamic range, the number of beams from the same AoA will be restricted only if the beams need to be differentiated by a power offset. The number of beams will then depend on the granularity of the required offset. For example, if the beams need to be differentiated by 2dB power offset, a 30dB power dynamic range will accommodate 15 beams.

Observation 20: Number of BS Tx beams (SSB, CSI-RS) from the same AoA, is restricted by the power dynamic range of 30dB, only if the beams are differentiated by power-offsets, and depends on the granularity of the power offsets.

Based on all observations above about TDL, we propose:

Proposal 2: Consider TDL testing for AI/ML BM, at least for multi-AoA UE Rx Beam sweeping.

## Baseline system

The current eIFF system described in TS 38.508-1 [10], Annex B.2.6, and used broadly for multi-AoA RRM testing supports:

- 2AoA with angular offsets [30°, 60°, 90°, 120°, 150°]

- Best far field conditions with lowest measurement uncertainty

- Big quiet zone of at least 30cm for all AoAs

This system covers all the RF / Demod / RRM tests up to Rel-17 (using AoA Setups 1-4, as per TS 38.133 [11], Annex A.3.15).

Observation 21: Current eIFF system with 2AoA covers all RF / Demod / RRM tests up to Rel-17.

However commercial implementations involve more than 2x CATRs in the eIFF system which allow up to 4AoA as depicted in Figure 10, with layout optimized for angular offsets as described above. This has enabled the usage of this system also for Rel-18 features Multi Rx and Enhanced MIMO (using AoA Setups 5-7, as per TS 38.133 [11], Annex A.3.15), which require 3AoA.

Figure 10 – Commercial implementation of eIFF system (up to 4 CATRs /AoAs)

As such, a single system type is currently covering all FR2 RF / Demod / RRM tests, and still has the capacity for 4AoA testing, under certain restrictions of the AoA layout.

Observation 22: Current eIFF commercial implementations cover all RF / Demod / RRM tests including Rel-18 and have probably capacity for up to 4AoAs, under certain AoA layout restrictions.

This is not a coincidence, but also intended. Especially for FR2, the systems are big, complex and expensive. Specifying dedicated system for specific features, would make full RRM coverage unfeasible for many stakeholders in the FR2 ecosystem, just because multiple FR2 test systems would be commercially not possible.

From a technical perspective, it is important to have a single system for RF / Demod and RRM, since RRM tests cases refer mostly to RF / Demod side conditions (EIS / SNR / Fading profiles), which need to be fulfilled in the RRM testing system.

Observation 23: The definition and availability of a single FR2 system with full coverage across RF / Demod / RRM features is essential when considering the complexity and commercial feasibility of the system. Also, it is the only way to fully ensure the inter-dependencies and cross-references between requirements in RF / Demod / RRM (side conditions).

In addition, the eIFF system is the only type of system, which has a physical quiet zone of at least 30cm for all AoAs, which allows black box testing (no knowledge of antenna configurations) with best MUs for devices fitting in the quiet zone. The MUs are crucial for the L1-RSRP measurement accuracy, whether the black box testing is crucial for a UE Rx Beam sweeping test, where the UE might utilize antennas spread all over the device formfactor.

Observation 24: eIFF systems are the only system type supporting multi AoA black box testing with best MU within a quiet zone of at least 30 cm for all AoAs. This is crucial for L1-RSRP measurement accuracy and UE Rx beam sweeping (precise UE antenna configuration for sweeping is unknown).

The proposed testing for CDL in section 2.1.3 as well as any kind of TDL testing are supported in the above system type. Thus, we propose to take this system as a baseline for AI/ML BM testing. As a positive side effect, this would ensure continuity in the FR2 testing ecosystem.

Observation 25: eIFF systems, including potential upgrades, support single-AoA CDL testing with measurement of UE Rx Beam/Antenna pattern, as well as multi-AoA TDL for UE Rx Beam sweeping testing, as described above. Thus, they are suitable to be baseline system for AI/ML BM testing.

Based on all observations above about eIFF, we propose:

Proposal 3: Define eIFF system, including potential upgrades, as the baseline system for AI/ML BM testing, since it is feasible and supports single-AoA CDL and multi-AoA TDL, as well as all RF / Demod / RRM legacy tests (one system fits all, side condition fulfilled), allows black box testing for beam sweeping (big quiet zone) and precise UE measurements (low MUs).

# Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: Following the MIMO-OTA / 3D MPAC methodology, to emulate 8 BS Tx Beams with a CDL-C UMi channel requires a test system with ≥100 probes and ≥200 RF channels which is unfeasible.

Observation 2: For a feasible spatial (multi-AoA) emulation of the CDL channel, simplifications in terms of number of Tx beams, their spatial distribution and channel model need to be investigated.

Observation 3:  AI/ML Beam prediction performance (e.g. Top 3, Top 1 Set AI beams) improves as the ratio Set B / Set A increases. For a given Set A, we need to emulate more Set B beams for better performance, especially for the prediction of the best (Top 1) beam.

Observation 4:  The spatial distribution of Set B beams affects the prediction performance. For a better prediction of multiple beams (e.g. Top 3) a wider spatial distribution of Set B beams is required.

Observation 5:  Emulating power-offset Set B beams, does probably not bring much value for prediction performance and might have unexpected impact on a CDL trained AI/ML model, since such beams are co-located and introduce inappropriate spatial information.

Observation 6:  Set A beams (up to 128) needs to be generated in the chamber for ground truth extraction.

Observation 7:  The overall number of beams and their spatial distribution needs to remain high for a good AI/ML prediction performance and ground truth extraction.

Observation 8: Since for a meaningful AI/ML performance the number of beams and their spatial distribution is required to be high (>8, ≤128), even significant simplification of the channel model (currently 8 beams requires 100 probes), would rather not downscale the number of probes to a feasible spatial (multi-AoA) emulation of CDL in TE.

Observation 9: Main reason for spatial (multi-AoA) emulation of CDL channel model is probably the inclusion of UE Rx Beam/Antenna pattern and UE Rx Beam sweeping performance in the AI/ML BM requirements.

Observation 10: With the knowledge of the UE Rx Beam/Antenna pattern, the CDL channel can be emulated in a practical and feasible single-AoA test setup, keeping the L1-RSRP and the CDL characteristics of the signal unchanged.

Observation 11: The UE Rx Beam/Antenna pattern for single-AoA CDL, can be defined as a reference for requirement definition and testing, or can be provided or measured before the testing.

Observation 12: The supported power dynamic range for FR2 testing is circa 30dB. This applies the same to all AoAs in terms of absolute powers. Thus, moving from a multi-AoA to a single-AoA CDL emulation, does not impact the number of Tx Beams supported.

Observation 13: The single-AoA emulation of the CDL channel with the knowledge of the UE Rx Beam/Antenna pattern, does not consider the time aspect of the UE Rx Beam sweeping.

Observation 14: The time aspect of the UE Rx Beam sweeping can be tested in a separate multi-AoA test, optimized only for this purpose and with less complexity in terms of number of beams and channel model (e.g. TDL).

Observation 15: TDL channels are currently supported by TE and used for testing, including multi-AoA scenarios. Further extensions in terms of taps and dynamic should probably not introduce feasibility issues for TE.

Observation 16: The emulation of a TDL channel is in general done from a single AoA as the signal taps do not contain explicit spatial information.

Observation 17: Currently, in multi-AoA tests with TDL, signals from different AoAs, go through separate TDL channels and have different content i.e. AoA(n) contains Beam(n) or Cell(n), which represent a multi-TRP scenario.

Observation 18: Multi-AoA scenarios, with same signal content per AoA i.e. single TRP (still with separate TDL per AoA), should be feasible from TE perspective and could be investigated for relevance for UE AI/ML BM requirements and testing.

Observation 19: Number of in baseband generated BS Tx beams (SSB, CSI-RS) is in general not limited, but might be restricted by the overall resources required in the test case (combination of numbed of beams, TDL channels, AoAs, BWs). This can be assessed during the test case parameters’ definition phase.

Observation 20: Number of BS Tx beams (SSB, CSI-RS) from the same AoA, is restricted by the power dynamic range of 30dB, only if the beams are differentiated by power-offsets, and depends on the granularity of the power offsets.

Observation 21: Current eIFF system with 2AoA covers all RF / Demod / RRM tests up to Rel-17.

Observation 22: Current eIFF commercial implementations cover all RF / Demod / RRM tests including Rel-18 and have probably capacity for up to 4AoAs, under certain AoA layout restrictions.

Observation 23: The definition and availability of a single FR2 system with full coverage across RF / Demod / RRM features is essential when considering the complexity and commercial feasibility of the system. Also, it is the only way to fully ensure the inter-dependencies and cross-references between requirements in RF / Demod / RRM (side conditions).

Observation 24: eIFF systems are the only system type supporting multi AoA black box testing with best MU within a quiet zone of at least 30 cm for all AoAs. This is crucial for L1-RSRP measurement accuracy and UE Rx beam sweeping (precise UE antenna configuration for sweeping is unknown).

Observation 25: eIFF systems, including potential upgrades, support single-AoA CDL testing with measurement of UE Rx Beam/Antenna pattern, as well as multi-AoA TDL for UE Rx Beam sweeping testing, as described above. Thus, they are suitable to be baseline system for AI/ML BM testing.

Proposal 1: Segregate CDL testing for AI/ML BM in a first single-AoA test with CDL emulation with known UE Rx Beam/Antenna pattern (referenced / provided / measured) and necessary number of beams, and in a second multi-AoA test for UE Rx Beam sweeping (channel TBD).

Proposal 2: Consider TDL testing for AI/ML BM, at least for multi-AoA UE Rx Beam sweeping.

Proposal 3: Define eIFF system, including potential upgrades, as the baseline system for AI/ML BM testing, since it is feasible and supports single-AoA CDL and multi-AoA TDL, as well as all RF / Demod / RRM legacy tests (one system fits all, side condition fulfilled), allows black box testing for beam sweeping (big quiet zone) and precise UE measurements (low MUs).

# References

- R4-2409739, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2411587, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112, August 2024

- R4-2416391, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112bis, October 2024

- R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, 3GPP TSG RAN WG4 Meeting #112bis, October 2024

- R4-2417211, “Simulation assumptions for beam prediction”, Vivo, 3GPP TSG RAN WG4 Meeting #112bis, October 2024

- 3GPP TR 38.827 V16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”

- 3GPP TS 38.151 V18.2.0, “NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements”

- 3GPP TS 38.521-4 V18.4.0, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance requirements”

- 3GPP TS 38.101-4 V18.5.0, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”

- 3GPP TS 38.508-1 V18.4.0, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”

- 3GPP TS 38.133 V18.7.0, “NR; Requirements for support of radio resource management”