3GPP TSG-RAN WG4 Meeting #113							R4-2418995

Orlando, Florida, USA, 18th to 22nd  November 2024

Agenda Item:	7.8.2.1

Source:	Ericsson

Title:	On Co-existence simulation results for NR-NTN and NB-IoT-NTN HP UE

Document for:	Information

# 1	Introduction

In the last RAN plenary meeting RAN#104, there was an agreement for a new Rel-19 RAN4 led WI Enhanced requirements and test methodology for NR and IoT NTN [1]. The baseline assumptions agreed in RAN4#112bis meeting have been captured in WF [2].

This contribution provides co-existence simulation results for the Case 4 when TN UL is the victim.

# 2	Discussion

In the following sections, we present our simulation results based on the agreed simulation parameters in WF [2].

## 2.1	Case 4 – TN UL as victim and NR NTN/ NB-IoT NTN UL as aggressor

In this sub-section, we share the simulation results between NR-NTN/ NB-IoT NTN HPUE interfering TN UL with Power class 2,1.5 and 1. In this scenario, NTN HPUE is transmitting as aggressor and TN gNB receiving as victim. The two NTN scenarios considered are GEO and LEO1200.

The isolation distance is assumed according to TR 38.863 between NTN UEs and the edge of TN cluster. The isolation distance is 1500m (2*ISD) and the ISD for Urban Macro is 750m. NTN UEs are placed at the NTN cluster edge. This section is further divided based on the assumption of with/ without wrap around functionality in order to evaluate performance impact.

Some of the other assumptions are as following:

- Isolation distance is taken as 1.5 km

- Statistics are based on Option 2: TN sectors which have NTN UEs at their edges

- NR NTN/ IoT-NTN HPUE ratio is 100%

- NTN UE NF is 9 dB

- NB-IoT NTN UL UEs and SCS – 9 UEs for 15 kHz

#### 2.1.1	With wrap around

##### 2.1.1.1	NR NTN; TN BS type – AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 6 | 8 | 10 | 12 | 14 | 16 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average |  | 7.14 | 5.31 | 3.91 | 2.84 | 2.05 |  |
|  |  |  | 5%-tile |  | 7.39 | 5.56 | 2.96 | 2.08 | 0.77 |  |
|  | PC1.5 | Throughput Loss | Average |  | 10.83 | 8.24 | 6.17 | 4.56 | 3.33 |  |
|  |  |  | 5%-tile |  | 19.29 | 11.46 | 5.63 | 4.12 | 2.46 |  |
|  | PC1 | Throughput Loss | Average |  |  | 10.83 | 8.24 | 6.17 | 4.56 | 3.33 |
|  |  |  | 5%-tile |  |  | 19.29 | 11.46 | 5.63 | 4.12 | 2.46 |
| LEO1200 | PC2 | Throughput Loss | Average | 11.18 | 8.43 | 6.22 | 4.50 |  |  |  |
|  |  |  | 5%-tile | 12.66 | 8.52 | 3.59 | 1.92 |  |  |  |
|  | PC1.5 | Throughput Loss | Average |  | 12.78 | 9.73 | 7.26 | 5.30 | 3.78 |  |
|  |  |  | 5%-tile |  | 14.96 | 11.93 | 5.76 | 2.58 | 0.98 |  |
|  | PC1 | Throughput Loss | Average |  |  |  | 9.73 | 7.26 | 5.30 | 3.80 |
|  |  |  | 5%-tile |  |  |  | 11.93 | 5.76 | 2.58 | 0.98 |

With wrap around - For GEO in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 15dB from 26 dBm to 31 dBm UE transmission power.

With wrap around - For LEO1200 in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 15dB from 26 dBm to 31 dBm UE transmission power.

##### 2.1.1.2	NR NTN; TN BS type – Non AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 8.66 | 6.19 | 4.41 | 3.13 | 2.23 | 1.60 |  |
|  |  |  | 5%-tile | 9.53 | 5.94 | 4.58 | 3.69 | 2.47 | 1.79 |  |
|  | PC1.5 | Throughput Loss | Average | 14.00 | 10.19 | 7.33 | 5.23 | 3.71 | 2.64 |  |
|  |  |  | 5%-tile | 18.66 | 10.11 | 7.84 | 5.02 | 4.05 | 3.40 |  |
|  | PC1 | Throughput Loss | Average | 18.84 | 14.00 | 10.19 | 7.33 | 5.23 | 3.71 |  |
|  |  |  | 5%-tile | 24.87 | 18.66 | 10.11 | 7.84 | 5.02 | 4.05 |  |
| LEO1200 | PC2 | Throughput Loss | Average | 8.47 | 5.91 | 4.05 | 2.74 |  |  |  |
|  |  |  | 5%-tile | 7.61 | 6.51 | 1.64 | 1.04 |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 13.98 | 10.06 | 7.09 | 4.90 | 3.34 | 2.24 |  |
|  |  |  | 5%-tile | 11.02 | 8.45 | 7.08 | 4.10 | 1.30 | 0.82 |  |
|  | PC1 | Throughput Loss | Average |  |  | 10.06 | 7.09 | 4.90 | 3.34 | 2.24 |
|  |  |  | 5%-tile |  |  | 8.43 | 7.08 | 4.10 | 1.30 | 0.82 |

With wrap around - For GEO in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 8dB from 26 dBm to 31 dBm UE transmission power.

With wrap around - For LEO1200 in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 7dB from 26 dBm to 31 dBm UE transmission power.

##### 2.1.1.3	NB-IoT NTN; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | SNR Loss | Average | 2.37 | 1.81 | 1.35 | 0.99 | 0.71 | 0.50 | 0.35 | 0.24 |
|  |  |  | 5%-tile | 0.55 | 0.33 | 0.18 | 0.11 | 0.09 | 0.08 | 0.07 | 0.07 |
|  | PC1.5 | SNR Loss | Average | 3.46 | 2.70 | 2.08 | 1.56 | 1.16 | 0.84 | 0.60 | 0.42 |
|  |  |  | 5%-tile | 1.21 | 0.80 | 0.44 | 0.22 | 0.16 | 0.10 | 0.09 | 0.08 |
|  | PC1 | SNR Loss | Average | 4.35 | 3.46 | 2.70 | 2.08 | 1.56 | 1.16 | 0.84 | 0.60 |
|  |  |  | 5%-tile | 2.30 | 1.21 | 0.80 | 0.44 | 0.22 | 0.16 | 0.10 | 0.09 |
| LEO1200 | PC2 | SNR Loss | Average | 2.84 | 2.19 | 1.67 | 1.25 | 0.93 | 0.68 |  |  |
|  |  |  | 5%-tile | 1.25 | 0.77 | 0.50 | 0.27 | 0.22 | 0.20 |  |  |
|  | PC1.5 | SNR Loss | Average | 4.06 | 3.21 | 2.50 | 1.92 | 1.45 | 1.08 | 0.79 |  |
|  |  |  | 5%-tile | 2.17 | 1.50 | 0.94 | 0.59 | 0.40 | 0.23 | 0.20 |  |
|  | PC1 | SNR Loss | Average | 5.05 | 4.06 | 3.21 | 2.50 | 1.92 | 1.45 | 1.08 | 0.79 |
|  |  |  | 5%-tile | 3.31 | 2.17 | 1.50 | 0.94 | 0.59 | 0.40 | 0.23 | 0.20 |

With wrap around - For GEO in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss increases to 3 dB.

With wrap around - For LEO1200 in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss increases to 4 dB.

#### 2.1.2	Without wrap around

##### 2.1.2.1	NR NTN; TN BS type – AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 10,0 | 8,0 | 7,0 | 5,0 | 4,0 |  |  |
|  |  |  | 5%-tile | 11,0 | 8,0 | 3,0 | 2,0 | 2,0 |  |  |
|  | PC1.5 | Throughput Loss | Average | 14,00 | 12,00 | 9,00 | 8,00 | 6,00 | 4,00 |  |
|  |  |  | 5%-tile | 27,00 | 16,00 | 11,00 | 6,00 | 2,00 | 2,00 |  |
|  | PC1 | Throughput Loss | Average | 17,0 | 14,0 | 12,0 | 9,0 | 8,0 | 6,0 |  |
|  |  |  | 5%-tile | 41,0 | 27,0 | 16,0 | 11,0 | 6,0 | 2,0 |  |
| LEO1200 | PC2 | Throughput Loss | Average |  |  | 6,00 | 5,00 | 4,00 | 3,00 | 2,00 |
|  |  |  | 5%-tile |  |  | 9,0 | 4,0 | 1,0 | 0,0 | 0,0 |
|  | PC1.5 | Throughput Loss | Average |  |  | 9,00 | 7,00 | 6,00 | 4,00 | 3,00 |
|  |  |  | 5%-tile |  |  | 13,00 | 10,00 | 7,00 | 3,00 |  |
|  | PC1 | Throughput Loss | Average |  |  |  |  |  | 6,0 | 4,0 |
|  |  |  | 5%-tile |  |  |  |  |  | 7,0 | 3,0 |

Without wrap around - For GEO in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 9dB from 26 dBm to 31 dBm UE transmission power.

Without wrap around - For LEO1200 in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 11dB from 26 dBm to 31 dBm UE transmission power.

##### 2.1.2.2	NR NTN; TN BS type – Non AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 4,0 | 3,0 | 3,0 | 2,0 | 1,0 | 1,0 |
|  |  |  | 5%-tile | 4,0 | 2,0 | 1,0 | 1,0 | 1,0 | 1,0 |
|  | PC1.5 | Throughput Loss | Average | 8,00 | 6,00 | 4,00 | 3,00 | 2,00 | 1,00 |
|  |  |  | 5%-tile | 14,00 | 6,00 | 2,00 | 1,00 | 1,00 | 1,00 |
|  | PC1 | Throughput Loss | Average | 11,0 | 8,0 | 6,0 | 4,0 | 3,0 | 2,0 |
|  |  |  | 5%-tile | 18,0 | 14,0 | 6,0 | 2,0 | 1,0 | 1,0 |
| LEO1200 | PC2 | Throughput Loss | Average | 4.4 | 2,0 | 2,0 |  |  |  |
|  |  |  | 5%-tile | 3.1 | 3.1 | 1.1 |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 7,0 | 5,0 | 4,0 | 3,0 |  |  |
|  |  |  | 5%-tile | 6,0 | 4,0 | 3,0 | 2,0 |  |  |
|  | PC1 | Throughput Loss | Average | 10,00 | 7,00 | 5,00 | 4,00 |  |  |
|  |  |  | 5%-tile | 11,00 | 6,00 | 4,00 | 3,00 |  |  |

Without wrap around - For GEO in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 5dB from 26 dBm to 31 dBm UE transmission power.

Without wrap around - For LEO1200 in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 3dB from 26 dBm to 31 dBm UE transmission power.

##### 2.1.2.3	NB-IoT NTN; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | SNR Loss | Average | 1.5 | 1.2 | 0.9 |  |  |  |
|  |  |  | 5%-tile | 0.6 | 0.3 | 0.2 |  |  |  |
|  | PC1.5 | SNR Loss | Average | 2.2 | 1.7 | 1.3 | 1.04 | 0.8 |  |
|  |  |  | 5%-tile | 1.1 | 0.7 | 0.4 | 0.3 | 0.2 |  |
|  | PC1 | SNR Loss | Average | 2.7 | 2.2 | 1.7 | 1.4 | 1.05 | 0.8 |
|  |  |  | 5%-tile | 1.3 | 1.1 | 0.70 | 0.4 | 0.3 | 0.2 |
| LEO1200 | PC2 | SNR Loss | Average | 1.7 | 1.3 | 1,0 | 0.7 |  |  |
|  |  |  | 5%-tile | 0.4 | 0.3 | 0.2 | 0.1 |  |  |
|  | PC1.5 | SNR Loss | Average | 2.4 | 1.9 | 1.5 | 1.1 | 0.9 | 0.6 |
|  |  |  | 5%-tile | 0.8 | 0.5 | 0.3 | 0.2 | 0.1 | 0.1 |
|  | PC1 | SNR Loss | Average | 3,0 | 2.4 | 1.9 | 1.5 | 1.1 | 0.9 |
|  |  |  | 5%-tile | 1.5 | 0.8 | 0.5 | 0.3 | 0.2 | 0.1 |

Without wrap around - For GEO in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss increases to 4 dB.

Without wrap around - For LEO1200 in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss increases to 2 dB.

## 2.2	Summary of ACIR requirements for PC2, PC 1.5 and PC 1

Table 2.2-1: Summary of ACIR requirements for PC2, 1.5 and 1

| ACIR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Case 4 |  | PC 2 | PC 1.5 | PC 1 |
| With wrap-around | GEO | NR NTN and TN BS with AAS | 11 | 13 | 15 |
|  |  | NR NTN and TN BS with Non-AAS | 3 | 6 | 8 |
|  |  | NB-IoT NTN | NA | 1 | 3 |
|  | LEO1200 | NR NTN and TN BS with AAS | 9 | 13 | 15 |
|  |  | NR NTN and TN BS with Non-AAS | 3 | 5 | 7 |
|  |  | NB-IoT NTN | 1 | 4 | 5 |
| Without wrap-around | GEO | NR NTN and TN BS with AAS | 3 | 7 | 9 |
|  |  | NR NTN and TN BS with Non-AAS | NA | 2 | 4 |
|  |  | NB-IoT NTN | NA | 2 | 4 |
|  | LEO1200 | NR NTN and TN BS with AAS | 6 | 9 | 11 |
|  |  | NR NTN and TN BS with Non-AAS | NA | 2 | 4 |
|  |  | NB-IoT NTN | NA | 0 | 2 |

The ACIR requirements are lower when isolation distance is considered.

# Conclusion

In the previous sections we made the following observations:

Observation 1	With wrap around - For GEO in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 15dB from 26 dBm to 31 dBm UE transmission power.

Observation 2	With wrap around - For LEO1200 in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 15dB from 26 dBm to 31 dBm UE transmission power.

Observation 3	With wrap around - For GEO in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 8dB from 26 dBm to 31 dBm UE transmission power.

Observation 4	With wrap around - For LEO1200 in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 7dB from 26 dBm to 31 dBm UE transmission power.

Observation 5	With wrap around - For GEO in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss increases to 3 dB.

Observation 6	With wrap around - For LEO1200 in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss increases to 4 dB.

Observation 7	Without wrap around - For GEO in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 9dB from 26 dBm to 31 dBm UE transmission power.

Observation 8	Without wrap around - For LEO1200 in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 11dB from 26 dBm to 31 dBm UE transmission power.

Observation 9	Without wrap around - For GEO in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 5dB from 26 dBm to 31 dBm UE transmission power.

Observation 10	Without wrap around - For LEO1200 in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 3dB from 26 dBm to 31 dBm UE transmission power.

Observation 11	Without wrap around - For GEO in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss increases to 4 dB.

Observation 12	Without wrap around - For LEO1200 in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss increases to 2 dB.

Observation 13	The ACIR requirements are lower when isolation distance is considered.

# References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei)

- R4-2417122,”WF on co-existence study and HPUE for NR-NTN”, 3GPP RAN4#112bis, October 2024