# 3GPP TSG-RAN WG4 Meeting #113	R4-2419003

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	4.5.1

Source: 	Ericsson

Title:	Discussion on multiple BWP switching with dormancy SCells

Document for:	Discussion

# Introduction

In RAN2 meetings, RAN2 agreed to introduce a new capability to handle the multiple BWP switching between non-dormant and dormant SCells.

RAN2 discussed the two options and concluded to down-select approach 2 because Approach 1 is a non-backward compatible change from RAN2 perspective. RAN2 introduced a field bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 corresponding to RAN4 feature 38-9, and a field scellDormancyWithinActiveTime-DCI-0-3-And-1-3-r18 corresponding to its prerequisite feature (RAN1 feature 49-9). It is up to RAN4 whether the UE shall report the same value between RAN4 features 6-3 and 38-9.

In last RAN4 meeting, companies discussed whether to define new requirement for the multiple BWP switching between non-dormant and dormant SCells.

- Requirement

The change on the requirements for simultaneous DCI based BWP switch delay on multiple CCs with new capability are as follows.

UE shall finish BWP switch within the time duration TMultipleBWPswitchDelay + Y, which is defined as: TMultipleBWPswitchDelay = TBWPswitchDelay + D*(N-1) Where: - TBWPswitchDelay is the BWP switching delay on single CC defined in Table 8.6.2-1 depending on UE capability bwp-SwitchingDelay [2]. TBWPswitchDelay shall be based on the smallest SCS among SCS of all involved CCs before and after BWP switch. If the BWP switch on multiple CCs results in the change of the SCS on any CC among involved CCs, TBWPswitchDelay shall be based on the smallest SCS among all SCS values of all involved CCs. - D is the incremental delay for each additional CC involved in simultaneous BWP switch and depends on UE capability bwp-SwitchingMultiCCs-r16 [TS 38.306, 14] for switching between non-dormant BWPs, and bwp-SwitchingMultiDormancyCCs-r16 or bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 for switching between non-dormant and dormant BWPs. - For UE which is capable of per-FR gap, and no BWP switch involves SCS change, N is the number of CCs in same FR; For UE which is not capable of per-FR gap, or the BWP switches on any CC involves SCS changing, N is the number of CCs undergoing simultaneous BWP switch. - Y=0, if the serving cell where UE receives DCI for BWP switch is same as the serving cell on which BWP switch occurs for each involved serving cell. Y equals to the length of one slot at smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change, - if the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs for any involved serving cell. If both scheduling cell and scheduled cell are in FR2-2, Y shall follow the SCS of 120 KHz.

The overall BWP switch delay is based BWP switching delay on single CC plus incremental delay D for each additional CC. There are two type of BWP switch delay, i.e., switching between non-dormant BWPs and switching between dormant and non-dormant BWP.

Companies proposed to update the requirement to consider the hybrid non-dormant and dormant SCells BWP switching. However, the legacy incremental delay requirement for dormancy SCell is agreed as follow. During the legacy discussion, companies believe the BWP switching requirement shall be reused for dormancy SCell.

In RAN1#98bis, the following agreements are made regarding SCell dormancy.

Agreements: - When the UE is outside Active Time, for the L1 based mechanism for transitioning from ’dormancy-like’ to ’non-dormancy like’ behaviour on activated Scells, an explicit information field for the UE is introduced to the PDCCH WUS - The explicit information field is configurable within a range of 0 to X1 bits - X1 <<15 - FFS whether to use BWP framework for transitioning from dormany to non-dormancy - FFS the case of ’non-dormancy-like’ to ’dormancy like’ transition - When the UE is in the Active Time, for the L1 based mechanism for transitioning between ’dormancy-like’ and ’non-dormancy like’ behaviour on activated Scells, an explicit information field is newly introduced to at least DCI formats 0-1 and 1-1 for the primary cell - The explicit information field is configurable within a range of 0 to X2 bits - X2 <<15 - FFS whether to use BWP framework for transitioning from dormany to non-dormancy or vice versa - FFS The DCI formats may or may not schedule data (if supported w/o data, the value of X2 can be separately discussed) - FFS DCI formats 0-0 and 1-0 - FFS the impact of CIF if any Agreements: - For the L1 based Scell dormancy indication sent on primary cell within active time - UE is configured with at least two BWPs for an Scell - The explicit information field in DCI indicates switching to/from dormant BWP configured for the Scell - FFS definition of dormant BWP - FFS whether or not to the same BWP switching delay to the non-dormant to dormant transition delay - Note: Rel15 behavior for case when 1BWP is configured for the Scell (i.e., no dormancy indication for that Scell) Agreements: - For the L1 based Scell dormancy indication sent on primary cell outside active time in WUS PDCCH - The explicit information field in DCI is a bitmap with up to X1 bits and 1 bit per group of configured Scells - Each Scell group can have one/multiple Scells and up to X1 Scell groups are configured via RRC. - The Scell group configuration is independent from the Scell group configuration for dormancy indication within active time (if supported) - X1 = [5] - Note: X1 is upper bound. - Note: Number of bits used for explicit information field in WUS PDCCH is based on configuration

Based on the RAN1 agreements, it can be seen that the switching between dormancy and non-dormancy behaviour on an SCell is based on BWP, i.e. a dormant BWP is defined for the SCell, and

- When network wants the UE to perform dormancy behaviour on the SCell, it will indicate the UE to switch to the dormant BWP (from a normal BWP)

- When network wants the UE to perform non-dormancy behaviour on the SCell, it will indicate the UE to switch from the dormant BWP (to a normal BWP)

Thus, in RAN4 93 meeting, RAN4 achieved the following agreement.

R4-2012278 Agreement: - Delay requirements for switching of multiple SCells between dormancy and non-dormancy shall be based on corresponding delay requirements for switching of multiple SCells between non-dormant BWPs, i.e., TMultipleBWPswitchDelay or TMultipleBWPswitchDelay+Z for simultaneous BWP switching case. - Incremental value D is reused from BWP switching on multiple CCs

We don’t see any necessary to introduce a new requirement when RAN2 introduces such hybrid non-dormant and dormant SCells’ BWP switching  since the fundamental mechanism is BWP switching.

Proposal 1: There is no spec impact when the new multiple BWP switching with both non-dormant and dormant SCells capability is introduced.

- Summary

In this contribution, we discussed the multiple BWP switching with non-dormant and dormant SCells. Based on the discussions, we have made following proposals and observations:

Proposal 1: There is no spec impact when the new multiple BWP switching with both non-dormant and dormant SCells capability is introduced.

- References

[1]. R4-2012278, “Way Forward on SCell Dormancy”, Ericsson