# 3GPP TSG-RAN WG4 Meeting #113	R4-2419005

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.23.4.2

Source: 	Ericsson

Title:	Discussion on LP-WUR RRM requirement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 had discussed the LP-WUS/WUR WI and made the following agreement [1]. In this contribution, we continue the discussion on the RRM impact of LP-WUR operation.

- LP-WUR sceanrios

Based on LP-WUS WID, different scenarios to support the RRM relaxation shall be defined as below:

- MR further relaxation with LR

- MR offloading by LR

In last RAN4 meetings, RAN4 agreed the case #1, #3, but the other MR relaxation case is FFS.

RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

When both NW and UE supports LP-WUS feature, different scenarios are valid by the NW using different thresholds and the UE will need to adapt its measurement behaviour to the respective zones. Adapting herein comprises turning ON/OFF the MR or LR receiver chains.

In legacy release, when neighbour cell measurement relaxation criteria were introduced, UE may perform neighbour cell measurements from normal measurement, relaxed measurement till stop the measurement. Now in Rel-19 LP-WUS, different serving cell measurement relaxation scenarios will be introduced. As shown in the figure below, different thresholds for neighbour cell measurement and serving cell measurement can be introduced for different serving cell and neighbour cell combinations.

Figure 1 Serving and neighbour cell measurement scenarios

RRM relaxation in scenario 3

In RAN2 #127 meeting, RAN2 has made the working assumption that both MR and LR can be used to evaluate the entry condition.

RAN2#127, LP-WUS entry/exit condition - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

When UE meets the LP-WUS entry condition, UE will enter the LP-WUS state. However, at the same time, UE may still need to evaluate the neighour cell if the serving cell quality is not better than the threshold SIntraSearch, SnonIntraSearch. RAN4 agreed to introduce the scenario 3 which means UE had to perform neighbour cell measurement when UE has already entered the LP-WUS. The issue will be which kind of RRM relaxation can be applied in scenario 3.

In our understanding, the Sintrasearch can be configured quite conservatively, i.e. basically requiring the UE to measure intra-frequency neighbouring cell in the complete cell. This is typically motivated in urban environments where cell coverage does not follow a perfect circle but is rather irregular, and the UE may detect a better cell suddenly around the corner. We expect no offloading scenario 4 in this typical deployment. In other words, the power saving from LR mainly come from the RRM relaxation(scenario 3).

Observation 1: NW may always configure the neighbour cell measurements in irregular deployment.

Example of coverage in urban environment:

On the one hand, UE is expected to achieve better power saving gain with a larger relaxation factor in some good deployments. On the other hand, to avoid mobility issue, it is preferred a quite conservative relaxation factor to neighbour cell measurements. At the same time, considering the LR measurement instead of MR, UE can still have chance to achieve better power saving to relax the serving cell measurements.

We understand that MR serving cell measurement can be further relaxed since UE will use LR to monitor the serving cell quality, but for the neighbour cell relaxation, it should be more careful. In legacy R16, neighbour cell measurement relaxation has already been introduced based on cell center and low mobility criteria in RRM relaxation WI. In that scenario, UE uses the serving cell measurements to monitor the cell quality and mobility. However, in Rel-19, UE will only use LR to monitor the serving cell quality. We have concerns to further relax the neighour cell measurement due to LR introduced since LR is used for power saving other than enhancing the measurement capability.

Observation 2: Both NW and UE need to tradeoff between the power saving and mobility.

In our understanding, NW has good knowledge of cell deployment and which kind of RRM relaxation can be achieved. NW can configure different relaxing factors in different deployment. Thus, we propose to introduce the NW controlled measurement relaxation. The relaxation factor should at least include 3 which means the same neighbour cell relaxation criteria as legacy.

Proposal 1: RAN4 to introduce NW controlled serving/neighbour cell measurement relaxation. The relaxation factor can be indicated by NW from one of the following values: 3, 8, 16 etc.

Scenario 2

There are still FFS for some scenarios. When UE enters LP-WUS mode, UE will use LR to monitor the serving cell quality without MR to achieve the power saving gain. Due to UE does not to combine the measurement results across MR and LR, it seems scenario 2 is not useful.

RAN4 #112 Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

Proposal 2: RAN4 not to introduce scenario 2 which is both MR and LR for serving cell measurement but neighbour cell measurement is OFF.

Higher priority for scenario 4

Another remaing issue related to scenario is about higher priority relaxation. In our understanding, legacy RRM relaxation has already relaxed the higher priority measurement. We don’t think any further higher priority relaxation criteria shall be introduced.

Issue 1-1-11: Considerations on higher priority frequency layer - Proposals - P1: higher priority layer neighbor cell measurement could also be OFF for case #1. (Apple vivo) - P2: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor. (ZTE)

Proposal 3: High priority measurement relaxation criteria shall be the same as Rel-16 RRM relaxarion.

Purely MR relaxation

In last meeting, we compromised to introduce the scenario 3, but we still have concern on whether NW will configure LR based serving cell measurement with neighbour cell measurement at the same time. Thus, to allow the flexibility of network’s configuration, we propose that NW can disable such scenario 3’s relaxation scenario based on the configured threshold. How to disable such scenario depends on RAN2’s criteria design.

Proposal 4: Whether LR based serving cell RRM relaxation case is valid is fully up to NW’s configuration, such as NW can enable/disable any RRM relaxation scenario based on the configured thresholds.

In last meetings, some companies also proposed to further discuss purely MR relaxation. In our understanding, this Rel-19 WI is for power saving due to LP-WUR. RAN4 should not discuss any purely MR relaxation without LP-WUR measurement.

Proposal 5: MR RRM further relaxation without LR measurement is out of scope in Rel-19.

- Measurement requirement of LP-WUR and MR relaxation

In legacy NR, UE will use the SSB to measure and evaluate the serving cell’s quality with the cell selection S criterion. If UE has evaluated that serving cell’s quality doesn’t fulfil the cell selection S criterion in N consecutive DRX cycles, UE will initiate all neighbour cells’ measurement.

When LP-WUS operation introduced, UE will use LP-WUS to monitor serving cell’s quality. The conditions to enter/exit the LP-WUS monitoring shall be defined, e.g. based on serving cell RSRP or/and RSRQ.

To avoid negative impact on paging or cell re-selection when using WUR, the following entry/exit pricinples should be followed.

- The entry should be made carefully and delay is not critical.

- The exit should be made fast because LP-SS measurement and mobility detection may delay for the exit condition to be fulfilled.

In last RAN2 meeting, RAN2 also made the WS on the criteria for entry/exit criteria.

RAN2#127, LP-WUS entry/exit condition - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. - The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

LR entry/exit criteria and requirement

From our understanding, such working assumption is aligned with the pricinples we mentioned to guarantee UE to enter the LP-WUR mode carefully. Bothe MR and LR RRM measurement criteria need to be evaluated. Furthermore, NW may only configure the MR evaluation criteria(threshold), in this case, UE only need to evaluate the MR RRM quality.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement Recommendations: To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself. Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation Recommendations: To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself. Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring Recommendations: To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself. Issue 1-1-8: On jointly consideration on issue 1-1-5, 1-1-6 and 1-1-7 Issue 1-1-9: LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS - Proposals - P1: LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring (Apple) - Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement - Alt 2 (preferred): MR is ON with RRM measurement on serving cell and neighbour cell (if any). But whether or not LP-WUR is ON for RRM measurement is up to UE implementation. - P2: RAN4 needs consider latest RAN2 agreement when determines LP-WUR status before entering/after exiting LP-WUS monitoring. (xiaomi vivo) - P3: Postpone the discussion on LR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring. RAN4 to informs RAN2 to revisit the working assumption on entry condition of LP-WUS monitoring: LR based thresholds are not considered for LP-WUS monitoring entry condition. (Huawei) Recommendations: Need to consider latest RAN2 agreement at RAN2 127 meeting when determines LP-WUR status before entering/after exiting LP-WUS monitoring

In other words, before UE entering LR-based RRM relaxation or offloading, LR may already wake up during a transition period once NW configures both LR and MR criteria. We don’t think there is any issue from UE side to wake up LR a little early and we clearly see the benefits to avoid the ping-pang between the LR and MR. Thus, we propose to confirm RAN2’s WS.

Proposal 6: RAN4 to confirm RAN2’s WS as follow.

When UE checks the LR entry threshold,

- UE is required to evaluate both MR and LR RRM quality if both configured from NW side;

- UE is optionally to only evaluate MR quality if only MR evaluation criteria is configured from NW side.

When UE checks the LR exit threshold, UE is only required to evaluate the LR quality.

Proposal 7: When NW configures both MR and LR criteria, UE needs to wake up LP-WUR earlier before LP-WUS monitoring.

Proposal 8: RAN4 to define the serving cell measurement based on LP-WUR to check whether UE entry/exit the LP-WUR.

LR based serving cell evaluation

In legacy IDLE mode requirement, no serving cell evaluation relaxed criteria is introduced. It means serving cell evaluation is always on from UE side. In Rel-19, when UE enters the LR mode, we believe the serving cell evaluation should also be kept. Thus, RAN4 needs to define the LP-WUR based serving cell evaluation requirement.

Issue 1-2-3: Possible cell selection evaluation requirements for fully offloading case based on LP-WUR Recommendations: Companies are encouraged to bring solutions/views on this issue.

Proposal 9: RAN4 to define LP-WUR based serving cell evaluation requirement.

Measurement periodicity for LP-WUR based on SSB

In last meeting, RAN4 spent a long time to discuss the periodicity of SSB based LP-WUR.

Issue X: Measurement delay requirements for LP-WUR based on SSB in IDLE/Inactive mode shall be based on Option 1: Based on DRX configured for MR (vivo) Option 2: follow the same periodicity of LP-SS if LP-SS periodicity is configured (CMCC, Xiaomi, vivo, HW, Apple, QC, MTK) Option 3: A fix value (like 320ms), FFS on the value (Apple - if LP-SS periodicity is not configured, HW- if LP-SS periodicity is not configured, LGE - if LP-SS periodicity is not configured, ) Option 4: max of option 1 and 2 (QC) Option 5: max of option 2 and 3 (OPPO) Other options are not precluded.

From our understanding, NW will transmit SSB with 20ms which is different with LP-SS. To guarantee UE can quickly quit the LR mode, SSB measurement periodicity shall be a little faster since LP-WUR is a low power receiver. Thus, we propose to use 320ms to simplify the design.

Proposal 10: RAN4 to define the measurement periodicity for SSB based LP-WUR with 320ms regardless of LP-SS periodicity is configured.

Sampling rate

Some companies also propose to discuss the measurement filtering. In legacy, the DRX case, the filtering interval is at least DRX/2. We think it’s reasonable to define the measurement filtering based on the periodicity of LP-SS. In LP-WUR, the typical LP-SS peridocity is 320ms. To achieve the comparable performance, we propose to also use 320/2ms as measurement interval.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least eDRX_IDLE cycle/2, if the UE is configured with eDRX cycle ≤ 10.24s; otherwise DRX cycle/2.

Proposal 11: RAN4 to define the minimum measurement interval in LP-WUR as 160ms for both LP-SS and SSB.

- Others

Accuracy

In RAN4 #111 meeting, RAN4 spent a long time to discuss how to define the accuracy requirement. Two possible way forwards were proposed.

- Define a new LP-SS accuracy directly in performance section

- Follow the legacy IDLE mode requirements to reflect accuracy in reselection margin

Issue 1-3-3: Accuracy for relaxed MR measurement - Proposals - P1: The legacy accuracy for relaxed MR measurement should be reused if Case#2 and/or Case#3 introduced.

We prefer the second option to reuse the same approach where the legacy SSB-based measurements in IDLE mode are used indirectly as reselection margin other than creating a new LP-SS performance part for IDLE/INACTIVE mode. The NR SSB based performance requirements were also defined because the SSB performance can be verified in CONNECTED mode, but LP-SS is only used in IDLE mode which cannot be verified due to no measurement reporting.

Proposal 12: No dedicated accuracy requirement in the performance section is defined for LP-SS based measurement and reflect the accuracy performance as a margin in the IDLE/INACTIVE mode core requirement.

Carrier frequency

In last meeting, RAN4 discussed the LS from RAN1 and made the conclusion that RAN4 assumed LR and MR are operating on the same carrier frequency as baseline, but FFS MR and LR on different carrier frequencies. In our understanding, RAN4 doesn’t need to spend time to discuss such flexible scenario since LP-WUS is introduced in the first time.

Issue 1-1-8: LP-WUR operating carrier frequency Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. Issue for further discussion based on contribution driven: - For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.

Proposal 13: The RAN1 LS should be handled in RF session. RAN4 to insist on MR and LR operating on the same carrier frequency to define the requirment.

There is another open issue related to the carrier frequency for LP-WUR. To ensure NW scheduling flexibility, it should be possible to have flexible placement of LP-SS resources in frequency and time. Considering in IDLE mode, MR should use to monitor the CD-SSB in sync raster and LP-SS is unnecessary to also occupy the sync raster frequency. Since both SSB and LP-SS are broadcasted periodically, it is beneficial if the LP-SS and SSB can be set with FDM such that it allows the gNB to go into micro sleep mode to help with NW energy saving. Thus, we propose RAN4 to at least consider LP-SS and SSB are FDM scenario and FFS other scenario.

Issue 1-2-4: LP-WUR operating carrier frequency - Proposals P1: RAN4 assumed LR and MR are operating on the same carrier frequency as baseline, and FFS on the other scenarios if RAN1 has agreement.

Proposal 14: For LP-SS and SSB FDMed multiplex, RAN4 to wait RAN1’s progress.

- RRC-CONNECTED mode

RAN4 shall focus on developing the RRM requirements for IDLE and INACTIVE mode during this meeting since not many agreements have been reached in other WGs for CONNECTED mode. Discussions on CONNECTED mode shall therefore be postponed until any agreements are achived in other WGs regarding, and if so RAN4 can check the RRM requirements for those at later stage.

Proposal 15: Discussions on LP-WUS CONNECTED mode requirements are postoponed until more progress is achieved in other WGs.

- Summary

In this contribution, we have initially discussed the LP-WUS operation. Based on the discussions, we have made following proposals and observations:

Observation 1: NW may always configure the neighbour cell measurements in irregular deployment.

Observation 2: Both NW and UE need to tradeoff between the power saving and mobility.

Proposal 1: RAN4 to introduce NW controlled serving/neighbour cell measurement relaxation. The relaxation factor can be indicated by NW from one of the following values: 3, 8, 16 etc.

Proposal 2: RAN4 not to introduce scenario 2 which is both MR and LR for serving cell measurement but neighbour cell measurement is OFF.

Proposal 3: High priority measurement relaxation criteria shall be the same as Rel-16 RRM relaxarion.

Proposal 4: Whether LR based serving cell RRM relaxation case is valid is fully up to NW’s configuration, such as NW can enable/disable any RRM relaxation scenario based on the configured thresholds.

Proposal 5: MR RRM further relaxation without LR measurement is out of scope in Rel-19.

Proposal 6: RAN4 to confirm RAN2’s WS as follow.

When UE checks the LR entry threshold,

- UE is required to evaluate both MR and LR RRM quality if both configured from NW side;

- UE is optionally to only evaluate MR quality if only MR evaluation criteria is configured from NW side.

When UE checks the LR exit threshold, UE is only required to evaluate the LR quality.

Proposal 7: When NW configures both MR and LR criteria, UE needs to wake up LP-WUR earlier before LP-WUS monitoring.

Proposal 8: RAN4 to define the serving cell measurement based on LP-WUR to check whether UE entry/exit the LP-WUR.

Proposal 9: RAN4 to define LP-WUR based serving cell evaluation requirement.

Proposal 10: RAN4 to define the measurement periodicity for SSB based LP-WUR with 320ms regardless of LP-SS periodicity is configured.

Proposal 11: RAN4 to define the minimum measurement filtering in LP-WUR as 160ms for both LP-SS and SSB.

Proposal 12: No dedicated accuracy requirement in the performance section is defined for LP-SS based measurement and reflect the accuracy performance as a margin in the IDLE/INACTIVE mode core requirement.

Proposal 13: The RAN1 LS should be handled in RF session. RAN4 to insist on MR and LR operating on the same carrier frequency to define the requirment.

Proposal 14: For LP-SS and SSB FDMed multiplex, RAN4 to wait RAN1’s progress.

Proposal 15: Discussions on LP-WUS CONNECTED mode requirements are postoponed until more progress is achieved in other WGs.

- References

[1]. R4-2416861, “WF for RRM requirements for LP-WUS/WUR”, vivo