3GPP TSG-RAN WG4 Meeting #113	R4-2419015
Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.8.2.2.1

Source: 	OPPO

Title: 	Discussion on Tx requirements for NR-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Tx RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved in WF [1]:

Agreements: Issue 2-1: General for Tx requirements - Follow the previous agreements. Issue 3-1-2: Consideration of band(s) Agreement: - Use 2GHz for the co-existence study - WI can be completed when the band specific requirements for one pair of {n256,256}and {n255, 255} are completed in this WI - Specify the band specific requirements for other potential NTN bands in the separate spectrum WI Issue 3-1-4: Consideration of regulation(s) - Agreement - Applicable regulations should be taken into account for requirements discussions. - If any identified corrections to be made for PC3, those should be considered in another work item, it is not in the current scope of this WI.. - And the following regulatory information contributed to this meeting are noted and can be further considered in proper places per the guidance from RAN4 leadership: - For bands 255/253, HPUE UL/TX requirements should not be driven by TN-NTN coexistence. (Inmarsat/Viasat) - ETSI EN 301 681 can be used to inform system design features for the protection of the radio astronomy bands at 1660-1660.5MHz and 1668-1670MHz. (Inmarsat/Viasat) - RAN4 to consider introducing protection limits below B255 for Iridium protection as specified in EN 301 681 and ITU-R M.1480 and evaluate how to meet these requirements with higher output power. (Qualcomm) - Based on the study outcome, necessary corrections should be made also for PC3 in 36.102 and 38.101-5 (Qualcomm) - In case ETSI TC SES identifies later that some requirements are not required for market access in Europe, those requirements should be removed from 3GPP. (Qualcomm) Issue 2-2: UE architecture/type considerations for NR-NTN PC2 UE - Follow previous agreement Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - .... Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE - Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions: - FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified - FFS whether PC1.5 handheld UE can be specified or not Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE - Confirm feasibility of PC1 1Tx for non-handheld UE. - For PC1, at least 1Tx to be considered for requirements. - Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE. - Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE. Issue 2-5: ACLR - Co-existence study results and conclusion is an important factor for deriving out-band emission requirements - Check the derived ACLR for A-MPR and additional spurious emission against regulatory information. Issue 2-8: SEM - To re-use existing SEM requirements from PC3 to other PCs as starting point. Issue 2-9: MOP tolerance - Re-use NR TN TS 38.101-1 nominal MOP and +2/[-3] tolerance for PC2 for NR-NTN Non-Redcap HPUE Open issues: Issue 2-6: MPR - Further discuss with ACLR candidate values, following agreement and above options. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified.. Issue 2-7: A-MPR - Further discuss with following agreement and above options. Issue 3-3-3: A-MPR - Agreement - FFS A-MPR requirements. Following potential factors to be further discussed: - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations - A-MPR simulation assumption including ACLR from co-existence study - Applicable bands - Regional requirements for A-MPR - Other factors.

This contribution further discussed the UE Tx RF requirements for NR NTN HPUE.

- Discussion

In the R-19, the purpose of NTN HPUE is to introduce PC1, PC1.5 and PC2 for NTN bands and

- PC2 will apply to both handheld and non-handheld UE

- PC1 and PC1.5 are prioritized for non-handheld UE

In previous meeting, the architecture of NTN HPUE was discussed, and it agreed for PC2 (NR NTN), 1 Tx and 2 Tx for PC2 NR NTN UE are to be considered for requirements and the architecture for other power class need further study.

PC1.5 for TN was introduced only for TDD currently and the typical architecture is 2Tx and 4Tx also is introduced. Although it doesn’t need duplexer for TDD bands, it still needs 2Tx for PC1.5 for handheld UE. That is, there is no single PA whose output power are enough to support 29dBm considering the front-end loss. For FDD bands, the insertion loss of duplexer also needs consider into the PA’s output power, it’s hard to achieve by 1PA. In additional, there is no suitable duplexer for PC 1.5 handheld UE. Therefore, it’s impossible for 1 Tx to support PC1.5 handheld UE.

Although it’s feasible for 2Tx to support PC1.5 NTN handheld UE based on PC2 with 1Tx. Here we can review the MPR for PC1.5 with 2Tx and PC2 with 1Tx in TN for 16QAM as shown in Table 2-1, and compare the maximum output power between PC1.5 and PC2 considering MPR as Table 2-2:

Table 2-1: Maximum power reduction (MPR) for TN 16QAM

|  | Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| PC1.5 with 2Tx | CP-OFDM | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
| PC2 with 1Tx | CP-OFDM | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |

Table 2-2: Compared the maximum output power considering the MPR

|  | Modulation |  | Maximum output power(dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| PC1.5 with 2Tx | CP-OFDM | 16 QAM | 22.5 | 24.5 | 26.5 |
| PC2 with 1Tx | CP-OFDM | 16 QAM | 22.5 | 23 | 24 |
| Delta between PC1.5 and PC2 | CP-OFDM | 16 QAM | 0 | 1.5 | 2.5 |

Based on Table 2-2, PC1.5 with 2Tx can get the 2.5dB diversity gain compared with PC2 only under the inner RB allocations. Although, the above analysis is based on MPR of TN, the ACLR of NTN for PC2 and PC1.5 is not significantly different from that of TN from the coexistence simulation results in last meeting. The MPR for NTN will very similar with the value of TN based on the same SEM and similar ACLR, it’s unnecessary for 2 Tx to support PC1.5 handheld UE.

Proposal 1: PC1.5 only apply to non-handheld UE in Rel-19.

About PC1, it applied to non-handheld UE only, the main limitation is still because of the higher power PA and duplexer.

Proposal 2: PC1 only apply to non-handheld UE in Rel-19.

- Conclusion

In this paper we give further discussion on the TX requirement for HPUE NTN. The proposals are as following:

Proposal 1: PC1.5 only apply to non-handheld UE in Rel-19.

Proposal 2: PC1 only apply to non-handheld UE in Rel-19.

- Reference

[1] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung, RAN4#112bis 2024-10