3GPP TSG-RAN WG4 Meeting #113	R4-2419017
Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.8.2.3.1

Source: 	OPPO

Title: 	Discussion on Tx requirements for IoT-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Tx RF requirements with IoT NTN operating in IoT-NTN bands were discussed and some agreements are approved in WF [1]:

Issue 1-1-1: Whether Iot NTN can be differentiated by handheld and non-handheld. - Iot NTN can be differentiated by handheld and non-handheld Issue 1-1-2: How many Tx number should be considered for Iot-NTN (in the specification). - Focus on 1Tx for PC2 [and PC1], further study [2Tx for PC1] - FFS on the applicability for handheld for PC1. - Continue work on 1Tx PC1 requirements. Issue 1-1-3: For NB-Iot based Iot-NTN, whether PC1 is feasible for handheld device. - Continue discussion in next meeting. Issue 1-1-4: For half duplex eMTC based Iot NTN, whether PC1 is feasible for handheld device. - FFS. Issue 1-1-5: For full duplex eMTC based Iot NTN, whether PC1 is feasible for handheld device. - FFS. Issue 1-1-6: For Iot-NTN, whether PC1 is feasible for non-handheld device. - For Iot-NTN, PC1 is feasible for non-handheld device. Issue 1-2-1: Regulatory requirements study. - FFS. Issue 1-2-2: ACLR - Encourage more input next meeting. Issue 1-2-3: SEM - Encourage more input next meeting. Issue 1-2-4: MPR - Encourage more input next meeting. Issue 1-2-5: MOP tolerance - FFS.

This contribution further discussed the UE Tx RF requirements for IoT NTN HPUE.

- Discussion

In the R-19, the purpose of IoT NTN HPUE is to introduce PC1 and PC2 for IoT NTN bands and

- PC2 will apply to both handheld and non-handheld UE

- PC1 are prioritized for non-handheld UE

In last meeting, some companies proposed that it can reuse the GSM PA for PC1 handheld IoT NTN UE [2]. Although the insertion loss of the duplexer in half-duplex mode does not need to be considered, the front-end filter still needs to be considered, it’s hard to get the 31dBm for PC1 handheld UE with 1Tx.

Proposal 1: Only PC2 with 1Tx apply to both of handheld and non-handheld IoT NTN UE.

Proposal 2: PC1 with 1Tx only apply to non-handheld IoT NTN UE.

The SEM of PC3 IoT NTN including MTC and NB1/NB2 reused the correspondingly requirements of MTC and NB1/NB2 in LTE. MTC in LTE reused the general 1.4MHz SEM requirements, and all power classes including PC1, PC2 and PC3 for general LTE UE use the same SEM requirements like NR.  Therefore, IoT NTN HPUE can re-use existing SEM requirements from PC3 of IoT NTN UE as starting point.

Proposal 3: IoT NTN HPUE can re-use existing SEM requirements from PC3 of IoT NTN UE for M1 and NB1/NB2 separately.

Proposal 4: ACLR should be defined based on the co-existence study results, then the MPR can be evaluated.

Proposal 5: Use the nominal MOP and +2/[-3] tolerance for PC1 and PC2 for IoT-NTN HPUE.

- Conclusion

In this paper we give further discussion on the TX requirement for IoT NTN HPUE. The proposals are as following:

Proposal 1: Only PC2 with 1Tx apply to both of handheld and non-handheld IoT NTN UE.

Proposal 2: PC1 with 1Tx only apply to non-handheld IoT NTN UE.

Proposal 3: IoT NTN HPUE can re-use existing SEM requirements from PC3 of IoT NTN UE for M1 and NB1/NB2 separately.

Proposal 4: ACLR should be defined based on the co-existence study results, then the MPR can be evaluated.

Proposal 5: Use the nominal MOP and +2/[-3] tolerance for PC1 and PC2 for IoT-NTN HPUE.

- Reference

[1] R4-2417205, WF on HPUE for IoT NTN, vivo, RAN4#112bis 2024-10.

[2] R4-2415790, Feasibility study and analysis for Iot NTN HPUE Tx Requirements, vivo, RAN4#112bis 2024-10.