3GPP TSG-RAN WG4 Meeting #113	R4-2419019
Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.26.2.1

Source: 	OPPO

Title: 	Discussion on RedCap UE RF requirements with NR NTN operating in FR1-NTN bands

Document for:	Discussion

- Introduction

In previous RAN4 meeting, the RedCap UE RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved in WF [1, 2, 3]:

Agreements: Issue 1-1: PC3 output power for HD-FDD (e)RedCap - Agreement: Nominal power for PC3 is 23 dBm. Issue 1-2: HD-FDD refsens for 2 Rx - Agreement: Agree n256 Refsens as below. Operating Band; SCS (kHz); 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm). 15; -100.0; -96.8; -95.0; -93.8. n256; 30; -97.1; -95.1; -94.0. 60; -97.5; -95.4; -94.2. Issue 1-4: Specification updates - Agreement: - The maximum transmission bandwidth for NTN eRedCap with bandwidth reduction should be 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths. - REFSENS of NTN eRedCap with bandwidth reduction for wider channel BW in FR1-NTN bands, it should be defined as below: - The 5MHz REFSENS requirements are applicable for all the channel bandwidths including 5MHz, 10MHz, 15MHz, 20MHz channel bandwidth. - 25 contiguous RB placed in middle of channel BW both in UL and DL for both HD-FDD and FD-FDD, where both UL and DL allocations are following the 5MHz NTN RedCap REFSENS configurations and located at the center of the channel BW. - UL RB size follow the 5MHz UL configuration table - Reuse the same scaling factor of NTN (e)RedCap without bandwidth reduction for REFSENS for NTN eRedCap with bandwidth reduction with 2Rx HD-FDD, 1Rx HD-FDD and 1Rx FD-FDD. - The bandwidth reduction in TS 38.101-1 5.3I applies also for NTN (e)RedCap Issue 1-3: Simultaneous operation with GNSS - Open items: - Strive towards common solution for (e)RedCap NR NTN and non-RedCap NR NTN - Consider asking RAN1/2 to ensure IDC framework allows appropriate configurations to ensure GNSS location acquisition and NR NTN can operate in parallel in bands with UL close to GNSS frequencies - Encourage further evaluations in RAN4 including but not limited to - Whether potential new solution(s) apply only from rel-19 or are early implementable - Whether UE speed needs to be consider

This contribution further discussed the UE RF requirements for RedCap NTN.

- Discussion

In the last meeting, for HD-FDD REFSENS for 2 Rx, 0.5 dB tightening was agreed for n256, however, the tightening amount for n254 and n255 are still under discussion.

For below 6GHz, the baseline REFSENS level was calculated by the equation below:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

We can get the values of baseline REFSENs with 2Rx for different channel bandwidth, here just list some channel bandwidth in Table 2-1:

Table 2-1 baseline REFSENs with 2Rx

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| Baseline | 15 | -100 | -96.8 | -95 | -93.8 |

And we can further compare the REFSENS of FDD band n1 with that of TDD band n39 as shown in Table 2-2:

Table 2-2 REFSENs with 2Rx for n1 and n39

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n1 | 15 | -100 | -96.8 | -95 | -93.8 |
| N39 | 15 | -100 | -96.8 | -95 | -93.8 |

We can see the REFSENS for both of FDD band n1 and TDD band n39 equal to the baseline REFSENS, since there is no duplexer for TDD bans, it demonstrates that the baseline REFSENs didn’t consider any impact from Tx noise due to the limitation of duplexer isolation for FDD bands.

Therefore, the REFSENs of FDD bands operating in HD-FDD for 2Tx should not be lower than the baseline REFSENs.

Proposal 1: The REFSENS of FDD bands operating in HD-FDD for 2Tx should not be lower than the baseline REFSENS calculated by the equation.

Proposal 2: n254 and n255 can adopt the same REFSENS values with n256 operating in HD-FDD for 2Tx as below Table 2-3.

Table 2-3 HD-FDD REFSENS for 2 Rx

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n256 n255 n254 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |

- Conclusion

This paper discussed the HD-FDD REFSENS for 2 Rx and proposed:

Proposal 1: The REFSENS of FDD bands operating in HD-FDD for 2Tx should not be lower than the baseline REFSENS calculated by the equation.

Proposal 2: n254 and n255 can adopt the same REFSENS values with n256 operating in HD-FDD for 2Tx as below Table 2-3.

Table 2-3 HD-FDD REFSENS for 2 Rx

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n256 n255 n254 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |

- Reference

[1] R4-2416586, Way Forward for [112bis][310] NR_NTN_Ph3_UE_RF, Qualcomm Inc., RAN4#112bis 2024-10