3GPP TSG-RAN WG4 Meeting #113                                         R4-2419027

Orlando, US, November 18 - 22, 2024

Source: 	Huawei, HiSilicon

Title: 	On MPR applicability for FR2 CA

Agenda Item:	7.1.1.2.3

Document for:	Approval

Introduction

During RAN4#112bis, MPR applicability for FR2 single carrier UL and intra-band UL CA with DL intra-band CA has been discussed. Remaining FFS points can be found in [1].

- CC activation-based MPR improvement - WF - Further discuss activated based MPR improvement. - FFS using common UE capability between configuration based and activation based, or another UE capability for activated-based improvements. - FFS MPR can be determined based on activated UL CCs BW instead of Cumulative aggregated channel bandwidth (CABW). (i.e., activation based also applies to UL CA case) - FFS limitation the enhancement to cases where the activated UL CCs form a contiguous block.

In this contribution, we would like to share our views regarding this topic.

Discussion

In last meeting, it was agreed to introduce configuration based MPR improvement, by which the MPR can be determined by UL aggregated channel bandwidth BWchannel_CA rather than cumulative aggregated channel BW. Regarding the activated-based mechanism, since implementation agnostic is emphasized and reflected in the WF, our understanding is the upcoming discussion for such mechanism should cover all possibilities. When we lay our eyes on the OOB requirements, it is clear that the requirements are based on UL aggregated channel bandwidth as defined in TS 38.101-2, which is configuration based.

##### Aggregated Channel Bandwidth: The RF bandwidth in which a UE is configured to transmit and receive multiple contiguously aggregated carriers. … ##### 6.5A.2.1.1 Spectrum emission mask for intra-band contiguous UL CA For intra-band contiguous UL carrier aggregation, the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the ± edge of the UL aggregated channel bandwidth (Table 5.3A.5-1). For any bandwidth class defined in Table 5.3A.5-1, the UE emission shall not exceed the levels specified in Table 6.5A.2.1-1. …… ##### 6.5A.2.3.1 Adjacent channel leakage ratio for CA intra-band contiguous UL CA In case the CA configuration consists of a single UL CC, the adjacent channel leakage ratio defined in subclause 6.5.2.3 applies. For intra-band contiguous UL carrier aggregation, the carrier aggregation NR adjacent channel leakage power ratio (CA NRACLR) is the ratio of the filtered mean power centred on the UL aggregated channel bandwidth to the filtered mean power centred on an adjacent UL aggregated channel bandwidth at spacing equal to the UL aggregated channel bandwidth. The assigned UL aggregated channel bandwidth power and adjacent UL aggregated channel bandwidth power are measured with rectangular filters with measurement bandwidths specified in Table 6.5A.2.3.1-1. If the measured adjacent channel power is greater than -35 dBm then the CA NRACLR shall be higher than the value specified in Table 6.5A.2.3.1-1.

Observation: The OOB requirements for FR2 UL intra-band contiguous CA are based on UL aggregated channel bandwidth, which is configuration based.

Thus we prefer to keep it as it is and leave some room for the UE that intends to achieve the improved MPR e.g. UE incapable of LO retuning.

Proposal 1: For Rel-19 FR2 intra-band contiguous UL CA MPR improvement (both configuration based and activation based), the OOB requirements are still based on UL aggregated channel bandwidth as defined in TS 38.101-2.

Another issue is about the UE capability design. We think a normal handling can be used here which is introducing two separate indicator under a unified capability for the whole feature to discriminate different UE would support configuration based or activation based mechanism:

[fr2-MPR-ImprovementUplinkDependent-r19]: {configuration-based, activation-based}

Proposal 2: Introduce separate indicator for configuration-based and activation-based under the UE capability for Rel-19 FR2 UL intra-band contiguous CA MPR improvement.

Conclusion

In this contribution we discussed on the remaining issue for Rel-19 MPR applicability for FR2 CA. We have the following observation and proposals:

Observation: The OOB requirements for FR2 UL intra-band contiguous CA are based on UL aggregated channel bandwidth, which is configuration based.

Proposal 1: For Rel-19 FR2 intra-band contiguous UL CA MPR improvement (both configuration based and activation based), the OOB requirements are still based on UL aggregated channel bandwidth as defined in TS 38.101-2.

Proposal 2: Introduce separate indicator for configuration-based and activation-based under the UE capability for Rel-19 FR2 UL intra-band contiguous CA MPR improvement.

- References

- R4-2417204, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #112bis, Hefei, China, October 14-18, 2024.