3GPP TSG-RAN WG4 Meeting #113                                R4-2419058

# Orlando, US, 18th – 22nd November, 2024

Title:	Draft LS on solutions to address doppler shift issue that RB falls into guard band

Response to:

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:	Peng, Zhang

zhangpeng169@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1	Overall description

In the RAN4#112 meeting, RAN4 identify one issue that there is a potential risk for NTN UE to comply with the out-of-band emission requirements when the edge RB falls into the guard band due to Doppler pre-compensation or Doppler shift in NGSO deployment. RAN4 agreed that it’s allowed for some UEs to report one capability whether UE is capable of supporting the deployment in NGSO scenario when the edge RB fall into the guard band.

Thus, RAN4 would like respectfully request RAN2 to design one per UE capability to indicate that NTN UE is supportive of being configured with the first RB and the last RB in the Maximum transmission bandwidth configuration or not when the edge RB fall into the guard band due to the doppler shift or doppler pre-compensation. If this capability is absent, it represents that NTN UE is supportive of being configured with the first RB and the last RB in the Maximum transmission bandwidth configuration even if the edge RB fall into the guard band due to the doppler shift or doppler pre-compensation.

- 2	Actions

To TSG RAN WG2

ACTION: 	RAN4 would like respectfully request RAN2 to design this UE capability and take the information above into account in future meetings.

- 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN4 Meeting#114 	      	17 – 21 Feb 2025 	Athens, GR

TSG-RAN4 Meeting#114bis 	     	07 – 11 Apr 2025 	TBD, China

- 4	References

[1] R4-2413524, Way Forward for Doppler pre-compensation issue, Huawei, HiSilicon