# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419063

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the NR NTN simultaneous operation with GNSS

Agenda Item:	7.26.2.1

Document for:	Approval

Introduction

In the last meeting, RAN4 reached the consensus on the way forward [1] for simultaneous operation with GNSS.

- Issue 1-3: Simultaneous operation with GNSS Open items: - Strive towards common solution for (e)RedCap NR NTN and non-RedCap NR NTN - Consider asking RAN1/2 to ensure IDC framework allows appropriate configurations to ensure GNSS location acquisition and NR NTN can operate in parallel in bands with UL close to GNSS frequencies - Encourage further evaluations in RAN4 including but not limited to - Whether potential new solution(s) apply only from rel-19 or are early implementable - Whether UE speed needs to be considered

In this paper, we’d like to further discuss the open issues for this topic.

Discussion on whether UE speed needs to be considered

In the last meeting, companies provided the discussion paper [2] to elaborate the simultaneous operation issue between NR NTN transmission and GNSS receiving. Due to insufficient GPS filter rejection and finite antenna isolation to n254 UL, the blocking signal seen at GPS LNA input could be as high as 5 dBm which would fail the GPS reception, irrespective of the n254 UL transmission bandwidth.

Observation 1: It would not be feasible to support simultaneous Rx/Tx between GPS receiver and L-band UL transmission at maximum output power.

In addition, the GNSS capabilities is mainly for UE to identify its own position such that it can pre-compensate for the delay of the transmission to the satellite to achieve synchronization before and during connection to an NTN cell. If UE position is fixed or UE is moving with small speed, we don’t need to consider some standardized solutions to address the simultaneous operation between NR NTN transmission in L-band and GNSS receiving. Because NTN UE don’t need to update the GNSS position very frequently.

Observation 2: If UE position is fixed or UE is moving with small speed, we don’t need to consider some standardized solutions to address the simultaneous operation between NR NTN transmission in L-band and GNSS receiving.

However, if UE speed is very high, e.g. 250km/h, 500km/h or 1000km/h, NTN UE has to update the GNSS position very frequently to achieve the latest UE position for performing doppler UL pre-compensation. Otherwise, NTN UE can’t meet current frequency error requirements.

For a real scenario illustrated below, it is assumed that a UE is moving with 250km/h velocity from Position 1 at T1 to Position 2 at T2 within approximate one minute.

figure 1 the real scenario illustration for 250km/h UE velocity

If UE GNSS position is updated larger than every one minute, the UE frequency pre-compensation error is about fd_error=(Vsat×sinθ-Vsat×cos90°)/λ=Vsat×sinarctan4.1600/c×f0=0.17ppm (Vsat = 7559m/s, LEO600)

Based on this model, we calculate the UE doppler pre-compensation error for different UE speed and different GNSS update periods below.

table 1 UE doppler pre-compensation error for different UE speeds and different GNSS update periods

| additional frequency error/ppm |  |  |  |  |  |  |  | GNSS update period / second |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10km/h UE | 20km/h UE | 80km/h UE | 100km/h UE | 120km/h UE | 250km/h UE | 500km/h UE | 1000km/h UE |  |
| 0.000 | 0.000 | 0.001 | 0.001 | 0.001 | 0.003 | 0.006 | 0.012 | 1 |
| 0.001 | 0.001 | 0.005 | 0.006 | 0.007 | 0.015 | 0.029 | 0.058 | 5 |
| 0.001 | 0.002 | 0.009 | 0.012 | 0.014 | 0.029 | 0.058 | 0.117 | 10 |
| 0.002 | 0.005 | 0.019 | 0.023 | 0.028 | 0.058 | 0.117 | 0.233 | 20 |
| 0.003 | 0.007 | 0.028 | 0.035 | 0.042 | 0.087 | 0.175 | 0.350 | 30 |
| 0.005 | 0.009 | 0.037 | 0.047 | 0.056 | 0.117 | 0.233 | 0.467 | 40 |
| 0.006 | 0.012 | 0.047 | 0.058 | 0.070 | 0.146 | 0.292 | 0.583 | 50 |
| 0.007 | 0.014 | 0.056 | 0.070 | 0.084 | 0.175 | 0.350 | 0.700 | 60 |
| 0.008 | 0.016 | 0.065 | 0.082 | 0.098 | 0.204 | 0.408 | 0.816 | 70 |
| 0.009 | 0.019 | 0.075 | 0.093 | 0.112 | 0.233 | 0.467 | 0.933 | 80 |
| 0.010 | 0.021 | 0.084 | 0.105 | 0.126 | 0.262 | 0.525 | 1.049 | 90 |

As we all know, the shorter GNSS update period UEs have to maintain, the higher power consumption UEs have to bear.

Observation 3: The shorter GNSS update period can help UE acquire the exact GNSS location timely. However, the shorter GNSS update period UEs have to maintain, the higher power consumption UEs have to bear.

In current TS 38.331, the in-device coexistence feature was introduced in Rel-18 with the following parameters.

However, it seems that the candidate numbers for cyclelength-r18 and activeDuration-r18 are not suitable for the simultaneous operation issue between NR NTN transmission in L-band and GNSS receiving especially considering the GNSS implementation. And GNSS implementation is related to the applicable UE speed scenarios. Different UE speed scenarios lead to the different requirements for GNSS update period.

Proposal 1: the applicable NR NTN UE speed range should be mentioned in the LS to RAN1/RAN2 on NR NTN and GNSS concurrency.

Summary

Observation 1: It would not be feasible to support simultaneous Rx/Tx between GPS receiver and L-band UL transmission at maximum output power.

Observation 2: If UE position is fixed or UE is moving with small speed, we don’t need to consider some standardized solutions to address the simultaneous operation between NR NTN transmission in L-band and GNSS receiving.

Observation 3: The shorter GNSS update period can help UE acquire the exact GNSS location timely. However, the shorter GNSS update period UEs have to maintain, the higher power consumption UEs have to bear.

Proposal 1: the applicable NR NTN UE speed range for this issue should be mentioned in the LS to RAN1/RAN2 on NR NTN and GNSS concurrency.

- References

[1] R4-2416586	Way Forward for [112bis][310] NR_NTN_Ph3_UE_RF Qualcomm

[2] R4-2415334	On NTN simultaneous operation with GNSS Apple