# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419066

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with UL MIMO

Agenda Item:	7.10.2.3

Document for:	Approval

Introduction

In the RAN4#112bis meeting, RAN4 reached some consensus on supporting UL MIMO feature in the approved WF [1]. There are two remaining issues.

Issue 3-1-1: ULFPTx mode for UL-MIMO WF: Consider whether and how to introduce this requirement. - Option 1: ULFPTx mode 1/2/Full in the following table can be supported depending on UE PA implementation - Table 2: Example of ATG ULFPTx (R-MOP = x dBm) ATG ULFPTx mode; PA1(dBm); PA2(dBm). Mode2; x; (x-3)~(x-1). Mode-Full; x; x. Mode1; (x-3)~(x-1); (x-3)~(x-1). - Option 2: Current ULFPTx mode requirements for legacy UE in TS38.101-1 can be reused for ATG UE, and there is no other specification affect. - Option 3: No need to extend ULFPTx mode to ATG UE. Issue 3-1-2: whether preclude the single antenna port related requirement WF: - Option 1: Yes. - Option 2: No.

In this meeting, we’d like to share our views on the remaining open issues.

Discussion

Since ATG UE can report the rated maximum output power, it’s totally different from handheld UE (power class report). Currently, the supported range of MOP for ATG UE is about 23~40dBm. Thus, the typical scenarios for handheld ULFPTx mode are 2*23dBm PA supporting 23dBm full power Tx per layer with 23dBm power class 3 report and 2*26dBm PA supporting 26dBm full power Tx per layer with 26dBm power class 2 report. However, for these scenarios, ATG UE can report higher MOP without full power mode as shown below.

| PA implementation | Handheld ULFPTx mode | ATG UE MOP report |
| --- | --- | --- |
| 2*23dBm PA | 23dBm full power Tx per layer with 23dBm power class 3 | 26dBm without ULFPTx mode |
| 2*26dBm PA | 26dBm full power Tx per layer with 26dBm power class 2 report | 29dBm without ULFPTx mode |

Since Handheld ULFPTx mode only discussed the limited kinds of RF architectures, it can’t cover all the implementations of ATG UE.

If we checked the proposed option 1, only two PAs are assumed for ATG UE implementation. However, generally one PA implementation can be assumed for passive antenna implementation. For the phased array antenna, there are a lot of same PA for ATG UE implementation. We don’t think two PA implementation for ATG UE is typical as handheld UE.

Observation 1: the two PAs implementation in the proposed option 1 is not typical for ATG UE.

For potential ATG UE supporting ULFPTx, It’s not expected to introduce additional spec impacts. Thus, as a compromise, it’s necessary to mention whether the ULFPTx mode is supported or not. It’s up to ATG UE implementation whether to leverage the existing ULFPTx mode or not since current Handheld ULFPTx mode is not always applicable to all of the ATG UE implementation.

Proposal 1: As a compromise, it’s necessary to mention whether the ULFPTx mode is supported or not. It’s up to ATG UE implementation whether to leverage the existing ULFPTx mode or not since current Handheld ULFPTx mode is not always applicable to all of the ATG UE implementation.

For the issue whether to preclude the single antenna port related requirement, we think Rel-18 ATG RF requirements have included the single antenna port related requirement. Thus, there is no need to further discuss this issue.

Proposal 2: There is no need to further discuss whether to preclude the single antenna port related requirement for ATG UE supporting UL MIMO as Rel-18 ATG RF requirements have included the single antenna port related requirement.

Summary

Observation 1: the two PAs implementation in the proposed option 1 is not typical for ATG UE.

Proposal 1: As a compromise, it’s necessary to mention whether the ULFPTx mode is supported or not. It’s up to ATG UE implementation whether to leverage the existing ULFPTx mode or not since current Handheld ULFPTx mode is not always applicable to all of the ATG UE implementation.

Proposal 2: There is no need to further discuss whether to preclude the single antenna port related requirement for ATG UE supporting UL MIMO as Rel-18 ATG RF requirements have included the single antenna port related requirement.

- References

[1] R4-2417121	WF on UE RF for ATG enhancement, CMCC