# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419071

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on coexistence study for NR NTN HPUE

Agenda Item:	7.8.2.1

Document for:	Approval

Introduction

In the RAN4#112bis meeting, some consensus were reached in the way forward [1]. And the option 2 was also considered in the coexistence study for NR NTN HPUE.

Issue 1-4 ~ Issue 1-6: Agreement: - Take option 2 in Issue 1-4 as an optional parameters, and the options in Issue 1-5 and Issue 1-6 as a package under this option. It’s shown in the table below. Scenarios; NR NTN for LEO; NR NTN for GEO; NB IoT NTN for LEO; NB IoT NTN for GEO. Baseline; Optional; Baseline; Baseline; Baseline; Optional. Orbits; LEO-1200 prioritized LEO-600 not precluded; LEO-1200 (Option 1 in Issue 1-5); GEO; LEO-1200 prioritized LEO-600 not precluded; GEO; GEO (Option 2 in Issue 1-5). UL BW; 2RB; 5MHz (Option 2 in Issue 1-4); 2RB; Single tone; Single tone; [0.18] MHz (Option 2 in Issue 1-4). SNR target (dB); 15; 3 (Option 2 in Issue 1-4); 15; R4-2217473; R4-2217473; 0 (Option 2 in Issue 1-4). ACLR model; TR 38.863; For the NTN UE1 which is closest to the victim TN channel, it’s assumed that flat ACLR1 will have impacts on all the Victim UE(s). For the other NTN UEs except for NTN UE1, it’s assumed that only flat ACLR2 will have impacts on all the Victim UE(s). (Option 1 in Issue 1-6); TR 38.863; R4-2217473; R4-2217473; R4-2217473.

In this meeting, we’d like to provide the corresponding results and further discuss the scenario for IoT NTN coexistence study.

Discussion on coexistence study for NR NTN

Simulation assumption

Referring to our discussion paper [2], we assume the following power control parameters.

| Parameters | NR NTN for LEO |  |  |
| --- | --- | --- | --- |
| Pmax Output power (Power Class) | 26dBm (PC2) | 29dBm (PC1.5) | 31dBm (PC1) |
| Rmin (dB) | -66 | -69 | -71 |
| SNRtarget (dB) | 3 | 3 | 3 |
| NFSAN (dB) | 4.3 | 4.3 | 4.3 |
| UL BW (MHz) | 5 | 5 | 5 |
| 10log10(kT) (dBm/Hz) | -174 | -174 | -174 |
| CLx-ile (dB) | 125.7 | 128.7 | 130.7 |
| γ | 1 | 1 | 1 |

For ACLR model, we consider a simplified ACLR model assuming four UL NTN aggressor UEs.

For the NTN UE1 which is closest to the victim TN channel, it’s assumed that flat ACLR1 will have impacts on all the Victim UE(s).

For the other NTN UEs except for NTN UE1, it’s assumed that only flat ACLR2 will have impacts on all the Victim UE(s).

We consider the scenario that NTN UE was dropped into the cluster of terrestrial network, the following red area can be considered with 1.5km above radius as the worst case. However, this scenario is much stricter than the real deployment.

Simulation results

For LEO1200-Urban Macro scenario, the results for 2GHz scenario 4 at PC2 are shown below.

| Sate UE ACLR1 [dB] | 20 | 22 | 24 | 26 | 28 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput baseline [Mbps] | 35.931 | 35.886 | 35.947 | 36.014 | 35.938 | 35.868 | 35.91 |
| Average throughput with adjacent interference [Mbps] | 35.642 | 35.66 | 35.736 | 35.85 | 35.789 | 35.758 | 35.82 |
| Average throughput loss | 0.80% | 0.63% | 0.59% | 0.46% | 0.41% | 0.31% | 0.25% |
| 5%-tile throughput baseline [Mbps] | 15.069 | 14.989 | 15.163 | 15.455 | 15.313 | 15.103 | 15.287 |
| 5%-tile throughput with adjacent interference [Mbps] | 13.97 | 14.102 | 14.428 | 14.827 | 14.765 | 14.707 | 14.911 |
| 5%-tile throughput loss | 7.29% | 5.92% | 4.85% | 4.06% | 3.58% | 2.62% | 2.46% |

For LEO1200-Urban Macro scenario, the results for 2GHz scenario 4 at PC1.5 are shown below.

| Sate UE ACLR1 [dB] | 20 | 22 | 24 | 26 | 28 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput baseline [Mbps] | 35.895 | 35.954 | 35.949 | 35.91 | 35.879 | 35.918 | 35.913 |
| Average throughput with adjacent interference [Mbps] | 35.537 | 35.64 | 35.673 | 35.71 | 35.704 | 35.773 | 35.782 |
| Average throughput loss | 1.00% | 0.87% | 0.77% | 0.56% | 0.49% | 0.40% | 0.36% |
| 5%-tile throughput baseline [Mbps] | 15.347 | 15.268 | 15.156 | 15.325 | 15.344 | 15.01 | 15.425 |
| 5%-tile throughput with adjacent interference [Mbps] | 13.869 | 14.043 | 14.095 | 14.54 | 14.714 | 14.469 | 14.915 |
| 5%-tile throughput loss | 9.63% | 8.02% | 7.00% | 5.12% | 4.11% | 3.60% | 3.31% |

For LEO1200-Urban Macro scenario, the results for 2GHz scenario 4 at PC1 are shown below.

| Sate UE ACLR1 [dB] | 20 | 22 | 24 | 26 | 28 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput baseline [Mbps] | 35.887 | 35.922 | 35.87 | 35.91 | 35.966 | 35.915 | 35.885 |
| Average throughput with adjacent interference [Mbps] | 35.496 | 35.571 | 35.583 | 35.681 | 35.758 | 35.731 | 35.741 |
| Average throughput loss | 1.09% | 0.98% | 0.80% | 0.64% | 0.58% | 0.51% | 0.40% |
| 5%-tile throughput baseline [Mbps] | 15.085 | 15.043 | 15.03 | 14.973 | 15.349 | 15.194 | 15.086 |
| 5%-tile throughput with adjacent interference [Mbps] | 13.604 | 13.597 | 13.847 | 14.173 | 14.618 | 14.561 | 14.565 |
| 5%-tile throughput loss | 9.82% | 9.61% | 7.87% | 5.34% | 4.76% | 4.17% | 3.45% |

Observation 1: Based on the simulation results for 2GHz scenario 4 (UL UEtoBS) LEO1200-Urban Macro, 24dB ACLR for PC2, 27dB ACLR for PC1.5 and 28dB ACLR for PC1 can be observed.

Discussion on coexistence study for NB-IoT NTN

Referring to the R18 WF for NB-IoT NTN coexistence, the following proposed frequency and bandwidth for coexistence study were captured.

The frequency and bandwidth are listed in table 6.1-3. Table 3.1-3. Proposed frequency and bandwidth for co-existence study Frequency; Bandwidth; Duplex mode; Frequency reuse factor. TN Rural; 2 GHz; 20MHz; FDD, TDD; 1. TN Urban macro; 2 GHz; 20MHz; FDD, TDD; 1. GEO; 2 GHz; 0.18MHz NB-IoT; FDD; 1. LEO; 2 GHz; 0.18MHz NB-IoT; FDD; 1.

However, in the end, RAN4 just reuse the TN IoT NTN ACLR requirements for coexistence with GSM and UTRA.

Table 6.5B.3.4-1: category NB1 and NB2 UE ACLR requirements GSMACLR; UTRAACLR. ACLR; 20 dB; 37 dB. Adjacent channel center frequency offset from category NB1 or NB2 Channel edge; ±200 kHz; ±2.5 MHz. Adjacent channel measurement bandwidth; 180 kHz; 3.84 MHz. Measurement filter; Rectangular; RRC-filter α=0.22. Category NB1 and NB2 channel measurement bandwidth; 180 kHz; 180 kHz. Category NB1 and NB2 channel Measurement filter; Rectangular; Rectangular.

It’s unclear whether we will follow the same approach for PC3 IoT NTN or we have to create new ACLR requirements for coexistence with 20MHz NR system.

Observation 2: It’s unclear whether we will follow the same approach for PC3 IoT NTN or we have to create new ACLR requirements for coexistence with 20MHz NR system.

Proposal 1: It’s better to clarify the relationship between the ACLR requirements for NB-IoT NTN UE and IoT NTN coexistence study since these two parts are not related during the Rel-18.

Summary

Observation 1: Based on the simulation results for 2GHz scenario 4 (UL UEtoBS) LEO1200-Urban Macro, 24dB ACLR for PC2, 27dB ACLR for PC1.5 and 28dB ACLR for PC1 can be observed.

Observation 2: It’s unclear whether we will follow the same approach for PC3 IoT NTN or we have to create new ACLR requirements for coexistence with 20MHz NR system.

Proposal 1: It’s better to clarify the relationship between the ACLR requirements for NB-IoT NTN UE and IoT NTN coexistence study since these two parts are not related during the Rel-18.

- References

[1] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung

[2] R4-2416055, Discussion on coexistence study for NR NTN HPUE, Huawei, HiSilicon