3GPP TSG-RAN WG4#113	R4-2419074

Orlando, US, November 18th – 22nd, 2024

Agenda item:	7.1.1.3.4

Source: 	Qualcomm France

Title: 	About SRS IL imbalance indication

Document for:	Approval

- Introduction

Considerations on 6Rx SRS IL imbalance are provided in this contribution.

- WF in RAN4#112-Bis

## Sub-topic 4-1:	General considerations for SRS IL imbalance issue

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19

Way Forward: RAN4 to further discuss the following options.

-	Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

-	Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 4-2-1.

Issue 4-1-2: Initial Considerations for SRS IL imbalance issue

Way forward: RAN4 to focus on the compromised solution proposals in Issue 4-2-1.

## Sub-topic 4-2:	SRS IL imbalance issue solutions

Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue

Way forward: Companies are encouraged to provide comments and indication of support for the solution proposals listed below by RAN4#113. Proponent companies (in parentheses) for the candidate solution and solution proposals below are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed. Other proposals including expected performance gains and draftCRs are not precluded.

- Candidate solutions

  - Option 1:

    - UE performs self-compensation of SRS IL up to its maximum Tx power capabilities

    - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g.

      - Per SRS resource power headroom reporting

      - Configured maximum output power per SRS resource reporting

      - SRS insertion loss value reporting (per SRS resource or per UE)

      - Other

    - Details of SRS IL self-compensation and assistance framework are FFS

  - Other options are not precluded

- Proposals

  - Proposal 1: Only introduce UE compensation and new accurate SRS power headroom reporting for non-CA/DC scenarios can be a compromised solution. Further study the granularity of new SRS power headroom report (China Telecom)

  - Proposal 2: IL imbalance reporting mechanism for SRS AS should include the configured maximum output power per SRS resource, the power headroom per SRS resource and ΔPPowerClass . The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission (Ericsson).

  - Proposal 3: Another way to resolve the SRS IL imbalance reporting issue would be to introduce two types of reporting for a UE: a “baseline” and an “advanced” reporting. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. The advanced reporting could in addition include the information on e.g. the configured maximum output power (Ericsson).

  - Proposal 4: Solution for IL imbalance issue as identified below. Reporting power threshold needs to be considered for static and dynamic reporting. The granularity of reporting could be per band (Spreadtrum).

    - If UE reports statically, report the actual SRS insertion loss with no UE self-compensation

    - If UE reports dynamically, UE report the difference value of each diversity branch output power to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time

  - Proposal 5: Introduce semi-static UE assistance or capability signalling on the information on the UE SRS IL (∆TRxSRS,UE) (Intel).

Adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

  - Proposal 6: Given that specification defines the Tx power should be equally distributed across SRS ports for each SRS transmission, per SRS resource IL reporting would be sufficient (Huawei, HiSilicon).

  - Proposal 7: If dynamic reporting can be considered for SRS IL reporting, network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency (Huawei, HiSilicon).

  - Proposal 8: To keep the current UE implementation untouched and avoid specification impact to most extent, following solution can be considered in Rel-19 (Huawei, Hi Silicon):

    - UE is allowed to indicate whether it enables self-compensation on the SRS IL once the network requests such information, which would benefit the network by adjusting expectation on the antenna switching SRS based PMI estimation.

  - Proposal 9: Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered. (ZTE, Sanechips)

  - Proposal 10: For Rel-19 UE, at least, optionally indicates self-compensation of SRS IL up to its maximum Tx power capabilities.

- Discussion

Proposals in previous meetings and consequently in WF are very diverse so instead of going them through one by one we begin with general comments.

In many proposals, it is said that the reporting should be per SRS resource. Basically the power difference between different SRS ports can be different (e.g 1dB between P0 and P1, 4dB between P0 and P2, 1dB between P0 and P3, 5dB between P0 and P4, 3dB between P0 and P5, etc). This can be accomplished by reporting per SRS resource for many cases where each SRS resource consists of one SRS port (e.g. 1T4R and 1T6R), but not for some other cases e.g. 2T4R and 2T6R where each SRS resource consists of 2 ports.

Observation 1: SRS TX power imbalance reporting per SRS resource does not allow to report different deltas for each port for all SRS cases.

One aspect which is discussed at length is whether reporting is Static or Dynamic. These can be understood in multiple ways, so we need to be very precise with definitions to avoid misunderstandings.

Observation 2: Terms “Static reporting” and “Dynamic reporting” must be clearly defined along with associated proposals to avoid misunderstandings.

One point, which was among the showstoppers in Rel-18 discussion was the applicability of the reporting for 2Rx/4Rx. This aspect should be discussed now rather than at the very late stage to progress the topic.

Observation 3: Applicability of the reporting to 2Rx/4Rx should be discussed now rather than at very late stage.

Next, we consider practical UE design, and related impacts to possible reporting. The mapping of SRS ports to physical antenna connectors may change very frequently. Hence static reporting does not work in our view, i.e. UE reporting a set of IL Delta values as UE capability seems not a viable option.

Observation 4: Static reporting meaning the reported SRS IL values or associated other parameters would not change over time seems not viable.

The reporting of SRS TX power imbalance must be dynamic of its nature. This could for instance be triggered by the UE so that UE could send new report when the reported values change. UE should report SRS TX power imbalance values which account the compensation at present time. Our current view is that RAN4 should not pursue SRS TX power imbalance reporting for UE’s which do not compensate SRS IL when possible.

Proposal 1: Pre-requisite for SRS TX power imbalance indication is that UE compensates IL when possible.

Proposal 2: If SRS TX power imbalance indication is specified, it must be dynamic so that the reported values change over time accounting UE compensation and that the reporting is reflecting the actual transmitted power differences of SRS ports.

As we pointed out in previous meeting, for vast majority of cases the UE is able to do compensation when UE is mid-cell/near-cell. In the cases, the SRS TX power sent from each SRS port is the same. Only in cases where UE does not have enough power to fully compensate each SRS port, there could be benefit of the indication. RAN4 has not studied how much indication would boost system performance for these cases.

Observation 5: Indication could be beneficial only for cases when UE does not have enough power reserve to fully compensate each SRS port.

Observation 6: RAN4 has not studied the system level benefits of indication under the assumption of UE compensating the SRS IL when possible.

The fact that there may or may not be RAN1 impacts needs to be accounted. This topic has been discussed at length in both RAN1 and RAN4 in the past so unnecessary back and forth game between RAN4 and RAN1 should be avoided yet understanding that RAN4 cannot simply request RAN1 to update their specifications as per RAN4 decisions. Making changes to RAN1 specifications requires discussion in RAN1. For instance, any updates to PHR to accommodate SRS TX power imbalance should be discussed in RAN1.

If RAN4 can find consensus that the SRS TX imbalance is specified and that RAN1 changes are needed, RAN4 should send an LS into RAN1 including RAN4 findings and let RAN1 to do their work.

The most important thing that RAN4 must do is to agree if SRS TX power imbalance reporting is done or not, including some kind of rough framework for it. Details can be ironed out later. We believe that if there is consensus in RAN4, there is also good chances to have consensus in RAN1 as well. We are positive with specifying this, if the solution is such that it really helps in the field.

Maintaining the uncertainty on if this is done or not is not very fruitful. In the previous meeting some companies who earlier were against specifying this seemed to be more positive. However, there are still many companies who were negative at least during 8Rx which did not make any comments during online, so it is hard to predict their current view.

Observation 7: The most important thing that RAN4 must do is to agree if SRS TX power imbalance reporting is done or not, including some kind of rough high-level framework for it. Details can be ironed out later.

Proposal 3: If RAN4 finds consensus on introducing SRS TX power imbalance and RAN1 changes are needed, RAN4 shall send an LS into RAN1 including RAN4 findings on the potential solutions. RAN1 may work on the RAN4 suggested solutions or other solutions.

- Conclusion

# 6Rx SRS TX power imbalance was considered with the following observations and proposals.

# Observation 1: SRS TX power imbalance reporting per SRS resource does not allow to report different deltas for each port for all SRS cases.

Observation 2: Terms “Static reporting” and “Dynamic reporting” must be clearly defined along with associated proposals to avoid misunderstandings.

Observation 3: Applicability of the reporting to 2Rx/4Rx should be discussed now rather than at very late stage.

Observation 4: Static reporting meaning the reported SRS IL values or associated other parameters would not change over time seems not viable.

Observation 5: Indication could be beneficial only for cases when UE does not have enough power reserve to fully compensate each SRS port.

Observation 6: RAN4 has not studied the system level benefits of indication under the assumption of UE compensating the SRS IL when possible.

Observation 7: The most important thing that RAN4 must do is to agree if SRS TX power imbalance reporting is done or not, including some kind of rough high-level framework for it. Details can be ironed out later.

Proposal 1: Pre-requisite for SRS TX power imbalance indication is that UE compensates IL when possible.

Proposal 2: If SRS TX power imbalance indication is specified, it must be dynamic so that the reported values change over time accounting UE compensation and that the reporting is reflecting the actual transmitted power differences of SRS ports.

Proposal 3: If RAN4 finds consensus on introducing SRS TX power imbalance and RAN1 changes are needed, RAN4 shall send an LS into RAN1 including RAN4 findings on the potential solutions. RAN1 may work on the RAN4 suggested solutions or other solutions.

- Reference

[1] RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei

[2] R4-247209, “WF on requirements for 6Rx”, AT&T