3GPP TSG-RAN WG4#113	R4-2419075

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	6.11.2

Source: 	Qualcomm France

Title: 	Considerations on CA_n40A-n41A

Document for:	Approval

# Introduction

Considerations on CA_n40A-n41C and CA_n41C-n79A are provided in this contribution.

- Background

WF was agreed in RAN4#112-bis [1]:

- Companies are asked to verify whether the same MSD value defined for CA_n40-n41 with 4Rx can be applied to 2Rx RF front end architecture for CA_n40-n41.

- Provide further information on the need for new test point for CA_n40-n41, taking into consideration the applicable bandwidth and frequency range of n40 and n41 from operator(s). For example, wider frequency separation between n40 and n41, and wider DL bandwidth.

- Companies are asked to propose suitable test point reflecting operator demand in next meeting

- 2Rx RF front end architecture can be considered.

- For n41, consider if a new signalling is necessary to support maximum of 2-layer MIMO with 2Rx antenna ports, if the UE reports both the capability of 4-layer MIMO for band n41 and simultaneous Rx-Tx for CA_n40A-n41A.

- Discussion

The first bullet in WF is a bit odd, at least unless we missed the point of it; MSD is always specified using 2RX architecture and 2RX REFSENS. In practice, 4RX UE is tested in 4RX mode in which case the REFSENS is adjusted using ΔRIB,4R and MSD testes against that. When MSD is derived using 2RX architecture, especially with PRX antenna supporting both aggressor and victim band, the MSD is usually the “worse case” compared to case when aggressor is in different antenna. Hence we think that the when the MSD value is defined using 2RX architecture as always, the resulting MSD would work fine if UE supports 4RX or 2RX, i.e. UE does not “need” more than one aggressor free antennas to meet MSD.

Proposal 1: MSD is derived using 2RX reference architecture as in all other cases, and the MSD works when UE supports 2Rx or 4RX.

We think that if additional operator specific test point is defined, it really should be the test point according to operator spectrum holdings or according to spectrum operator has access into. In our view, defining additional test point which is not reflecting these is not useful and should not be defined.

Proposal 2: If operator-specific test point is defined for CA_n40A-n41A, it shall be aligned with operator spectrum.

The last bullet has been discussed for a long time without conclusion. The requirements exist in 38.101 with 4RX mandatory. Currently CA_n40A-n41A works in non-simultaneous mode. Simultaneous Tx/Rx for CA-n40A-n41A is very challenging so a lot of efforts in RFFE should be done to make it work properly. It is also good to note that legacy UE’s could not no longer support CA_n40A-n41A in the network if the network would begin to work in simultaneous TX/RX mode. Given the timelines and e.g. upcoming 6Rx feature which might help to support 4Rx for CA_n40A-n41A, we suggest leaving this topic for now, and get back later if operator plans to deploy simultaneous TX/RX for CA_n40A-n41A.

Proposal 3: Stop discussion on relaxing the number of RX requirements. RAN4 can resume the discussion later if operator has clear plans to deploy Simultaneous TX/RX for CA_n40A-n41A.

- Conclusion

Considerations on CA_n40A-n41A were provided with the following proposals.

Proposal 1: MSD is derived using 2RX reference architecture as in all other cases, and the MSD works when UE supports 2Rx or 4RX.

Proposal 2: If operator-specific test point is defined for CA_n40A-n41A, it shall be aligned with operator spectrum.

Proposal 3: Stop discussion on relaxing the number of RX requirements. RAN4 can resume the discussion later if operator has clear plans to deploy Simultaneous TX/RX for CA_n40A-n41A.

- Reference

[1] R4-2417058, “WF on simultaneous Rx-Tx for CA_n40A-n41A”, Huawei