3GPP TSG-RAN WG4#113	R4-2419082

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	7.1.1.1.1

Source: 	Qualcomm France

Title: 	MSD for HPUE

Document for:	Approval

- Introduction

Considerations on MSD for HPUE is provided in this contribution.

- WF in RAN4#112bis

As part HPUE of WF [1], the following was agreed for MSD:

Way forward:

Option 1:

  - RAN4 agrees on the following directions in this meeting and continues on details in subsequent meetings.

  - RAN 4 defines the default power class inter-band CA/DC and intra-band CA/DC MSD requirements, and RAN4 specifies equations/simplified guidelines/lookup tables “LUT” that define the relationship between the default power class MSD and the HPUE MSD requirements to replace the existing HPUE MSD tables:

  - Equations/LUT/Simplified guidelines identifying the HPUE MSD requirements are expected to be captured in RAN4 spec as normative requirements, in Rel-19 if possible, the details are FFS

  - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19

- Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done

  - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus]

- [Agree to capture just single worst case MSD in TS for HPUE PC which is agnostic to number of TX, for each kind of MSD]

NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done.

NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow.

Option 2:

  - Current working procedure of HPUE CA/DC introduction are kept unchanged

  - HPUE MSD tables are kept

  - Unified Excel tool of calculating HPUE MSD are being developed with following details to be addressed

- Companies to evaluate if the interferer level for PC2/PC1.5 can be scaled by less than current 100% to achieve more meaningful PC2/PC1.5 IMD4 and IMD5.

- [Agree to capture just single worst case MSD value in TS for each PC which is agnostic to number of TX].

- Agree to use the following rules in deriving PC2/PC1.5 MSD for cases where PC3 MSD is 0dB:

- If one of the rules above is met, PC2 1TX MSD=0.4dB, PC2 2TX MSD=0.5dB, PC1.5 2TX MSD=0.8dB.

- If both rules above are met, PC2 1TX MSD=0.8dB, PC2 2TX MSD=0.9dB, PC1.5 2TX MSD=1.5dB.

- In case it is discovered that the PC3 MSD is omitted or is illogical compared to respective similar band combinations, then naturally the PC3 MSD can be further evaluated before PC2/PC1.5 assessment.

- Agree that the assumed PRX/DRX difference is 6dB, and additional noise into PRX is 1dB due to 2 TX case for PC2/PC1.5 NR CA and EN-DC.

- The tool is published in 3GPP in Excel format if all proposals are agreeable.

NOTE: Freedom for any company to provide PC3 MSD evaluations using companies’ calculation methods and other assumptions fully remains because PC3 is the baseline to derive PC2/PC1.5 MSD. It is not precluded to provide evaluations for PC2/PC1.5.

Option 3:

  - PC2/PC1.5 MSD requirements are not specified. UE only need to be verified under the Rx requirements of default power class, with default power class transmission

  - Add the Equations/LUT/Simplified guidelines into the Annex of TS to derive the HPUE MSD for information, in Rel-19 if possible, the details are FFS

  - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19

-Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done

  - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus]

NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done.

NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow.

Option 4:

  - PC2/PC1.5 MSD requirements are not specified, MSD for PC2 and PC1.5 are only specified in TR or Annes of TS which is not mandate, UE shall meet the Rx requirements of default power class, with default power class transmission

Agreement:

- To clarify the Equations/LUT/Simplified guidelines mentioned in Option 1 and Option 3

  - Some examples are provided in the Annex of this WF

Other options such as Option2 are not precluded

- Discussion

There was a lot of discussion about HPUE MSD in previous meeting. Overall, OEM’s seemed to favor specifying and verifying only PC3 MSD while at least some operators and other companies including RFFE consider it important to specify and perhaps also to verify HPUE MSD. The outcome of first serious discussion was not a surprise; all options are still on the table. It must be mentioned that current Rel-19 HPUE baskets include MSD for PC2/PC1.5 as one of the objectives, so changing the current practice of specifying HPUE MSD would require agreements in RAN4 and respective modifications into HPUE baskets; If nothing is agreed than Rel-19 HPUE basket goes forward as tasked by RAN4 including HPUE MSD’s

Our high-level view remains unchanged. We believe there is value to have MSD specified for HPUE in addition to PC3. Recognizing that Rel-19 is second to last release before 6G era, it would be consistent to continue specifying HPUE MSD.

We recognize that there is discrepancy in how the current PC2/PC1.5 TDD HPUE and PC2 FDD HPUE MSD’s are specified:

-PC2 TDD MSD is agnostic to number of TX i.e. same MSD with 1*26dBm or 2*23dBm

-PC1.5 TDD MSD is assuming 3dB higher interferer compared to PC2, i.e. PC1.5 MSD is max 3dB higher than that of PC2

-PC2 FDD MSD is different with 1*26dBm and 2*26dBm

We think that changes to old requirements should be avoided when possible so hence we don’t propose to make any changes to existing MSD’s.

We propose that following, which is in line with Option1/Option2 with some amendments.

- Continue specifying HPUE MSD in 38.101 for each HPUE combination

- Modify current FDD PC2 MSD to become agnostic to # of TX, i.e. use single MSD which is according to current 2TX FDD PC2 MSD

  - The larger MSD out of the two existing ones applies for currently specified combinations

- MSD values for TDD and FDD PC2 and PC1.5 are derived from PC3 MSD using Look-Up table shown below, using 6dB PRX/DRX imbalance and 0.5dB Reverse IM for 1 UL band aggressor cases (Cross-band, harmonics, Harmonic Mixing).

  - No changes are done to existing requirements. The difference between existing and new combinations shows in new TDD PC2 aggressors where the delta between PC3 and PC2 is higher (Max ~2.5dB higher) than that for the existing TDD PC2 combinations. The delta between existing PC2 and PC1.5 and new PC2 and PC1.5 remains the same

  - Please note that there is a lot of room to further reduce the size of the LUT

- TP procedure is kept unchanged, i.e. they include e.g. MSD numbers

- For cases where PC3 MSD=0dB, a dedicated rules based on the frequency relations between the bands are used to derive non-zero PC2/PC1.5 MSD

- For IMD cases, the interference level increase for PC2/PC1.5 compared to PC3 interference is calculated using coefficient 1 for per band per IMD coefficient (same approach as in the past). We will provide exact LUT or equivalent in February meeting

- For PC2/PC1.5 IMD4/IMD5, consider refining coefficient for scaling the interference increase per band smaller than 1 to specify more meaningful IMD4/IMD5

We did slightly refine the formulas in the calculator, based on the latest version a LUT could look as follows. This is based, for simplicity, on non-correlated noise. The difference between non-correlated and correlated noise is very small in this case

For cases where PC3 MSD= 0dB, we continue proposing to use the following rules in deriving PC2/PC1.5 MSD:

- Minimum distance between aggressor UL and Victim DLMax Aggressor BW<4

- Minimum distance between aggressor UL and Victim DLClosest aggressor UL Frequency<0.25

- If one of the rules above is met, PC2 MSD=0.5dB, PC1.5 MSD=0.9dB

- If both rules above are met, PC2 MSD=0.9dB, PC1.5 MSD=1.7dB

- In case it is discovered that the PC3 MSD is omitted or is illogical compared to respective similar band combinations, then naturally the PC3 MSD can be further evaluated before PC2/PC1.5 assessment

The proposed Look-up table for any new HPUE MSD’s is as follows:

- Conclusion

HPUE MSD was thoroughly considered in this contribution with the following proposals.

Proposal:

- Continue specifying HPUE MSD in 38.101 for each HPUE combination

- Modify current FDD PC2 MSD to become agnostic to # of TX, i.e. use single MSD which is according to current 2TX FDD PC2 MSD

  - The larger MSD out of the two existing ones applies for currently specified combinations

- MSD values for TDD and FDD PC2 and PC1.5 are derived from PC3 MSD using Look-Up table shown below, using 6dB PRX/DRX imbalance and 0.5dB Reverse IM for 1 UL band aggressor cases (Cross-band, harmonics, Harmonic Mixing).

  - No changes are done to existing requirements. The difference between existing and new combinations shows in new TDD PC2 aggressors where the delta between PC3 and PC2 is higher (Max ~2.5dB higher) than that for the existing TDD PC2 combinations. The delta between existing PC2 and PC1.5 and new PC2 and PC1.5 remains the same

  - Please note that there is a lot of room to further reduce the size of the LUT

- TP procedure is kept unchanged, i.e. they include e.g. MSD numbers

- For cases where PC3 MSD=0dB, a dedicated rules based on the frequency relations between the bands are used to derive non-zero PC2/PC1.5 MSD

- Minimum distance between aggressor UL and Victim DLMax Aggressor BW<4

- Minimum distance between aggressor UL and Victim DLClosest aggressor UL Frequency<0.25

- If one of the rules above is met, PC2 MSD=0.5dB, PC1.5 MSD=0.9dB

- If both rules above are met, PC2 MSD=0.9dB, PC1.5 MSD=1.7dB

- In case it is discovered that the PC3 MSD is omitted or is illogical compared to respective similar band combinations, then naturally the PC3 MSD can be further evaluated before PC2/PC1.5 assessment

- For IMD cases, the interference level increase for PC2/PC1.5 compared to PC3 interference is calculated using coefficient 1 for per band per IMD coefficient (same approach as in the past). We will provide exact LUT or equivalent in February meeting

- For PC2/PC1.5 IMD4/IMD5, consider refining coefficient for scaling the interference increase per band smaller than 1 to specify more meaningful IMD4/IMD5

- Reference

[1] R4-2417098, “WF on HPUE RF requirements”, Samsung

[2] R4-2412619, “MSD for HPUE”, Qualcomm France

- Appendix