3GPP TSG-RAN WG4#113	R4-2419084

Orlando, US, November 18th – 22nd, 2024

Agenda item:	7.5.2.3

Source: 	Qualcomm France

Title: 	Non-collocated Intra-band NR CA/EN-DC Other aspects

Document for:	Approval

- Introduction

Considerations on other aspects for Non-collocated NR CA/EN-DC UE are provided in this contribution.

- WF in RAN4#112-bis

WF [1]:

## < Issue 3-1: UE Type definition for EN-DC >

Way Forward: [1]

- The following table is a starting point, and continue further discussion the following table in the next meeting.

- The table will be added into TS 38.101-3: 7.10B	power imbalance for DC in FR1.

- Table 1: EN-DC UE Capability types

| [UE Capability Type] | Type description | Dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133. | N/A |
| Type 2 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18 |
| Type 4 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | TBD |

## < Issue 3-2: UE Type definition for NR-CA>

Way Forward: [1]

- The following table is a starting point, and continue further discussion the following table in the next meeting.

- The table will be added into TS 38.101-1: 7.10A	Power imbalance for CA.

- Table 2: NR-CA UE Capability types

| [UE capability types] | Type description | dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133 | N/A |
| Type 2 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | intraBandNR-CA-non-collocated-r18 |
| Type 4 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | TBD |

## < Issue 3-3: Harmonized approach on UE Type definition with RRM part >

Agreement:

- It would be better to have the RRM CRs after the RF CRs in order to ensure a harmonized approach.

## < Issue 3-4: Reference UE architecture >

Way Forward:

- Continue further discussion on the following aspects for reference UE architecture in the next meeting.

- Comments of proposed reference UE architecture table

- Partially/fully shared

- Differentiate Type 1a and 1b

- If identified, other additional issues

- Discussion

< Issue 3-1: UE Type definition for EN-DC >

< Issue 3-2: UE Type definition for NR-CA>

Wording “UE capability type” is a bit clumsy, but it can be used unless better wording is found. In general, we may not need the leftmost column at all. Additionally, one suggestion is to change the header “Type description” to “Description”

Proposal 1: Consider removing the leftmost column from UE Type definition tables. Additionally, header “Type description” should be changed to “Description”

< Issue 3-4: Reference UE architecture >

The most recent Reference UE architecture table is as follows.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

There has been discussion on whether this needs to be updated or not. In our view, there is in principle no reason to modify this as this has in any case been a reference roughly describing different types.

Observation 1: In principle, the table is just a reference roughly characterising different UE types. The important thing is imbalance and number of Layers.

If the table is modified, we propose to simplify it quite a bit. The reason is that the reference table, if any, should be extremely simple just capturing the main characteristics.

Proposal 2: If Reference UE architecture table is modified, the following is used

| UE Type | CC# | antenna / LNA |  | # Max Layers per CC | Frequency Separation between CC1 and CC2 | NRCA/ENDC | power imbalance |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared or 8 shared1 |  | 4 or 82 | ≤ X MHz | NRCA, ENDC | 6dB full range |
|  | 2 |  |  | 4 or 82 |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | No limitation or ≤ X MHz | NRCA, ENDC | 25dB full range |
|  | 2 | 2 |  | 2 |  |  |  |
| 3a | 1 | 4 shared |  | 4 | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range |
|  | 2 |  |  | 2 |  |  |  |
| 3b | 1 | 4 shared |  | 4 | No limitation or ≤ X MHz | NRCA, ENDC | 6<P≤25dB partial range |
|  | 2 |  |  | 4 |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | No limitation or ≤ X MHz | ENDC | 25dB full range |
|  | 2 | 2 |  | 2 |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | No limitation or ≤ X MHz | NRCA, ENDC | 25dB full range |
|  | 2 | 4 |  | 4 |  |  |  |
| Note 1: Applicable when UE supports Type 4A/4B Note 2: Applicable when UE supports 6L/8L per CC |  |  |  |  |  |  |  |

- Conclusion

Remaining aspects for Non-collocated Rel-19 Intra-band Non-contiguous CA/EN-DC were considered with the following observations and proposals.

Proposal 1: Consider removing the leftmost column from UE Type definition tables. Additionally, header “Type description” should be changed to “Description”.

Observation 1: In principle, the table is just a reference roughly characterising different UE types. The important thing is imbalance and number of Layers.

Proposal 2: If Reference UE architecture table is modified, the following is used.

| UE Type | CC# | antenna / LNA |  | # Max Layers per CC | Frequency Separation between CC1 and CC2 | NRCA/ENDC | power imbalance |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared or 8 shared1 |  | 4 or 82 | ≤ X MHz | NRCA, ENDC | 6dB full range |
|  | 2 |  |  | 4 or 82 |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | No limitation or ≤ X MHz | NRCA, ENDC | 25dB full range |
|  | 2 | 2 |  | 2 |  |  |  |
| 3a | 1 | 4 shared |  | 4 | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range |
|  | 2 |  |  | 2 |  |  |  |
| 3b | 1 | 4 shared |  | 4 | No limitation or ≤ X MHz | NRCA, ENDC | 6<P≤25dB partial range |
|  | 2 |  |  | 4 |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | No limitation or ≤ X MHz | ENDC | 25dB full range |
|  | 2 | 2 |  | 2 |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | No limitation or ≤ X MHz | NRCA, ENDC | 25dB full range |
|  | 2 | 4 |  | 4 |  |  |  |
| Note 1: Applicable when UE supports Type 4A/4B Note 2: Applicable when UE supports 6L/8L per CC |  |  |  |  |  |  |  |

- Reference

[1] R4-2417115, “WF on rest issues for intra-band non-collocated EN-DC/NR-CA”, KDDI