3GPP TSG RAN WG4 Meeting #113	R4-2419091

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.15.3

Source:	vivo

Title:	Discussion on fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

For the objective of fast SCell activation, the corresponding work was triggered in the RAN4#112 meeting. In the last meeting, RAN4 further discussed the corresponding scope and the details on how to update the known condition with consideration of valid eEMR reporting. In this contribution, we further provide our views based on the latest progress captured in WF [1].

- Discussion

#### Issue 1-1-1: Applicability of fast SCell activation delay requirements Proposals: - Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK) - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. - Option 2: (OPPO, Nokia, Samsung, Ericsson) - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR. - Option 2a: (OPPO) - Consider the following two approaches to support the case: - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration. - Option 2b: (Samsung) - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

For the case on FFS part, according to the description specified in the spec, when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured, UE is not required to perform validity check. As per the WID description, the objective aims to enhance the SCell activation delay based on ‘valid eEMR reporting’. In our understanding, the measurement results without validity checking are not considered as ‘valid’ results to some extent. On the other hand, from SCell activation perspective, the reported results may have been obtained a long time ago and the quality of SCell to be activated cannot be guaranteed.

Clause 4.7.3 in TS38.133 If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

Proposal 1: RAN4 not to define the fast SCell activation delay requirements for the case when measIdleValidityDuration-r18 and/ or measReselectionValidityDuration-r18 are not configured

#### Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting Agreement: - RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. - For FR1, - [Y] equals to [5s] for direct SCell activation - FFS: Y for other SCell activation case. - For FR2, - Reuse the existing value of Y in direct SCell activation - FFS: Y for other SCell activation case. Agreement: - the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting). - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. For continued discussion in next meeting: - For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured: - For FR1: - use existing SSB based known SCell activation delay requirement with [2] SSB samples. - For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test

In the last meeting, RAN4 further discussed how to update the known condition and the definition of corresponding requirements.

On how to define the corresponding delay requirements, for FR1, it was agreed to use existing SSB-based known SCell activation delay requirement with [2] SSB samples. And for FR2, it is still open to the details on how to use the existing SSB-based known SCell activation delay requirement with some adaptations.

In our understanding, the approach for Option 1 makes more sense to us. Based on the design in Rel-18, the configurable values of the duration of validity timer include {5s,10s,20s,50s,100s}. For FR2, the measurement delay for EMR will be longer compared with FR1 scenario and the duration of validity timer may be possibly configured as a large value, such as 50s, or 100s. Under this scenario, the measurement results may be out of date so they may not be reliable for known SCell activation. From this perspective, our preference is to limit the applicable scenario to the cases when measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X]. And for Option 3, the advantage for this option lies in adopting the same approach for requirement definition as FR1 case specified. We are also fine with this approach if it is the preference of the majority of companies.

Proposal 2: For the fast SCell activation delay requirements in FR2 with Rel-18 valid eEMR reporting, RAN4 to use existing SSB-based known SCell activation delay requirement provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X]

- Summary

Proposal 1: RAN4 not to define the fast SCell activation delay requirements for the case when measIdleValidityDuration-r18 and/ or measReselectionValidityDuration-r18 are not configured

Proposal 2: For the fast SCell activation delay requirements in FR2 with Rel-18 valid eEMR reporting, RAN4 to use existing SSB-based known SCell activation delay requirement provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X]

- References

- R4-2416854, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT