# 3GPP TSG-RAN WG4 Meeting #113  		R4-2419099

# Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on maintenance of R18 MIMO RRM requirements

Agenda Item:	5.25.1

Document for:	Discussion

Introduction

In RAN4 #112, the CR [1] is discussed. Based on proposal in [2], it is proposed to restrict scenario for RTD>CP case in FR2-1.

Based on all above information, we provide our views on how to understand progress in R18 MIMO regarding supporting of RTD>CP.

Discussion

In RAN4 #107(2023/05 Incheon meeting), the follow reply LS is agreed [3].

In RAN4#104-bis-e, RAN4 approved an LS [1] to RAN1 on MTTD for multi-DCI multi-TRP with two TAs, with the following agreements: RAN4 thanks RAN1 for the LS on maximum uplink timing difference between the two TAs for multi-DCI multi-TRP with two TAs. After RAN4 further discussion, following values are agreed as MTTD values. For a UE capable of supporting Receive Time Difference (RTD) > CP, MRTD/MTTD value for FR1 is 33/34.6 µs and MRTD/MTTD value for FR2 is 8/8.5 µs. For a UE not capable of supporting RTD>CP, MTTD is within (CP + M1 µs) for FR1 and MTTD is within (CP + M2 µs) for FR2. Where M1 and M2 are FFS in RAN4.. In RAN4#107e, RAN4 continued discussion on M1/M2 and reached the following agreements: - MTTD for UE not capable of supporting RTD > CP - If UE supports STxMP - The MTTD between multiple TRPs can be defined as (CP + M1) for FR1 and (CP + M2) for FR2 - M1=1.6us and M2=0.5 us

The above conclusion is captured in clause 7.5.7 and clause 7.6.8 of TS 38.133. Taking clause 7.6.8 as example, which is cited below, the MRTD for single carrier multi-TRP scenario is clarified as follows:

7.6.8 Minimum Requirements for Multi-TRPs A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in Table 7.6.8-1 below, if the UE does not indicate support of [FG 40-2-6]. A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in Table 7.6.8-2 below, provided that the UE indicates that it is capable of [FG 40-2-6]. Table 7.6.8-1: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE not capable of [FG 40-2-6] Frequency Range; Maximum receive timing difference. FR1; CP lengthnote 1. FR2-1; CP lengthnote 1. Note 1: CP length of the maximum SCS on the carrier.. Table 7.6.8-2: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE capable of [FG 40-2-6] Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8. A UE supporting [FG 30-1 or fastBeamSweepingMultiRx-r18] shall be capable of handling at least a relative receive timing difference between slot timing of different TCI states on the same carrier at the UE receiver as shown in Table 7.6.8-3. Table 7.6.8-3: Maximum receive timing difference requirement for UE supporting multi-RX Frequency Range; Maximum receive timing difference. FR2-1; CP length.

According to TS 38.306, TS 38.331 and RAN1 feature list, 40-2-1 and 40-2-2 are multiDCI-IntraCellMultiTRP-TwoTA-r18, multiDCI-InterCellMultiTRP-TwoTA-r18, respectively, and 40-2-6 is rxTimingDiff-r18. Although no pre-requisite is capture in TS 38.331, TS 38.306, nor RAN1 feature list, according to TS 38.133 it is clear that the new MTTD/MRTD requirements are only applicable if UE supports rxTimingDiff-r18 and one from multiDCI-IntraCellMultiTRP-TwoTA-r18 and multiDCI-InterCellMultiTRP-TwoTA-r18.

On the other hand, UE supporting mDCI m-TRP with RTD>CP does not mean UE is only able to support RTD>CP L1 measurements when UE is configured with more the one coresetPoolIndex. In our understanding, no matter for intra-cell or inter-cell L1 measurement, if it is meant to enable m-TRP with RTD>CP, e.g. by providing L1 measurement results to the gNB for further decision, then UE shall be able to support RTD>CP L1 measurement.

Observation 1  Although mDCI m-TRP with 2-TA is the pre-requisite UE capability for RTD>CP L1 measurement capability in R18 MIMO, it does not mean UE can only perform L1 measurement for RTD>CP case when more than one coresetPoolIndex is configured. L1 measurements with RTD>CP will provide measurement results for gNB to decide whether to configure/activate more than one coresetPoolIndex.

Note that for above feature, RAN4 has already agreed to define test case for FR2-1. The agreed test case list in RAN4 #110bis is cited below.

|  | Note | Source Company |
| --- | --- | --- |
| TDCP mapping tables | in Section 10 | Apple |
| TDCP tests | Subclauses in: A.4.x for EN-DC A.6.7.x for SA | Ericsson |
| TC: FR1 UE transmit timing from two TRPs. | A.4.x for EN-DC A.6.4.x for SA | Samsung |
| TC: FR2 UE transmit timing from two TRPs. | A.5.4.x for EN-DC A.7.4.x for SA | vivo |
| TC: sDCI mTRP, FR2 separate DL TCI state switching |  | Xiaomi |
| TC: sDCI mTRP, FR2 separate UL TCI state switching |  | Nokia |
| TC: sDCI mTRP, FR2 joint TCI state switching |  | MTK |
| TC: mDCI FR1 two TA, RTD>CP, joint TCI state switching |  | Huawei |

Based on above TC list, RTD>CP case is already agreed to be tested. Companies may further check A.5.4.1.2 for further information.

Observation 2  For FR2-1, RTD>CP case is already agreed to be tested, as captured in TS A.5.4.1.2, with 2-TAGs and 2 coresetPoolIndex-r16 configured.

Note that according TS 38.306, rxTimingDiff-r18 is a per-FSPC capability. UE may not need to pass above sub-test if UE does not support RTD>CP in an FR2 CC.

However, in [2], the technical justification for restriction on RTD<=CP in FR2-1 is as follows.

WF [1] on RAN4 111 Issue 1-2-4: RLM/BFD/CBD requirements for mTRP? < Way forward > Option 1 (MediaTek) - The legacy evaluation delay of RLM/BFD/CBD is applicable to RTD>CP case in FR1. The legacy RLM, BFD and CBD requirements are not applicable to RTD>CP case in FR2.. In our understanding, the requirement of RLM, BFD and CBD when RTD > CP is still unclear in current spec. There’s no statement for RTD > CP applicability defined in the requirement of RLM, BFD and CBD. Similar as L1-RSRP measurement, the measurement restriction of RLM/BFD/CBD when RTD>CP is the same as the requirements when RTD<CP in FR1. We provided corresponding CR and open to discuss how to address this in spec. Regarding FR2, RAN4 agreed not to consider RTD>CP for RLM/BFD/CBD in FR2 in R18 MIMO. Take BFD for instance, UE is required to measure SSB for L1 measurement on the BFD evaluation period in FR2. But when BFD SSB is overlapping or adjacent to the L1-RSRP SSB of the other TRP, we never discussed how to handle this case in legacy requirement of L1 measurement (RLM, BFD and CBD). Therefore, we need to make it clear that the current RLM, BFD and CBD requirements are not applicable to RTD > CP in FR2. Proposal 2: The legacy measurement restriction of RLM/BFD/CBD is applicable to RTD>CP case in FR1. The legacy RLM, BFD and CBD requirements are not applicable to RTD>CP case in FR2.

For the green highlighted part, in our view RAN4 has never made such agreement. We only observed that RAN4 has not yet agreed any spec impact regarding measurement restriction and scheduling restriction in FR2.

For the cyan highlighted part, our understanding is that at least for the case of SSB inter-cell case, there is no issue. The following is already captured in existing TS 38.133.

For FR2, when the SSB for RLM measurement on one CC is in the same or adjacent OFDM symbol as SSB from cell with additional PCI for BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based RLM is expected, and no requirements are defined. … For FR2, when the SSB for BFD measurement on one CC is in the same or adjacent OFDM symbol as SSB from cell with additional PCI for L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based BFD is expected, and no requirements are defined.

In our view, the measurement restriction also needs to be revised for the SSB+CSI-RS case, and CSI-RS+CSI-RS case. Corresponding change is in our CR [4].

For scheduling restriction, RTD>CP case is also missing in FR2. RAN4 shall introduce scheduling restriction similar as FR1.

Observation 3  Existing measurement restriction between SSBs for the inter-cell case may also apply to the case when RTD>CP

Proposal 1  RAN4 to further introduce measurement restriction and scheduling restriction in FR2-1 for the RTD>CP with the following scenarios:

- Measurement restriction: CSI-RS+SSB

- Measurement restriction: CSI-RS+CSI-RS

- Scheduling restriction: SSB + DL/UL

- Scheduling restriction: CSI-RS + DL/UL

Proposal 2  RAN4 not to revert the agreement on introducing FR2-1 UE capability of supporting RTD>CP for R18 mDCI mTRP with 2-TA, which is agreed in RAN4 #104bis. With existing R18 MRTD/MTTD requirements and test cases, RAN4 further solve identified issues for supporting such feature in R18 maintenance.

Another issue would be how to differentiate L1-RSRP measurement requirements for RTD>CP achieved in different WI. As discussed above in observation 1, L1-RSRP measurement with RTD>CP shall not only be applied when mDCI mTRP is configured. Therefore, we prefer to keep existing 9.5 and 9.13 unchanged, which requirement applicability specified in 9.5.2 and 9.13.2.

Proposal 3  RAN4 not to add additional requirement applicability for 9.5 and 9.13 regarding RTD>CP, because they are already clarified in 9.5.2 and 9.13.2.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  Although mDCI m-TRP with 2-TA is the pre-requisite UE capability for RTD>CP L1 measurement capability in R18 MIMO, it does not mean UE can only perform L1 measurement for RTD>CP case when more than one coresetPoolIndex is configured. L1 measurements with RTD>CP will provide measurement results for gNB to decide whether to configure/activate more than one coresetPoolIndex.

Observation 2  For FR2-1, RTD>CP case is already agreed to be tested, with 2-TAGs and 2 coresetPoolIndex-r16 configured, as captured in TS A.5.4.1.2.

Observation 3  Existing measurement restriction between SSBs for the inter-cell case may also apply to the case when RTD>CP

Proposal 1  RAN4 to further introduce measurement restriction and scheduling restriction in FR2-1 for the RTD>CP with the following scenarios:

- Measurement restriction: CSI-RS+SSB

- Measurement restriction: CSI-RS+CSI-RS

- Scheduling restriction: SSB + DL/UL

- Scheduling restriction: CSI-RS + DL/UL

Proposal 2  RAN4 not to revert the agreement on introducing FR2-1 UE capability of supporting RTD>CP for R18 mDCI mTRP with 2-TA, which is agreed in RAN4 #104bis. With existing R18 MRTD/MTTD requirements and test cases, RAN4 further solve identified issues for supporting such feature in R18 maintenance.

Proposal 3  RAN4 not to add additional requirement applicability for 9.5 and 9.13 regarding RTD>CP, because they are already clarified in 9.5.2 and 9.13.2.

- References

[1]  R4-2411708  CR on core maintenance for R18 MIMO, MediaTek.  RAN4 #112

[2]  R4-2411707  Discussion on R18 MIMO for RRM core part requirement, MediaTek.  RAN4 #112

[3]  R4-2310174  LS on MTTD for multi-DCI mult-TRP with two TAs, Apple. RAN4 #107

[4]  R4-2419100  CR on R18 MIMO RRM requirements, vivo.  RAN4 #113