# 3GPP TSG-RAN WG4 Meeting #113  		R4-2419104

# Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on RRM requirements for on-demand SSB SCell operation in R19 NES

Agenda Item:	7.22.2.1

Document for:	Discussion

Introduction

In RAN4 #112bis, the WF and LS to RAN1 [1], [2] are approved.

Based on all above information, we provide our views on RRM requirements for on-demand SSB SCell operation.

Discussion

<On UE processing time when OD-SSB is triggered >

In last meeting, RAN4 discussed RAN1’s LS [3], but no conclusion has been achieved. Summary of proposal is listed as follows.

- Proposal - Option 1: Ericsson, CMCC, CATT, Samsung, ZTE, Huawei, LG, [vivo] - RAN4 to confirm that T_min equals to +1. - Option 2: Qualcomm, Apple, [vivo] - Additional processing time (T△) is needed. - Option 2-1: T△ > 1 slot - Option 2-2: 5ms - Option 3: Huawei - RAN4 to further study the following scenarios if agreed by RAN1 - Scenario 2 in issue 1-3-1: when the OD-SSB is very close to the last always-on SSB in time domain - Scenario 3: when the OD-SSB and always-on SSB is in different frequency (frequency location w.r.t active BWP)

In legacy L3 measurement requirements in connected mode, CSSF is defined for both measurement outside gaps and measurement within gaps. For measurement within gaps, as cited from Table 9.1.5.2.1-1 as follows, one searcher is assumed for PCC, and one searcher is shared between PSCC, SCC and inter-frequency SSB measurements without gaps, if any. Considering PCC and SCC, an illustration of UE measurement behaviour for outside gap case is illustrated in Figure 1.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required; CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap; CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap [TBD]. FR1 only CA; 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO; NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO; N/A; N/A; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO; NSCC_SSB +Y+Z+2x NSCC_CSIRS. FR2 only intra band CA; N/A; N/A; 1+NPCC_CSIRS; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS. FR2 only inter band CA; N/A; N/A; 1+NPCC_CSIRS; 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5; 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM). FR1 +FR2 CA (FR1 PCell) Note 1; 1+NPCC_CSIRS; 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); N/A; 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5; 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM). FR1 +FR2 CA (FR2 PCell) Note 1; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; 1+NPCC_CSIRS; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS. Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. Note 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. Note 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. Note 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured Note 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. Note 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Note 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. Note 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0..

Figure 1  Illustration of connected mode CSSF for L3 measurement performed outside gaps.

For OD-SSB based L3 measurement in case 1, RAN4 has agreed to perform OD-SSB based L3 measurement in last meeting. The purpose is to allow gNB to only transmit SSB in a short interval. This would have a lot of impacts to existing measurement scheduler, if cell identification for OD-SSB based SCell is needed. For example, gNB may have no information on which carrier UE is performing cell search, when it indicates UE to perform OD-SSB-based L3 measurements on this SCell. To achieve higher network energy saving gain, UE shall prioritize such cell search on the SSB frequency layer of OD-SSB-based SCell.

In this case UE would need some time to re-schedule its searcher, and prepare RF for this carrier. Such time would be different, depending on different cases. For example, for case 1, RF retuning is anyway needed. However, for case 2, if OD-SSB and AO-SSB(always-on SSB) have the same SSB frequency, and the spatial relation of the two kinds of SSBs are the same, then UE may not need to retune its RF, but still needs to perform cell search if the SCell is unknown, hence time for re-scheduling its searcher is needed. To deal with the worst case without complicating the spec, it is proposed to consider 1 ms for UE processing time. In our view most of the UEs would need such processing time, hence it is preferred to be captured in RAN1 spec.

As discussed above, it is not good for network energy saving if UE performs OD-SSB based cell identification by sharing searcher with legacy SCell measurement. Therefore, if cell identification is needed, it is also proposed to prioritize the OD-SSB based L3 measurements over legacy L3 measurements on PSCCs and other SCCs when applicable.

Note that RAN1/2 have not yet precluded the case that UE may need to perform OD-SSB based L3 measurements on more than one SSB frequencies. For example, UE may receive 2 OD-SSB indication MAC CEs in the same DL slot. In this case, RAN4 may need to discuss whether an OD-SSB specific CSSF is needed.

Proposal 1  Additional processing time is needed after OD-SSB is activated by RRC or MAC CE. Prefer [1] ms for this delay and this delay is captured in RAN1 specs.

Proposal 2  If cell identification is needed for OD-SSB-based L3 measurements, OD-SSB based L3 measurement is prioritized over legacy L3 measurement on PSCC and other SCCs, i.e. RRM requirements on PSCC and other SCCs are not applicable during the time-window when UE performs OD-SSB based L3 measurement with cell identification.

- This applies to both Case 1 and Case 2.

- RAN4 further discuss CSSF if UE receives OD-SSB activation for multiple SSB frequencies.

On the other hand, in last meeting, RAN4 have also discussed about scenarios when OD-SSB-based cell identification are not needed.

Issue 1-2-3: Cell identification - Proposals - Option 1: Ericsson - The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios. - Case 1: No always-on SSB on the cell - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [5s] and then the OD-SSB is transmitted again with the same spatial reception parameter - Case 2: Always-on SSB on the cell. - Option 2: vivo - Specify RRM requirements for OD-SSB based cell identification, i.e. SSB-based rough sync on the SCell, at least for the case when there is no inter-carrier synchronization indication provided by gNB. - Option 3: China Telecom - RAN4 to define OD-SSB based deactivated SCell measurement which includes cell identification requirements and measurement period requirements. - Option 4: LG - RAN4 to define multiple times OD-SSB indication/deactivation requirement.

We generally agree with the direction of option 1. If there is always-on SSB, UE might not need to perform OD-SSB based cell identification if the SCell is known. However, if the SCell is unknown, OD-SSB based cell identification might also be needed even if there is AO-SSB. If there is no always-on SSB, and the SCell is known based on OD-SSB-based L3 measurement reporting, UE might also not need to perform OD-SSB based cell identification. Another case is that if cross-carrier time-frequency synchronization information can be provided to UE, like QCL information or reference Cell information. But we agree with moderator that those needs to be discussed and agreed in RAN1 first.

Since in these cases, UE may not need to perform multi-FFT based cell search, it might be possible for UE to perform L3 measurements on other SCC during the time window that UE performs OD-SSB based L3 measurements.

Proposal 3  UE may not need to perform OD-SSB based SCell identification if this SCell is known. RAN4 may further discuss whether the known condition for SCell activation can be re-used here for OD-SSB based cell identification.

Proposal 4  RAN4 further discuss that if OD-SSB based cell identification is not needed, whether UE can perform legacy L3 measurement on other SCC/PSCC during the time window that UE performs OD-SSB based L3 measurement, i.e. whether legacy L3 measurement on other SCC are not impacted by OD-SSB based L3 measurement.

<On OD-SSB based deactivated SCell L3 measurement (Case 1)>

In last meeting, RAN4 achieved the following agreements for case 1:

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

Firstly, there is no big difference between option 2-2 and option 2-3. We would prefer option 2-1 so as to make solution aligned with case 2.

Regarding the time window is configurable or not, we do not yet see the use case for such flexibility by configuration, but open to discuss.

Regarding the requirements, in our view it shall be based on OD-SSB periodicity. Any further scaling, such as CSSF proposed above, can be further discussed.

Regarding how UE receives OD-SSB deactivation, this is still in RAN1 discussion. For example, UE may receive OD-SSB de-activation in the signalling for OD-SSB activation indication, i.e. N available SSB occasions are indicated in the OD-SSB activation MAC CE.

Proposal 5  For case 1, if UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window:

- the time window is pre-defined by default. RAN4 may further discuss the necessity of gNB configured time window.

- the fast L3 measurement requirement is based on OD-SSB periodicity. The detailed MO configuration is up to RAN2.

- How UE receives OD-SSB deactivation is up to RAN1.

<On OD-SSB based deactivated SCell L3 measurement (Case 2)>

For case 2, RAN4 had a lot of discussion in last meeting. The following is captured in [1].

Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation - Background From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible based on RAN1’s current agreements. - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different frequencies. Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement. Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreements: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE) - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO) - Option 3: RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE) - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm) - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

In our understanding, RAN1 will discuss whether to support scenario 2 and scenario 3 as mentioned above.

For case 2 scenario 1, there are similar issues as case 1, as shown in Figure 1. When OD-SSB indication is provided, network may not know which SCC frequency layer UE is performing cell search. Moreover, UE sharing searcher among SCC layers is not good enough for network energy saving.

Therefore, we would propose to follow the same approach also for case 2 scenario 1, i.e. the time-window-based fast OD-SSB L3 measurement is also applied to case 2. Outside the time window, UE falls back to the legacy behaviour by using MeasCycleSCell.

Note that for case 2, UE may anyway be able to perform L3 measurement for this SCell based on AO-SSB. Therefore, it is also possible that gNB indicates SMTC on this frequency layer in the MO configuration, but not OD-SSB periodicity. Such MO shall be the servingCellMO. By indicating SMTC on this frequency layer for L3 measurements, i.e. in the MO configuration, there is no any impact to legacy L3 measurements, no matter whether OD-SSB is indicated or not, and UE may only perform L1 measurements after active SCell or OD-SSB based SCell activation. However, the details of MO and servingCellMO configurations shall be discussed in RAN2. Whether RAN4 also supports UE performing de-activated SCell measurement based on measCycleSCell depends on RAN2 detailed signalling design on MO configuration.

Proposal 6  For case 2, RAN4 waits further conclusion from RAN2 on how MO and servingCellMO are configured. Whether RAN4 also supports UE performing de-activated SCell measurement based on measCycleSCell depends on RAN2 detailed signalling design on MO configuration.

Proposal 7   For case 2 scenario 1, RAN4 at least supports UE performing de-activated SCell measurement based on OD-SSB periodicity only within a time window, same as case 1, at least when cell identification is needed.

<On OD-SSB based SCell activation requirement>

Firstly, for intra-band co-located CA case, how much NW energy saving gain can be achieved for the OD-SSB case would be unclear, and such scenario significantly complicates the UE RRM requirements for SCell activation. As discussed in the SI phase, most NW energy comes from gNB TxRUs to support large antenna array, and with intra-band collocated active serving cell, such TxRUs shall be already activated. Since the power consumption is already high, periodic SSB transmission, if needed, on an intra-band collocated SCell, will not cause a significant power consumption issue. On the other hand, no matter SSB is transmitted or not for an intra-band collocated SCell, spec impacts would be quite large for both cases, at least in FR1. Therefore, RAN4 can consider deprioritize such scenario, since the necessity of such feature and the network energy saving gain are not clear, but big spec impact is foreseen.

Proposal 8  RAN4 to deprioritize the intra-band co-located CA for OD-SSB based SCell operation, at least in FR1.

Regarding known condition, RAN4 discussed and made the following agreements in last meeting.

Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded.

The legacy known cell condition is cited as follows.

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

In above cited known cell condition, taking FR1 as example, max(5*measCycleSCell,  5*DRX cycles) is the duration for the worst case UE performs measurement with SSB. Even if OD-SSB is considered, we do not see an issue. As discussed in above de-activated SCell measurement, UE may perform SCell measurement on this OD-SSB frequency layer based on this measCycleSCell in some cases. Therefore, to have a more general form of known condition, we think the legacy way still works. Moreover, for case 2, when AO-SSB is available, such L3-RSRP reporting can be based on either OD-SSB or AO-SSB.

Proposal 9  RAN4 re-use legacy known/unknown SCell condition for SCell activation based on OD-SSB. For case #2, a valid measurement report and the SSB measured can be based on either AO-SSB or OD-SSB.

If proposal 8 above is agreeable for both FR1 and FR2, then there would be no need to consider intra-band collocated CA for both FR1 and FR2.

For intra-band non-collocated CA in FR1, as discussed in R18, it shall be up to UE capability whether AGC can be adjusted for two intra-band CCs with non-collocated serving cells separately. For simplicity, it is possible to only consider the case UE have such capability.

Proposal 10  RAN4 not to consider TFirstSSB relaxation of AGC adjustment for all active serving cells and to-be-activated SCell in the same band, i.e. the similar UE behaviour as supporting shortMeasInterval-r18 is considered.

- This is at least applied if intra-band collocated CA are not considered.

- FFS: For intra-band non-collocated CA in FR1, only consider UE with similar behaviour as supporting intraBandNR-CA-non-collocated-r18

<On active SCell with OD-SSB>

After SCell activation, UE may still need to perform serving cell L3/L1 measurement, and BFD/CBD if needed, based on OD-SSB or AO-SSB for this SCell.

For case #1 defined in RAN1, where there is no ‘always-on’ SSB, UE shall also be able to perform rough sync on SSB for the SCell if needed, e.g. when DL SINR is low and UE somehow lost sync momentarily. In such case, no matter whether L3 measurement is configured or not, network shall ensure the availability of OD-SSB, which is the QCL-train source of the SCell in most cases. In other word, after SCell activation is completed, all RRM requirements for the OD-SSB based SCell, including TCI activation, L1/L3 RSRP reporting, and potential BFD/CBD, are only applicable if there is available OD-SSB transmission. Otherwise, if there is sudden signal quality drop due to random UE movement, UE may not be able to re-sync based on SSB. Note that to ensure the performance of UE rough sync, the periodicity of OD-SSB shall be no larger than 160ms

Proposal 11  If OD-SSB is assumed to be the QCL-train source of the activated SCell, all RRM requirements, including TCI activation, L1/L3 measurement reporting, and potential BFD/CBD etc., are only applicable when there is available OD-SSB transmission as indicated by gNB, and the periodicity of the OD-SSB is not more than 160ms.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  Additional processing time is needed after OD-SSB is activated by RRC or MAC CE. Prefer [1] ms for this delay and this delay is captured in RAN1 specs.

Proposal 2  If cell identification is needed for OD-SSB-based L3 measurements, OD-SSB based L3 measurement is prioritized over legacy L3 measurement on PSCC and other SCCs, i.e. RRM requirements on PSCC and other SCCs are not applicable during the time-window when UE performs OD-SSB based L3 measurement with cell identification.

- This applies to both Case 1 and Case 2.

- RAN4 further discuss CSSF if UE receives OD-SSB activation for multiple SSB frequencies.

Proposal 3  UE may not need to perform OD-SSB based SCell identification if this SCell is known. RAN4 may further discuss whether the known condition for SCell activation can be re-used here for OD-SSB based cell identification.

Proposal 4  RAN4 further discuss that if OD-SSB based cell identification is not needed, whether UE can perform legacy L3 measurement on other SCC/PSCC during the time window that UE performs OD-SSB based L3 measurement, i.e. whether legacy L3 measurement on other SCC are not impacted by OD-SSB based L3 measurement.

Proposal 5  For case 1, if UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window:

- the time window is pre-defined by default. RAN4 may further discuss the necessity of gNB configured time window.

- the fast L3 measurement requirement is based on OD-SSB periodicity. The detailed MO configuration is up to RAN2.

- How UE receives OD-SSB deactivation is up to RAN1.

Proposal 6  For case 2, RAN4 waits further conclusion from RAN2 on how MO and servingCellMO are configured. Whether RAN4 also supports UE performing de-activated SCell measurement based on measCycleSCell depends on RAN2 detailed signalling design on MO configuration.

Proposal 7   For case 2 scenario 1, RAN4 at least supports UE performing de-activated SCell measurement based on OD-SSB periodicity only within a time window, same as case 1, at least when cell identification is needed.

Proposal 8  RAN4 to deprioritize the intra-band co-located CA for OD-SSB based SCell operation, at least in FR1.

Proposal 9  RAN4 re-use legacy known/unknown SCell condition for SCell activation based on OD-SSB. For case #2, a valid measurement report and the SSB measured can be based on either AO-SSB or OD-SSB.

Proposal 10  RAN4 not to consider TFirstSSB relaxation of AGC adjustment for all active serving cells and to-be-activated SCell in the same band, i.e. the similar UE behaviour as supporting shortMeasInterval-r18 is considered.

- This is at least applied if intra-band collocated CA are not considered.

- FFS: For intra-band non-collocated CA in FR1, only consider UE with similar behaviour as supporting intraBandNR-CA-non-collocated-r18

Proposal 11  If OD-SSB is assumed to be the QCL-train source of the activated SCell, all RRM requirements, including TCI activation, L1/L3 measurement reporting, and potential BFD/CBD etc., are only applicable when there is available OD-SSB transmission as indicated by gNB, and the periodicity of the OD-SSB is not more than 160ms.

- References

[1]  R4-2416916  WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson.  RAN4 #112bis

[2]  R4-2416913  LS on SSB relation in On-demand SSB and SSB adaptation on SCell, Ericsson.  RAN4 #112bis

[3]  R4-2414911  LS on timeline for On-demand SSB operation on SCell, Apple. RAN4 #112bis