# 3GPP TSG-RAN WG4 Meeting #113  		R4-2419105

# Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on RRM requirements for common signal or channel adaptation in R19 NES

Agenda Item:	7.22.2.2

Document for:	Discussion

Introduction

In RAN4 #112bis, the WF and LS to RAN2 [1], [2] are approved. In one the follow spec impact is agreed.

Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

Based on all above information, we provide our views on RRM requirements SSB adaptation in connected mode.

Discussion

<On impacts to L1 measurements>

Here L1 measurements refers to the first 4 bullets above, i.e. L1-RSRP for serving cell, L1-SINR, BFD, CBD.

Firstly, our understanding is that CBD is only performed after beam failure is detected. Also note that there is no legacy CBD transition period requirement defined in TS 38.133. Therefore, we prefer to follow the legacy way, i.e. not to define RRM requirements if the adaptation of SSB periodicity happens during the time UE performs CBD.

Proposal 1  RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs CBD.

For the other 3 parts, so far, the RAN1 conclusion for SSB adaptation is dynamic adjustment of SSB periodicity, as discussed above. Actually, RRM requirements for transition period, when UE switches between DRX and non-DRX, is already clear for all kinds of RRM requirements, e.g. RLM/BFD/CBD, L1-RSRP/L1-SINR measurement and L3 measurement. RAN4 may further check whether no additional impact is necessary.

In existing BFD requirements, transition period requirements are defined for 4 different cases, as cited below. The transition period requirements defined for L3 measurements and L1-RSRP/L1-RSRP measurements are also cited.

TS 38.133 v18.6.0 ### 8.5.10 Minimum requirement at transitions for beam failure detection When the UE transitions between DRX and no DRX or when DRX cycle periodicity changes, for each BFD-RS resource, for a duration of time equal to the evaluation period corresponding to the second mode after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation period corresponding to the first mode and the second mode. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second mode for each BFD-RS resource. When the UE transitions from a first configuration of BFD resources to a second configuration of BFD resources that is different from the first configuration, for each BFD resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each BFD resource present in the second configuration. When the UE transitions from a first configuration of active TCI state of the CORESET to a second configuration of active TCI state of the CORESET, for each CSI-RS for BFD present in the second configuration, the UE shall use an evaluation period corresponding to the second configuration from the time of transition. For UE supporting ncd-SSB-BWP-Wor-r18, when the UE transitions between BFD CD-SSB resource and BFD NCD-SSB resource due to BWP switching or due to SCG activation or deactivation during one evaluation period, the UE shall use an evaluation period that is the maximum of the evaluation period corresponding to the first SSB type and the second SSB type after the BWP switching or the SCG activation or deactivation. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type for each BFD-RS resource. ### 9.1.6 Minimum requirement at transitions When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply. The carrier-specific scaling factor specified in clause 9.1.5 that applies to the other impacted measurement objects will also apply based on the longer measurement or cell identification delay before or after the transition. When the UE transitions between DRX and non-DRX or when DRX cycle periodicity changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition. Subsequent to this measurement period, the cell identification and measurement period requirements on each measurement object are corresponding to the second mode after transition.

For the first two types of transition requirements of BFD, there is no requirements during transitions. For the third type, there is no transition period, as the requirements switches immediately after TCI is switched. For the 4th type, UE follows the more relaxed requirements between the two different requirements, when possible.

For the L1/L3 part, similar requirements are defined in 9.1.6 of TS 38.133. In that case, only the case with more relaxed requirements is considered.

Based on existing requirements, it is worth to discuss how to capture the SSB adaptation requirements by transition period. Our understanding is that at least RAN4 shall follow the L3 measurement approach, i.e. UE follows the more relaxed requirements between the two different requirements, when possible.

Proposal 2  For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD, L1-RSRP measurement of serving cell and L1-SINR measurement of serving cell.

Another issue would be whether to consider inter-cell L1 measurement for this SSB adaptation. In Rel-17, ICBM with inter-cell L1 measurement is supported by R17 MIMO WI. So far there is no discussion in RAN1 yet regarding whether SSB adaptation also needs to consider the inter-cell scenario. In our view, support of SSB adaptation under inter-cell scenario is possible considering intra-DU deployment, but the corresponding signalling and mechanism need to be agreed in RAN1/2 first.

Proposal 3  The support of SSB adaptation under inter-cell scenario needs to be discussed and agreed in RAN1/2 first. RAN4 postpone corresponding discussion until RAN1/2 make conclusions.

<On impacts to TCI activation and SCell activation>

This is similar to the issue of CBD. In legacy, change of SSB measurement period due to change of DRX cycle length is also not considered in TCI activation and SCell activation. It would be a corner case if this happens. The rule shall be followed when we discuss SSB periodicity adaptation.

Proposal 4  RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation.

<On impacts to L3 measurements>

In last meeting, RAN4 has sent LS to RAN2 regarding the whether there is any SMTC impacts. Therefore, we think RAN4 shall wait for RAN2 conclusion first. RAN4 may stop the corresponding discussion if RAN2 agrees no impact to SMTC.

Proposal 5  For impacts to L3 measurements, RAN4 to wait for RAN2 conclusion regarding whether there is impact to SMTC due to SSB adaptation.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs CBD.

Proposal 2  For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD, L1-RSRP measurement of serving cell and L1-SINR measurement of serving cell.

Proposal 3  The support of SSB adaptation under inter-cell scenario needs to be discussed and agreed in RAN1/2 first. RAN4 to postpone corresponding discussion until RAN1/2 make conclusions.

Proposal 4  RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation.

Proposal 5  For impacts to L3 measurements, RAN4 to wait for RAN2 conclusion regarding whether there is impact to SMTC due to SSB adaptation.

- References

[1]  R4-2416860  WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei.  RAN4 #112bis

[2]  R4-2416911  LS on SSB adaptation, Apple.  RAN4 #112bis