# 3GPP TSG-RAN WG4 Meeting #113  		R4-2419106

# Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on RRM requirements for event-triggered L1-RSRP reporting in R19 LTM

Agenda Item:	7.24.2.1

Document for:	Discussion

Introduction

In RAN4 #112bis, the WF for R19 LTM [1] is approved.

Based on all above information, we provide our views on RRM requirements for event-triggered L1-RSRP reporting in R19 LTM.

Discussion

<On number of events UE may supports>

In last meeting, the following issue is discussed.

- For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options: - Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers. - Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling.

In R18, LTM-CSI-ReportConfig is configured under csi-MeasConfig of ServingCellConfig. The max number of report configs is limited by UE capability supportedMaxAperiodic-LTM-CSI-ReportConfig-r18, supportedMaxPeriodic-LTM-CSI-ReportConfig-r18, and supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r18, which are reported per-BC. In each LTM-CSI-ReportConfig, only one report type, either periodic, semi-persistent or aperiodic, is allowed. For event-triggered L1 reporting, how to configure this new kind of reporting is subject to RAN1/2 discussion. But in our view, it is highly possible that this new kind of reporting is treated as a new report type, while the max number of report config supported by UE shall be discussed and determined by RAN1/2.

So far, RAN1/2 has made the following agreements about the resource type that can be configured in an LTM event.

RAN2 #126 Agreements - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. RAN2 #127 Agreements - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. RAN2 #127bis Agreements - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

RAN1 #118bis Agreements - The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS - Note: This does not imply the support of mTRP scenarios

RAN1/2 have already agreed to use same RS type for event evaluation. For each event, which type of RS is reported shall be explicitly configured in the report quantity, i.e. either SSBRI or CRI. SSBRI is the only supported report quantity in R18. For R18 gNB triggered reporting, the number of layers can be identified by number of SSB frequency configured for measurement. Hence, even multiple candidate cells with different SSB frequency is configured in a same resourceSet, UE may perform SSB-based L1 measurement at different time occasion if UE supports RTD>CP in FR1.

The issue would be similar for the event-triggered case. UE may identify the SSB frequency layer based on SSB frequency, and perform L1 measurement according to R18 rules. For LTM3 and LTM5, there is no differentiation between frequency layers, i.e. any L1 measurement results that meet the event-triggered condition may trigger L1 measurement reporting.

However, both option 1 and option 2 above are based on the methodology that LTM events are triggered in a per-frequency layer basis. This is probably not aligned with RAN1/2 design. In case RAN4 has concern on the conclusions from RAN1/2, we can send the LS. However, we do not see conclusion or discussion in RAN1/2 yet. RAN4 shall further wait RAN1/2 conclusion for this.

Proposal 1  Regarding whether to count LTM event in a per-layer basis, RAN4 shall wait for further RAN1/2 conclusions regarding the detailed resource configuration for event-triggered L1-RSRP reporting in R19 LTM.

There is another proposal in RAN4 #112bis to update the requirements for reporting criteria per measurement category captured in TS 38.133, so as to cover L1-RSRP measurement on neighbor cell.

However, for clause 9.1.4 of TS 38.133, the following general rule is captured.

- 9.1.4.1 Introduction This clause contains requirements on UE capabilities for support of event triggering and reporting criteria. As long as the measurement configuration does not exceed the requirements stated in clause 9.1.4.2, the UE shall meet all other performance requirements defined in clause 9 and clause 10. The requirements in this clause also apply for a UE in EN-DC with PSCell on a carrier frequency with CCA or SA NR with PCell on a carrier frequency with CCA. The UE can be requested to make measurements under different measurement identities defined in TS 38.331 [2]. Each measurement identity corresponds to either event-based reporting, periodic reporting, or no reporting. In case of event-based reporting, each measurement identity is associated with an event triggering criterion. In case of periodic reporting, a measurement identity is associated with one periodic reporting criterion. In case of no reporting, a measurement identity is associated with one no reporting criterion. The purpose of this clause is to set some limits on the number of different event triggering, periodic, and no reporting criteria the UE may be requested to track in parallel. - 9.1.4.2 Requirements In this clause a reporting criterion corresponds to either one event (in the case of event-based reporting), or one periodic reporting criterion (in case of periodic reporting), or one no reporting criterion (in case of no reporting). For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table 9.1.4.2-1.

As cited above, the requirements count criteria for event-based reporting, periodic reporting and no reporting. These are all belongs to the L3 reports. As analysed above, RAN1 has already defined capability for L1 event reporting. Since periodic reporting is also considered, we prefer to discuss the impacts from L1 reporting in R18 maintenance. After that, we may further reuse R18 methodologies in R19.

Observation 1  Existing requirements for number of reporting criteria per category also counts periodic reporting. If L1 reporting has impact on this, gNB-triggered reporting might also need to be considered. In this case the discussion shall be in R18 LTM maintenance first.

Proposal 2  RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first.

< On UL resource waiting time>

RAN2 has made the following conclusions in R19 LTM.

RAN2 #127 Agreements - Event-triggered L1-measurements are reported by the UE to the network via MAC CE. RAN2 #127bis Agreements - The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. - NW can configure a dedicated SR configuration for MR MAC CE transmission.

In case L1 reporting based on MAC CE is need, UE may send a dedicated SR if there is no existing UL grant. If there is UL grant, UE may directly pack the L1 content. There is no need for further SR transmission. Therefore, RAN4 may follow existing logic specified in 9.2.4 of TS 38.133 for defining delay requirements of event-triggered L1 reporting, i.e. RRM requirements are defined assuming UL resource for the reporting is always available.

Proposal 3  RAN4 to re-use the uncertainty defined for L3 reporting delay as the clarification of UL resource waiting time in for event-triggered L1-RSRP reporting in R19 LTM.

< On L1 event triggered reporting delay >

Regarding the how the report shall be triggered, RAN1 and RAN2 have made the following conclusions.

RAN2 Agreements - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - Event LTM1 is not defined. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met. - MAC layer handles the event evaluation and measurement report triggering. - MR can be sent when the leaving condition is met, based on NW configuration. - Event triggered periodic MR can be supported, based on NW configuration. - The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. - TTT operates only based on a timer (like TTT used in L3 event triggered MR). - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

RAN1 Agreements - The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS - Note: This does not imply the support of mTRP scenarios

RAN1 and RAN2 are still discussing more details on how the report shall be triggered. For example, if the indicated TCI from serving cell changes due to MAC CE based or DCI based TCI state switching, whether TTT timer needs to be re-started, or whether event leaving condition shall be automatically triggered. On the other hand, for legacy L3 measurement and reporting, from testability point of view, normally testing of measurement and reporting delay only verifies the duration that UE needs for evaluation when a fixed condition is met over the air. There is no handover or other procedures that may change the serving cell(s) during the time UE is expected to perform L3 measurements.

Proposal 4  RAN4 starts the discussion on measurement reporting delay focusing:

- No indicate TCI change during the evaluation period.

- No configuration change w.r.t measurement RS during the evaluation period

- After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

In last meeting, companies discussed whether to use beam level measurement result or cell level result. In our view this is already agreed in RAN2.

Proposal 5  For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation, as RAN2 agreed.

Regarding the number of samples and the basic beam sharing principle, at least for SSB-based L1 measurement, in our view R18 rules can be re-used.

Proposal 6  On number of samples for L1 measurement, RAN4 re-use M=1 for intra-frequency and M=2 for inter-frequency for SSB based L1 measurements.

<On spec structure>

Regarding the spec structure, our view is that the R18 spec still needs further polishing. LTM related L1 measurements shall be captured in LTM related clauses without impacting legacy L1 measurement for serving cell, i.e. without impacting clause 9.5 of TS 38.133. Our view is that, in case UE supports multiple FFT based L1 measurement for mobility, such UE behaviour shall be significantly different from legacy UE behaviour for serving cell, hence the results would be possibly not comparable for serving cell and neighbour cell. For LTM related reporting, serving cell L1 measurement shall also be captured in 9.14.

Therefore, although it would be slightly early to discuss the spec structure, our view is that, event triggered L1 measurement reporting for both serving cell and neighbour cells are captured in 9.14 and 9.15 (for inter-frequency without gap case).

Proposal 7  Event Triggered Reporting for LTM L1 measurement is to be introduced in section 9.14 and section 9.15.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  Regarding whether to count LTM event in a per-layer basis, RAN4 shall wait for further RAN1/2 conclusions regarding the detailed resource configuration for event-triggered L1-RSRP reporting in R19 LTM.

Observation 1  Existing requirements for number of reporting criteria per category also counts periodic reporting. If L1 reporting has impact on this, gNB-triggered reporting might also need to be considered. In this case the discussion shall be in R18 LTM maintenance first.

Proposal 2  RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first.

Proposal 3  RAN4 to re-use the uncertainty defined for L3 reporting delay as the clarification of UL resource waiting time in for event-triggered L1-RSRP reporting in R19 LTM.

Proposal 4  RAN4 starts the discussion on measurement reporting delay focusing:

- No indicate TCI change during the evaluation period.

- No configuration change w.r.t measurement RS during the evaluation period

- After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

Proposal 5  For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation, as RAN2 agreed.

Proposal 6  On number of samples for L1 measurement, RAN4 re-use M=1 for intra-frequency and M=2 for inter-frequency for SSB based L1 measurements.

Proposal 7  Event Triggered Reporting for LTM L1 measurement is to be introduced in section 9.14 and section 9.15.

- References

[1]  R4-2416862  WF on RRM requirements for NR_Mob_Ph4_Part1, Apple.  RAN4 #112bis