# 3GPP TSG-RAN WG4 Meeting #113  		R4-2419107

# Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on RRM requirements for CSI-RS based L1 measurement in R19 LTM

Agenda Item:	7.24.2.2

Document for:	Discussion

Introduction

In RAN4 #112bis, the WF for R19 LTM [1] is approved.

Based on all above information, we provide our views on RRM requirements for CSI-RS based L1 measurement in R19 LTM.

Discussion

<On Known conditions>

In RAN4 #112bis, the pre-requisite measurement for CSI-RS based L1 measurement on LTM candidate cell was discussed as captured in issue 2-4 of [1]. Note that this is also related to issue 2-2 of [1].

- Issue 2-4: whether SSB based L3 measurement is a pre-requisite of CSI-RS based L1 measurement Tentative Agreement during online session: further discuss the two alternatives Alternative 1: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1. Alternative 2: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Key difference between Alt 1 and Alt 2 is that whether SSB-based L1 measurement is a necessary pre-requisite for FR2-1. In TS 38.133 [2], for RLM/BFD/L1-RSRP measurements, beam sweeping factor is 8 for SSB based measurement, and beam sweeping factor is 1 for CSI-RS based measurement with repetition = ‘off’. The only exception is CBD measurement, where beam sweeping factor is 8 for both SSB based measurement and CSI-RS based measurement.

We also note that LTM is mainly for SpCell switch. In R18, RAN4 agreed that L1 measurement is pre-requisite to LTM procedures in FR2-1. In our understanding, for FR2-1, that means the main advantage of LTM is enforcing UE to perform finer-beams-based measurement with less samples before cell switch, so that UE may perform faster cell switch. On the other hand, L3 measurement in FR2 is based on UE rough beams. UE is not always able to perform finer-beam-based measurement with N = 1 on CSI-RS if there is no SSB-based L1 measurement performed at first. On the other hand, CBD-like approach was also discussed in RAN4, but we doubt the benefit for this, since anyway SSB-based L1 measurement is necessary for FR2-1 LTM, from RAN4 requirements perspective. In other word, for FR2-1, based on R18 conclusions, anyway UE may need to support SSB-based L1 measurement, and it is more straight forward to re-use legacy principle for L1 RSRP measurements.

Observation 1  For FR2-1, finer-Rx-beams based L1-RSRP measurement is one of the main benefits for LTM according to R18 discussion. Considering UE anyway needs to support SSB-based L1-RSRP measurement in FR2-1 to support LTM, the benefit for further considering a new UE behaviour, i.e. UE performs CSI-RS-based L1-RSRP measurement with finer-Rx-beam sweeping (N=8) is not clear.

Proposal 1  For FR2-1, SSB based L1 measurement is the pre-requisite for CSI-RS based L1 measurement on an LTM candidate cell. RRM requirements are only applicable if the CSI-RS of candidate cell is configured with QCL-D to one of the SSBs for L1 measurements.

On the other hand, we also note that in above alternatives, restrictions, such that CSI-RSs are QCL-ed with SSB, are also added for FR1. However, we doubt the necessity of such note. In FR1, cell level timing shall be considered, and if UE does not support R18 2-TA, UE does not need to consider RTD > CP within any cell. Note that RAN1 is also having similar discussion on this issue for the event-triggered case, and we shall wait for RAN1 conclusions.

Another discussion was that whether CSI-RS based L3 measurement can be the pre-requisite for CSI-RS-based L1 measurements. Our understanding is that it is OK to extend the known cell condition for LTM cell switch, so that CSI-RS based L3 reporting can be included in FR1. However, R16 requirements for CSI-RS based L3 measurement also need the pre-requisite of SSB-based L3 measurement on the associated SSBs. Therefore, we see no need to mention CSI-RS based L3 measurements in the known conditions of CSI-RS based L1 measurement

Proposal 2  For FR1, SSB based L3 measurement is the pre-requisite for CSI-RS based L1 measurements on the same candidate cell. CSI-RS based L1 measurements on the same candidate cell can be performed even without QCL-C configuration to the SSBs.

<On Scenarios for CSI-RS based L1 measurement>

In last meeting, whether to also support the case when RTD>CP was discussed. The main concern from network side is that gNBs may not know the actual RTD at UE side, especially for the mobility scenarios.

Therefore, it would be OK to follow the same methodology as R18. RAN4 may define requirements for both RTD>CP and RTD<CP. An optional UE capability shall be introduced for the requirements when RTD<CP.

Proposal 3  Define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP.

For CSI-RS based L1 measurement, also similar methodology can be followed. An optional UE capability can be introduced for CSI-RS based L1 measurement outside active BWP, i.e. within gaps, if needed.

However, even we see the scenario of CSI-RS based L1 measurement performed outside active BWP, e.g. in another band that is different from current serving cells, we still doubt the benefit of such huge specification works. Here is our analysis based existing R19 RAN1/2 agreements.

Agreements in RAN1 #118 - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting - For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell Agreements in RAN1 #118bis - The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported - From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement Agreements in RAN2 #127bis - For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

In R18 LTM, NZP CSI-RS resources can be configured for candidate cell. Existing TS 38.331 [3] is as follows.

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

}

LTM-TCI-Info-r18 ::=             SEQUENCE {

ltm-DL-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18

OPTIONAL,    -- Need N

ltm-DL-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF TCI-StateId

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToAddModList-r18               SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToReleaseList-r18              SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF TCI-UL-StateId-r17

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToAddModList-r18        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r18     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,    -- Need N

pathlossReferenceRS-ToAddModList-r18           SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17

OPTIONAL,    -- Need N

pathlossReferenceRS-ToReleaseList-r18          SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17

OPTIONAL,    -- Need N

unifiedTCI-StateType-r18                       ENUMERATED {separate, joint}                          OPTIONAL,    -- Need R

...

}

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]]

}

CSI-RS-ResourceMapping ::=          SEQUENCE {

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12)),

row4                                BIT STRING (SIZE (3)),

other                               BIT STRING (SIZE (6))

},

nrofPorts                           ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

firstOFDMSymbolInTimeDomain2        INTEGER (2..12)                                                         OPTIONAL,   -- Need R

cdm-Type                            ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

density                             CHOICE {

dot5                                ENUMERATED {evenPRBs, oddPRBs},

one                                 NULL,

three                               NULL,

spare                               NULL

},

freqBand                            CSI-FrequencyOccupation,

...

}

CSI-FrequencyOccupation ::=         SEQUENCE {

startingRB                          INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs                             INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

...

}

It is clear that R18 NZP CSI-RS can be configured for LTM candidate cells, while the point A of the NZP CSI-RS resource is configured per-resource. In other word, UE may not be able to know the actual channel bandwidth or the BWP where the NZP CSI-RS for LTM locates, based on the candidate cell configuration outside ltm-CandidateConfig-r18, i.e. configurations that UE can obtain before ASN.1 decoding of candidate cell. Since configuration signalling of the frequency domain position and RBs of the NZP CSI-RS for LTM follows R15 methods, UE may obtain the frequency location of candidate cell CSI-RS directly based on point A, SCS, startingRB and nrofRBs, without knowing the channel bandwidth or the BWP location.

R18 NZP CSI-RS for LTM is configured for the purpose of providing candidate cell TCI configuration. Normally the QCL-D source CSI-RS of PDCCH TCI is used for L1-RSRP measurements. Hence, it is straight forward that R18 NZP CSI-RS for LTM is re-used for the configuration of R19 CSI-RS for beam management, which is already agreed in RAN1.

Observation 2  RAN1 and RAN2 have agreed to re-use R18 CSI resource configuration as much as possible for R19 LTM, and candidate cell CSI-RS for beam management shall be used, at least, for CSI-RS based L1 measurement. In legacy, CSI-RS for beam management is also configured based on NZP CSI-RS configuration, and R18 NZP CSI-RS configuration already supports the case of candidate cell in LTM.

Observation 3  Frequency location of R18 NZP CSI-RS for LTM candidate cell is derived based on pointA, SCS, startingRB and nrofRBs that are per-resource configured, and RAN1/2 have agreed not to restrict frequency location of the CSI-RS from their perspective.

In legacy, UE can be configured with more than one BWPs for any active serving cells. In each BWP, the CSI-RSs for TRS, and the CSI-RS for L1-RSRP reporting can be specifically configured. Each CSI resource configuration is associated to a BWP-Id. Therefore, for serving cell CSI-RS based L1 measurement the used CSI-RS resource sets may change due to active BWP change, and it is much easier to ensure all configured CSI-RSs within the active BWP.

On the other hand, if the CSI-RS for BM configured in each cell is also used for LTM purpose, i.e. configured to neighbour cell UE, and if such information is shared between gNBs on different NR bands, e.g. an gNB on band n1 configures UE to perform CSI-RS based L1 measurement on band n78, the UE may have no any active BWP that can cover such CSI-RS. It is unclear what kind of channel bandwidth UE shall assume for such kind of L1 measurement. Without a worst-case assumption on RF filtering for the ‘inter-band’ case, it is difficult to discuss how UE can meet the legacy accuracy requirements. Note that such issue is not only for the ‘inter-band’ case. At least if the CSI-RS from an LTM candidate cell is completely outside any configured BWP(s) of the UE, such issue would happen.

The similar issue was also discussed in R18, and companies thought that UE can set up an ‘initial BWP’ for itself to perform L1 measurements on SSBs outside active BWPs. However, for CSI-RS, it is unclear how wide such BWP would be. Note that as discussed in previous proposal, to ensure mobility performance, a BWP shall not be too wide for multi-FFT based approach, and UE would need to ensure mobility performance under RTD>CP case based on multi-FFT

Observation 4  For CSI-RS based L1 measurement, if the CSI-RS is completely outside all configured BWPs of the UE, and if there is no any restriction in frequency domain configuration on such CSI-RS, UE may not know how wide UE shall set its RF filtering bandwidth, so as to efficiently perform mobility measurement based on these CSI-RS.

In our view, it would be more difficult for RAN4 to solve this issue in R19 CSI-RS based LTM. This would have quite a lot impact to the spec, e.g. gap scheduling and CSSF for the within gap case. It is quite risky that RAN4 specifies a lot of things but not implemented by any vendors. Hence, we prefer to deprioritize the CSI-RS based LTM L1 measurement outside active BWPs.

Proposal 4  If RAN4 agree to define requirements for both within active BWP and outside active BWP, an optional UE capability for CSI-RSs are outside active BWP shall be introduced. Prefer to deprioritize the case where CSI-RS for LTM L1 measurements are outside active BWP(s).

Another issue would be how to define the case of within/outside active BWP of the UE. As discussed above, for serving cell, different CSI-RSs can be configured to different UE BWPs. However, this may not be the case for LTM candidate cell. As in Figure 1, Based on legacy definition, all cells in Figure 1 shall be intra-frequency cells, since their SSB frequency are the same. However, if UE connects to Cell C in Figure 1, based on legacy definition, CSI-RSs of Cell A and Cell B shall be out of active BWP.

Figure 1  Intra-frequency cells having different channel bandwidth, hence having different CSI-RS w.r.t RBs and center frequency

Therefore, for gNBs that supports more than one active BWPS, it is important for gNB to share one CSI-RS configuration that is more ‘common’ for different kinds of active BWPs on the same SSB frequency layer. In this case, it could be a typical case that the neighbor cell CSI-RS is outside UE active BWP. However, In TS 38.214 v 15.16.0, the following is captured in clause 5.2.2.3.1.

TS 38.214 v15.16.0 Clause 5.2.2.3.1 The bandwidth and initial common resource block (CRB) index of a CSI-RS resource within a BWP, as defined in clause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min⁡(24, NBWPsize).

Figure 2 has provided an illustration for this truncation. According to this truncation, only Case 5 is not measured. However, in our understanding, R15 RRM requirements did not consider this truncation.

Figure 2  Truncation of NZP CSI-RS based on R15 TS 38.214

Observation 5  If a NZP CSI-RS is configured to occupy a bunch of RBs, which are derived based nrofRBs and startingRB, and if the bandwidth of this bunch of RBs is wider than active BWP, UE may perform truncation on the CSI-RS first, based on bandwidth of active BWP, and then perform L1 measurement with the truncated CSI-RS, according to R15 TS 38.214.

Therefore, no matter how to interpret R15 RAN4 requirements, for R19 CSI-RS based LTM, in our view RAN4 shall first clarify the definition of CSI-RS based L1 measurements that is outside active BWP.

Proposal 5  From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM:

- Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP.

- Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE.

- Option 3: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

Based on above discussion, option 3 is slightly preferred, in our view, for R19 LTM.

In last meeting, RAN4 has also discussed about the frequency layer of CSI-RS.

- Issue 2-6: other applicability - Further discuss the following assumption: - RAN4 requirements apply all CSI-RS resources on candidate cells on the same layer are configured with the SCS and CP type. - FFS on other parameters, such as same center frequency, BW, periodicity and etc.

In our understanding, it is quite challenging for the network to ensure both of the below points:

- All the cells having the same SSB frequency shares the same CSI-RS configuration in frequency domain.

- Any two cells having different SSB frequency do not share the same CSI-RS configuration in frequency domain.

For example, in Figure 1, Cell 3 shares the same SSB frequency as Cell 1 and Cell 2. However, the center-frequency and the number of RBs are different between CSI-RSs from different cells.

We can also consider another cell, e.g. Cell 4 in the scenario of Figure 1. It configures exactly the same CSI-RS as Cell 1, but SSB frequency of Cell 4 is different from Cell 1. From SSB perspective, Cell 4 and Cell 1 could be inter-frequency cells.

Note that for NZP CSI-RSs, the QCL relation is configured at a per-resource level, not at per-layer level. This is different from CSI-RS based L3 measurements. In legacy CSI-RS based L3 measurements, all resources configured in the same MO shall have the same center frequency, and associated to the same SSB frequency, which is restricted by RAN1/2 specs. However, for R19, RAN1 has already agreed not to have any restriction on the frequency location of CSI-RS.

Therefore, we have the following observation:

Observation 6  If a new definition of CSI-RS frequency layer is to be introduced, it is necessary to restrict that, the corresponding SSBs, which provides timing reference for the CSI-RSs on the same layer, also have the same SSB frequency, so that UE may obtain the time-frequency sync at the same time.

Based on Observation 3, Observation 4 and Observation 6, we have the following proposal.

Proposal 6  From RRM requirement perspective, at least if RRM requirements for CSI-RSs outside active BWP are to be considered, RAN4 needs to clarify the frequency layer of the CSI-RS based L1 measurement.

- The CSI-RSs of the target cells are considered as the same frequency layer if the SSBs of these target cells are of the same SSB frequency, SCS and CP type.

- No RRM requirements are defined, if CSI-RSs on the same frequency layer are configured with the different SCS or CP type.

- Note: No restriction on the center frequency and/or BW of the CSI-RS.

Moreover, based on R18 LTM principle, SSB-based L1 measurement are performed only on the PCell in a candidate cell group. According to the candidate configuration in TS 38.331 below, it is impossible to config SSB frequency of SCell outside the LTM candidate config, i.e. before ASN.1 decoding.

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

Therefore, although pointA of the CSI-RS is per-resource configured, it can only be performed on the candidate PCell. Otherwise the cell shall be unknown. Another point is that, CSI-RS based L1 measurement performed for LTM shall be for PCell switch. For a cell without SSB, clearly it cannot be PCell.

Proposal 7  Although pointA of the CSI-RS is per-resource configured, RAN4 not to consider CSI-RS based L1 measurement on any SCell(s) in any candidate CG in Rel-19.

<On Intra-freq./Inter-freq. Definition>

In last meeting, the following issue was discussed.

- Issue 2-14: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement (already discussed during online session) - Candidate solutions: - Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?]) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs. (Xiaomi) - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy. (CATT) - One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell. - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency. (vivo) - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements. - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. (HW) - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

Although option 1 seems to be supported by majority, as discussed about Figure 1, there could be some issue if RAN4 define intra-frequency only for the case that all CSI-RSs are of the same center frequency. Therefore, similar to Proposal 6, we have the following proposal on intra-f/inter-f definition.

Proposal 8  No need to introduce new definition of intra-frequency or inter-frequency for CSI-RS based L1 measurement. CSI-RS based L1 measurement on one candidate cell is intra-frequency L1 measurement if the SSBs of this candidate cell is of the same SSB frequency, SCS and CP type.

<On UE beam level operation regarding repetition = ‘ON’ or ‘OFF’>

In last meeting, RAN4 has discussed whether repetition = ‘ON’ shall be supported for LTM candidate cells. However, we see similar discussion in RAN1. Actually, this discussion shall be in RAN1 scope. The key points are:

- What are the additional spec impacts if repetition = ‘ON’ is supported for LTM candidate cell, on top existing R18 LTM L1 reporting UCI definition? Note that in this case SSBRI or CRI shall not be reported

- What is the benefit of having such feature? For example, how many UE can be able to support repetition = ‘ON’?

For the 2nd key point, according to TS 38.133, UE needs to try as many as ‘maxNumberRxBeam’, on the number of resources in the resource set. For LTM L1 measurements, this set can contain much more resources than serving cell. Therefore, it would be difficult for LTM L1 measurement to support repetition on.

Proposal 9  Not to support CSI-RS resources with repetition ON for L1 measurement on an LTM candidate cell. The decision shall be made in RAN1.

We are open to further discuss whether RAN4 may send LS to RAN1 for this issue.

For the case repetition OFF, as discussed in Observation 1, UE shall perform L1-RSRP measurement on the QCL-D SSB first in FR2-2. For FR1, UE also needs to perform SSB based L3 measurement on the SSB from the same cell.

Proposal 10  From RRM requirements perspective, for CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1.

<CSI-RS resource type>

RAN1 has agreed the following for the CSI-RS resource type.

Agreements in RAN1 #118 - For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell Agreements in RAN1 #118bis Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability.

RAN1 has not yet confirmed to support semi-persistent CSI-RS, due to the issue of coordination between cells. Moreover, semi-persistent CSI-RS is an optional UE feature for LTM. Therefore, we still prefer to work on periodic CSI-RS first.

Proposal 11  RAN4 prioritize CSI-RS based L1 measurement requirements for periodic CSI-RS.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  For FR2-1, finer-Rx-beams based L1-RSRP measurement is one of the main benefits for LTM according to R18 discussion. Considering UE anyway needs to support SSB-based L1-RSRP measurement in FR2-1 to support LTM, the benefit for further considering a new UE behaviour, i.e. UE performs CSI-RS-based L1-RSRP measurement with finer-Rx-beam sweeping (N=8) is not clear.

Proposal 1  For FR2-1, SSB based L1 measurement is the pre-requisite for CSI-RS based L1 measurement on an LTM candidate cell. RRM requirements are only applicable if the CSI-RS of candidate cell is configured with QCL-D to one of the SSBs for L1 measurements.

Proposal 2  For FR1, SSB based L3 measurement is the pre-requisite for CSI-RS based L1 measurements on the same candidate cell. CSI-RS based L1 measurements on the same candidate cell can be performed even without QCL-C configuration to the SSBs.

Proposal 3  Define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP.

Observation 2  RAN1 and RAN2 have agreed to re-use R18 CSI resource configuration as much as possible for R19 LTM, and candidate cell CSI-RS for beam management shall be used, at least, for CSI-RS based L1 measurement. In legacy, CSI-RS for beam management is also configured based on NZP CSI-RS configuration, and R18 NZP CSI-RS configuration already supports the case of candidate cell in LTM.

Observation 3  Frequency location of R18 NZP CSI-RS for LTM candidate cell is derived based on pointA, SCS, startingRB and nrofRBs that are per-resource configured, and RAN1/2 have agreed not to restrict frequency location of the CSI-RS from their perspective.

Observation 4  For CSI-RS based L1 measurement, if the CSI-RS is completely outside all configured BWPs of the UE, and if there is no any restriction in frequency domain configuration on such CSI-RS, UE may not know how wide UE shall set its RF filtering bandwidth, so as to efficiently perform mobility measurement based on these CSI-RS.

Proposal 4  If RAN4 agree to define requirements for both within active BWP and outside active BWP, an optional UE capability for CSI-RSs are outside active BWP shall be introduced. Prefer to deprioritize the case where CSI-RS for LTM L1 measurements are outside active BWP(s).

Observation 5  If a NZP CSI-RS is configured to occupy a bunch of RBs, which are derived based nrofRBs and startingRB, and if the bandwidth of this bunch of RBs is wider than active BWP, UE may perform truncation on the CSI-RS first, based on bandwidth of active BWP, and then perform L1 measurement with the truncated CSI-RS, according to R15 TS 38.214.

Proposal 5  From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM:

- Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP.

- Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE.

- Option 3: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

Observation 6  If a new definition of CSI-RS frequency layer is to be introduced, it is necessary to restrict the corresponding SSBs, which provides timing reference for the CSI-RSs on the same layer, also have the same SSB frequency, so that UE may obtain the time-frequency sync at the same time.

Proposal 6  From RRM requirement perspective, at least if RRM requirements for CSI-RSs outside active BWP are to be considered, RAN4 needs to clarify the frequency layer of the CSI-RS based L1 measurement.

- The CSI-RSs of the target cells are considered as the same frequency layer if the SSBs of these target cells are of the same SSB frequency, SCS and CP type.

- No RRM requirements are defined, if CSI-RSs on the same frequency layer are configured with the different SCS or CP type.

- Note: No restriction on the center frequency and/or BW of the CSI-RS.

Proposal 7  Although pointA of the CSI-RS is per-resource configured, RAN4 not to consider CSI-RS based L1 measurement on any SCell(s) in any candidate CG in Rel-19.

Proposal 8  No need to introduce new definition of intra-frequency or inter-frequency for CSI-RS based L1 measurement. CSI-RS based L1 measurement on one candidate cell is intra-frequency L1 measurement if the SSBs of this candidate cell is of the same SSB frequency, SCS and CP type.

Proposal 9  Not to support CSI-RS resources with repetition ON for L1 measurement on an LTM candidate cell. The decision shall be made in RAN1.

Proposal 10  From RRM requirements perspective, for CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1.

Proposal 11  RAN4 prioritize CSI-RS based L1 measurement requirements for periodic CSI-RS.

- References

[1]  R4-2416862  WF on RRM requirements for NR_Mob_Ph4_Part1, Apple.  RAN4 #112bis

[2]  TS 38.133 v18.7.0

[3]  TS 38.331 v18.3.0