3GPP TSG RAN WG4 Meeting #113	R4-2419117

Orlando, Florida, USA, 18 Nov. 2024 – 22 Nov. 2024

Agenda Item:	7.4.3

Source: 	Ericsson

Title:	Discussion on the remaining issues of Rel-19 NR CA/DC in less than 5 MHz

Document for:	Discussion

# Introduction

In 3GPP RAN4#112-bis, we agreed on most of the essential issues regarding less than 5 MHz Phase 2 core requirements for NR CA/DC. WF can be found in [1]. Since this is the last meeting for the core part of this WI, the following open issues need to be discussed.

Issue 1-1-3: Whether intra-band CA is in the scope: - Proposals - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19. - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations. - Option 3: No need to discuss since there is no impact to RRM requirements in Rel-19. Issue 1-2-2a: PSCell change delay requirements: - Proposals - Option 1: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell change delay requirements. ## 8.11 PSCell Change This clause defines requirements for the delay within which the UE shall be able to change PSCell to other cell in EN-DC or NR-DC. The requirements in this clause are applicable to EN-DC and NR-DC. In the case of NR-DC PSCell change in FR1, when the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target PSCell that is with 12PRB SSB bandwidth, the UE shall transmit PRACH preamble towards the target PSCell no later than the delay requirements specified in clause 8.9.2.. - Option 2: No need to update the text. Issue 1-4-3: Conditional handover delay requirements to PCell with 12PRB SSB: - Proposals - Option 1: T∆ = 3*Trs for target PCell operating with 12 PRB SSB BW. - Option 2: No changes to Rel-18 requirement.

This paper further discusses the open issues and detailed specifications needed for the agreements.

# Discussion

## Scope of Intra-Band CA

According to Release 19 WID [2], intra-band carrier aggregation (CA) is excluded from the scope of the less-than-5 MHz CA/DC WI, as most 3 MHz bands are too narrow for it to be relevant. During the RAN4#112 and RNA4#112-bis meeting, we discussed three options:

o	Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19.

o	Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations.

o	Option 3: No need to discuss since there is no impact to RRM requirements in Rel-19.

To our understanding, some operators are interested in this feature, but the RF room has not discussed it in depth. If no further efforts are needed, we can proceed with Option 1, revising and approving the WID accordingly. Additionally, considering the time constraint, contiguous and non-contiguous intra-band CA should be considered as a second priority. In other words, after completing the inter-band work, if time permits, we can specify contiguous and non-contiguous intra-band CA.

RAN4 should set contiguous and non-contiguous intra-band CA as a second priority after completing the inter-band work; if time permits, we can specify contiguous and non-contiguous intra-band CA provided that it is also supported in RF.

## PSCell Change Delay Requirements

As the impact of less than 5MHz is already agreed to be captured in clause 8.9.2 of TS 38.133, we do not need to Specify any text in clause 8.11. The UE can get information of the 12PRB SSB bandwidth during the configuration of PSCell. Hence,

No need to update the text of PSCell change delay requirements.

## Conditional Handover Delay Requirements

Similar to NR handover, NR conditional handover (CHO) also aims to change the NR PCell to another NR cell. In Rel-18, we considered Rel-15 as a baseline. Therefore, the CA/DC requirements introduced in Rel-16 are not considered in the less than 5MHz WI for Rel-18. CHO was also not considered in Rel-18 as well. In Rel-19, RAN4 should consider and define CHO requirements for less than 5 MHz.

RAN4 should define conditional handover requirements to support NR CA/DC for less than 5 MHz.

Furthermore, as specified in clause 6.1.1.2.2 for Rel-18, interruption time for fine time tracking and acquiring full timing information of the target cell T∆ should be specified as:

T∆ is time for fine time tracking and acquiring full timing information of the target cell. Where,

T∆ = Trs for target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

RAN4 to capture the following text for NR Conditional Handover.

- T∆ is time for fine time tracking and acquiring full timing information of the target cell. Where,

- T∆ = Trs for target cells operating with 20 PRB SSB BW.

- T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

## NW Indication on SCell Addition During Handover

It is agreed on the RAN4#112 RF session that no new signalling from the network to inform UE of the number of PRB transmissions is required. Furthermore, in the RAN4#112-bis meeting, the RF session agreed that the SCell SSB block with 12 PRB Bandwidth should be only on sync raster.

PRB-related information can already be obtained from the existing signalling during handover. Hence, we believe that RAN4 RRM should not introduce new network signalling on PRB transmission in alignment with RF.

RAN4 RF discussed and concluded not to introduce new network signalling on PRB transmission.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 aspects for NR CA/DC less than 5 MHz and made the following observations and proposals.

Observation 1	RAN4 RF discussed and concluded not to introduce new network signalling on PRB transmission.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 should set contiguous and non-contiguous intra-band CA as a second priority after completing the inter-band work; if time permits, we can specify contiguous and non-contiguous intra-band CA provided that it is also supported in RF.

Proposal 2	No need to update the text of PSCell change delay requirements.

Proposal 3	RAN4 should define conditional handover requirements to support NR CA/DC for less than 5 MHz.

Proposal 4	RAN4 to capture the following text for NR Conditional Handover.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. Where,

T∆ = Trs for target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

# References

- R4- 2416849, WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2, Intel

- RP- 241622 “Revised WID: NR channel BW less than 5MHz for FR1 Phase 2”, Intel

- 3GPP TS 38.133: "NR; Requirements for support of radio resource management"