3GPP TSG-RAN4 Meeting #113	R4-2419136

# Orlando, US, 18 – 22 November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on PRS RMC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The PRB number in PRS RMC for FH is not aligned with the total BW after FH in the accuracy requirement. - There is a redundant table in clause A.3.31.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the PRB number in PRS RMC for FH to be aligned with the total BW after FH in the accuracy requirement. - Remove the redundant table in clause A.3.31.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Accuracy requirements for RedCap UE are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on revised version of big draftCR R4-2416892 from RAN4 reflector after RAN4#112-bis |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## A.3.31	PRS Configurations

### A.3.31.1	PRS Configurations for FR1

#### A.3.31.1.1	PRS pattern 1 in FR1: SCS=15 kHz

Table A.3.31.1.1-1: PRS.1 FR1: PRS Pattern 1 for SCS=15 kHz

| PRS Parameters | Values |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.3 FR1 |  | PRS.1.4 FR1 |  | PRS.1.5 FR1 |
| Resource index in resource set | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| PRS periodicity | 160ms |  |  |  |  |  |  |
| PRS Resource set slot offset Note 1 | 10 ms |  |  |  |  |  |  |
| PRS Resource slot offset (slot) Note 1 | 0 | 4 | 0 |  | 4 |  | 4 |
| PRS RE offset Note 1 | 0 |  | 0 | 1 | 0 | 1 | 1 |
| SCS | 15kHz |  |  |  |  |  |  |
| PRS comb size | 2 | 4 | 2 |  | 4 |  | 4 |
| Number of PRS symbol | 4 | 4 | 4 |  | 4 |  | 4 |
| Repetion factor | 2 | 1 | 2 |  | 1 |  | 6 |
| PRS resource time gap (slot) | 1 | 1 | 1 |  | 1 |  | 1 |
| PRB numbers containing PRS within channel BW Note 1 | 0-23 | 0-103 | 0-23 |  | 0-103 |  | 0-267 |
| PRS Start PRB | 0 |  |  |  |  |  |  |
| NOTE 1: Unless otherwise specified in the test case. |  |  |  |  |  |  |  |

#### A.3.31.1.2	PRS pattern 2 in FR1: SCS=30 kHz

Table A.3.31.1.2-1: PRS.2 FR1: PRS Pattern 2 for SCS=30 kHz

| PRS Parameters | Values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel | PRS.2.1 FR1 | PRS.2.2 FR1 | PRS.2.3 FR1 |  | PRS.2.4 FR1 |  | PRS.2.5 FR1 | PRS.2.6 FR1 |
| Resource index in resource set | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| PRS periodicity | 160 ms |  |  |  |  |  |  |  |
| PRS Resource set slot offset Note 1 | 10 ms |  |  |  |  |  |  |  |
| PRS Resource slot offset (slot) Note 1 | 0 | 4 | 0 |  | 4 |  | 0 | 0 |
| PRS RE offset Note 1 | 0 |  | 0 | 1 | 0 | 1 | 0 | 0 |
| SCS | 30 kHz |  |  |  |  |  |  |  |
| PRS comb size | 2 | 4 | 2 |  | 4 |  | 4 | 4 |
| Number of PRS symbol | 4 | 4 | 4 |  | 4 |  | 4 | 4 |
| Repetion factor | 2 | 1 | 2 |  | 1 |  | 1 | 6 |
| PRS resource time gap (slot) | 1 | 1 | 1 |  | 1 |  | 1 | 1 |
| PRB numbers containing PRS within channel BW Note 1 | 0-23 | 0-131 | 0-23 |  | 0-131 |  | 0-47 | 0-271 |
| PRS Start PRB | 0 |  |  |  |  |  |  |  |
| NOTE 1: Unless otherwise specified in the test case. |  |  |  |  |  |  |  |  |

### A.3.31.2	PRS Configurations for FR2

#### A.3.31.2.1	PRS pattern 1 in FR2: SCS=120 kHz

Table A.3.31.2.1-1: PRS.1 FR2: PRS Pattern 1 for SCS=120 kHz

| PRS Parameters | Values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel | PRS.1.1 FR2 | PRS.1.2 FR2 | PRS.1.3 FR2 |  | PRS.1.4 FR2 |  | PRS.1.5 FR2 | PRS.1.6 FR2 |
| Resource index in resource set | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| PRS periodicity | 160 ms |  |  |  |  |  |  |  |
| PRS Resource set slot offset Note 1 | 10 ms |  |  |  |  |  |  |  |
| PRS Resource slot offset (slot) Note 1 | 0 | 4 | 0 |  | 4 |  | 0 | 0 |
| PRS RE offset Note 1 | 0 |  | 0 | 1 | 0 | 1 | 0 | 0 |
| SCS | 120 kHz |  |  |  |  |  |  |  |
| PRS comb size | 2 | 4 | 2 |  | 4 |  | 4 | 4 |
| Number of PRS symbol | 4 | 4 | 4 |  | 4 |  | 4 | 4 |
| Repetion factor | 2 | 1 | 2 |  | 1 |  | 1 | 6 |
| PRS resource time gap (slot) | 1 | 1 | 1 |  | 1 |  | 1 | 1 |
| PRB numbers containing PRS within channel BW Note 1 | 0-31 | 0-127 | 0-31 |  | 0-127 |  | 0-63 | 0-263 |
| PRS Start PRB | 0 |  |  |  |  |  |  |  |
| NOTE 1: Unless otherwise specified in the test case. |  |  |  |  |  |  |  |  |

<End of Change 1>