3GPP TSG-RAN WG4 Meeting #113	R4-2419137

Orlando, US, 18 – 22 November, 2024

Title: 	Discussion on performance requirements for SL positioning

Source: 	Huawei, HiSilicon

Agenda item:	5.22.2.2

Document for:	Discussion

Introduction

RRM performance requirements for SL positioning are discussed in RAN4#112-bis, and the outcomes are captured in [1]. Based on the WF, RAN4 need to further discuss how to multiplex TX UEs in the TC.

In this paper, we will provide our views on remaining issues in performance requirements for SL positioning.

Discussion

### Issue 1-2-5: Multiplexing of TX UEs in TCs Discuss further: - For 2 transmitting UEs, the time offset between the two SL-PRS configurations can be, e.g.: - MUX of multiple TX UEs: TX UE1: slot n, TX UE2: slot n + 1 and slot n + 160ms. - Time offset for 3 transmitting anchor UEs needs to be also decided. Come back next meeting.

SL PRS measurement period depends on following UE capabilities:

- Number of active resources per slot (component 2 of FG 41-1-1)

2. Maximum number of active SL PRS resources across all configured RPs in a slot assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE Component 2 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64, 128} for each SCS: 60kHz, 120kHz

- Number of active slots (component 3 of FG 41-1-1)

3. Maximum number of slots with active SL PRS resources across all configured RPs assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE Component 3 candidate values: FR1: {1, 2, 3, 4, 6, 8} FR2: {1, 2, 4, 8, 12, 16, 24, 32, 48, 64}

- Processing time (component 4 of FG 41-1-1)

4. Minimum time after the end of a slot carrying the active SL-PRS resource(s) assuming maximum number of symbols and maximum bandwidth for a UE to finish the SL-PRS resource and the associated PSCCH processing which is supported and reported by UE Component 4 candidate values: {20ms, 30ms, 40ms, 50ms, 80ms, 100ms, 160ms}

In our view, the TC should verify UE performance considering at least component 3 and 4. For component 2, having 2 TX UEs in the same slot will complicate the test setup, e.g. to multiplex multiple PSCCH in the same slot, and we understand it may not be a typical case in real deployment, especially for shared resource pool where different TX UEs in one slot can only be FDM-ed on sub-channel level. Since the TCs are now defined agnostic to resource pool type, we suggest to consider only the case with one TX UE per slot.

For TCs with 2 TX UEs, it is straightforward how to account for component 3 and 4

- If UE supports at least 2 active slots, then the 2 TX UEs can be in consecutive SL slots, and we can verify that UE can process 2 active slots at same time.

- If UE supports only 1 active slot, then the 2nd TX UE should be in the next SL slot immediately after UE finishes processing the 1st TX UE, based on component 4.

For TCs with 2 TX UEs, following the same logic,

- If UE supports more than 2 active slots, then the 3 TX UEs can be in consecutive SL slots, and we can verify that UE can process 3 active slots at same time.

- If UE supports 2 active slots, then the first 2 TX UEs can be in consecutive SL slots, and the 3rd TX UE should be in the next SL slot immediately after UE finishes processing the 1st TX UE

- If UE supports only 1 active slot, then the 2nd TX UE should be in the next SL slot immediately after UE finishes processing the 1st TX UE, and the 3rd Tx UE should be in the next SL slot immediately after UE finishes processing the 2nd TX UE.

Proposal: RAN4 to adopt the following setup for multiplexing of TX UEs in SL positioning TCs

- For TCs with 2 TX UEs

  - If UE supports at least 2 active slots, the 2 TX UEs are in consecutive SL slots.

  - If UE supports only 1 active slot, the 2nd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE.

- For TCs with 2 TX UEs, following the same logic,

  - If UE supports more than 2 active slots, the 3 TX UEs can be in consecutive SL slots.

  - If UE supports 2 active slots, the first 2 TX UEs are in consecutive SL slots, and the 3rd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE

  - If UE supports only 1 active slot, the 2nd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE, and the 3rd Tx UE is in the next SL slot immediately after UE finishes processing the 2nd TX UE.

Conclusions

In this paper we provided our views on remaining issues in performance requirements for SL positioning.

Proposal: RAN4 to adopt the following setup for multiplexing of TX UEs in SL positioning TCs

- For TCs with 2 TX UEs

  - If UE supports at least 2 active slots, the 2 TX UEs are in consecutive SL slots.

  - If UE supports only 1 active slot, the 2nd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE.

- For TCs with 2 TX UEs, following the same logic,

  - If UE supports more than 2 active slots, the 3 TX UEs can be in consecutive SL slots.

  - If UE supports 2 active slots, the first 2 TX UEs are in consecutive SL slots, and the 3rd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE

  - If UE supports only 1 active slot, the 2nd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE, and the 3rd Tx UE is in the next SL slot immediately after UE finishes processing the 2nd TX UE.

Reference

- R4-2416870, WF on RRM requirements for NR_pos_enh2_Part2, CATT