3GPP TSG-RAN WG4 Meeting #113	R4-2419142

Orlando, US, 18 – 22 November, 2024

Title: 	Discussion on RRM requirements for less than 5MHz Ph2

Source: 	Huawei, HiSilicon

Agenda item:	7.4.3

Document for:	Discussion

Introduction

RRM requirements Rel-19 less than 5MHz are discussed in RAN4#112-bis, and the outcomes are captured in WF [1]. Based on the discussion, most issues were resolved, and a few needs to be further discussed.

- Intra-band CA

- PSCell related requirements

- Requirement applicability

In this paper, we will provide our views on RRM requirements for Rel-19 less than 5MHz.

Discussion

Issue 1-1-3: Whether intra-band CA is in the scope: - Proposals - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19. - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations. - Option 3: No need to discuss since there is no impact to RRM requirements in Rel-19.

We support option 3. The only technical impact identified so far is whether requirements for the case where the SCell being activated is intra-band contiguous to an active cell apply or not. Technically, we understand only inter-band CA is supported in the WI, so the requirements would not apply for SCell with less than 5MHz. However, we do not think this applicability needs to be captured in spec because for SCell with less than 5MHz, the case the requirements are concerned with would not happen, and the requirements would not apply by default. Therefore, this issue has no spec change.

Proposal 1: No spec impact is needed regarding no support of intra-band CA in the WI.

Issue 1-2-2a: PSCell change delay requirements: - Proposals - Option 1: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell change delay requirements. ## 8.11 PSCell Change This clause defines requirements for the delay within which the UE shall be able to change PSCell to other cell in EN-DC or NR-DC. The requirements in this clause are applicable to EN-DC and NR-DC. In the case of NR-DC PSCell change in FR1, when the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target PSCell that is with 12PRB SSB bandwidth, the UE shall transmit PRACH preamble towards the target PSCell no later than the delay requirements specified in clause 8.9.2.. - Option 2: No need to update the text. Issue 1-2-3a: Handover with PSCell: - Proposals - Option 1: SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. - Option 2: The HO requirements with PSCell-PCell and PSCell-PSCell can be reused for NR CA/DC in less than 5MHz.

Some PSCell related requirements are defined by referring to other requirements. For example, PSCell change requirements in clause 8.11 are defined by referring to SCell addition requirements in clause 8.9.2. For such cases, our view is that no spec impact is needed because the requirements in clause 8.9.2 already account less than 5MHz. We do not support the additional texts as in option 1 of Issue 1-2-2a.

The same principle applies also for HO with PSCell. The requirements are defined by referring to HO and PSCell addition requirements, and no spec impact is needed.

Proposal 2: No spec impact is needed for PSCell change and Handover with PSCell requirements.

Issue 1-4-2: Requirements applicability: - Proposals - Proposal 1: Even though UE capability indicates to support fast SCell activation or inter-band SSB-less SCell activation, no requirement shall be applied for fast SCell activation or inter-band SSB-less SCell activation when the target SCell BW is less than 5MHz, i.e., UE capability of fast SCell activation and inter-band SSB-less SCell activation only applies to SCell on 5MHz and larger BW. - Proposal 2: SCell activation requirement shall not be applied when CSI-RS is configured for L1-RSRP measurement/report for this being-activated SCell Agreement: SCell activation requirement specified in Rel-19 shall not be applied when CSI-RS is configured for L1-RSRP measurement/report for the target SCell.

In our view, the requirements for fast SCell activation or inter-band SSB-less SCell activation should apply for SCell with less than 5MHz. We agree that there are no requirements for CSI-RS based L1 or L3 measurement, but here CSI-RS is used as TRS for time tracking but not for measurement. As using 12 RB TRS for time tracking is anyway needed for normal operation on PCell or active SCell, including both periodic and aperiodic TRS, we do not see an issue to use 12 RB aperiodic TRS during SCell activation. Therefore, we understand current requirements for fast SCell activation or inter-band SSB-less SCell activation also apply for SCell with less than 5MHz, and no spec change is needed.

Proposal 3: No spec impact is needed for fast SCell activation or inter-band SSB-less SCell activation, i.e. current requirements apply for SCell with less than 5MHz.

We also would like to clarify the agreement from last meeting related to CSI-RS is configured for L1-RSRP measurement/report. There is a case where both SSB and CSI-RS are configured for L1-RSRP for the target SCell, and in our view, the SCell activation requirements should still apply in this case because SSB based L1-RSRP measurement and report should still work. Therefore, we suggest to clarify the previous agreement as in Proposal 4.

Proposal 4: Clarify the previous agreement as

SCell activation requirement specified in Rel-19 shall not be applied when only CSI-RS is configured for L1-RSRP measurement/report for the target SCell.

Conclusions

In this paper we provided provide our views on RRM requirements for Rel-19 less than 5MHz.

Proposal 1: No spec impact is needed regarding no support of intra-band CA in the WI.

Proposal 2: No spec impact is needed for PSCell change and Handover with PSCell requirements.

Proposal 3: No spec impact is needed for fast SCell activation or inter-band SSB-less SCell activation, i.e. current requirements apply for SCell with less than 5MHz.

Proposal 4: Clarify the previous agreement as

SCell activation requirement specified in Rel-19 shall not be applied when only CSI-RS is configured for L1-RSRP measurement/report for the target SCell.

Reference

- R4-2416849, WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2, Intel Corporation