3GPP TSG-RAN WG4 Meeting #113	R4-2419145

Orlando, US, 18 – 22 November, 2024

Title: 	Simulation assumption and additional simulation results for LP-WUR measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.23.4.1

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#112, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to evaluate the link level performance of LP-WUR measurement, and part of simulation assumption is agreed as listed in [1].

In this paper, we will provide our views on the simulation assumption and also provide initial simulation results for LP-WUR measurement.

Discussion

Simulation assumption

Issue 2-1-1: SINR setting

Agreement: - As starting point for RAN4 RRM simulation purpose only, -3dB, 0.5dB, 2dB dB Ês/Iot value is used for serving cell for both OOK based and OFDM based LP-WUR. - The SINR value can be updated if any update on the noise figure value in RF session. - FFS the side condition for RAN4 requirement.

Issue 2-1-1-2: Same or different SINR setting for OOK based and OFDM based LP-WUR

Recommendations: Covered by issue 2-1-1. Close this issue

In our view, RAN4 should use -3dB SINR as side condition of accuracy requirements for LR as baseline. The values provided by RAN1 are for different NF assumptions, and we believe RAN4 should take the worst case assumption for defining requirements, which corresponds to -3dB SINR.

We understand RF session may take different assumptions for OOK based and OFDM based LR for defining the refsense requirements, but it does not mean RRM requirements should be defined with different side conditions. In particular, if we can assume NF for OFDM based LR will be smaller than that for OOK based LR, we can have 2 options:

- Option 1: define side condition for OOK based LR as -3dB and for OFDM based LR as -0.5dB

- Option 2: define side condition for OOK and OFDM based LR as -3dB

Option 1 assumes the coverage of both LRs is same, so OFDM based LR is only required to work in a better SINR side condition than OOK based LR. However, OFDM based LR is supposed to be more powerful than OOK based LR, and we should not relax its requirements for baseband measurement performance just because it has a better NF. Instead, it is more reasonable to assume OFDM based LR is able to work in the same SINR side condition as OOK based LR as in option 2, and the smaller NF can translate into better coverage, i.e. the feature can be more widely used in a cell.

Proposal 1: As baseline, RAN4 to use -3dB SINR as side condition of accuracy requirements for both OOK based and OFDM based LR.

Initial simulation results

In this section, we will provide our simulation results based on the simulation assumption in [1]. All the results are for TDL-C channel.

The parameters selected for simulations are as follows.

- Serving cell SINR: -3dB

  - Serving cell SNR = -2.4dB, interfering cell SNR = -8.4dB, i.e. 6dB power difference

- Interference type: LP-SS or SSB, interfering cell is fully synchronous to serving cell

- Sample number: 1

- Sampling rate: 7.68MHz

- ADC bit width: 8bit

- Filter: 3th Order Butterworth with 3.96MHz bandwidth

LP-SS performance

The results in this section are for OOK based LR, i.e. without utilizing the overlaid sequence on LP-SS. The parameters related to LP-SS used in the simulation are as follows.

- Number of OFDM symbols: 8

- OOK scheme: OOK-4 with M=4

- Binary sequence: length-16 M sequence with Manchester coding

Table 1: simulation results for LP-SS measurement

| AWGN | Sample | Max(5%,95%) |
| --- | --- | --- |
| LP-RSRP | 1 | 4.08 |
| LP-RSRQ |  | 4.13 |

SSS performance

The results in this section are for OFDM based LR.

Table 2: simulation results for SSB measurement

| AWGN | Sample | Max(5%,95%) |
| --- | --- | --- |
| SS-RSRP | 1 | 3.60 |
| SS-RSRQ |  | 3.85 |

Conclusions

In this paper we provided provide our views on the simulation assumption and also provide initial simulation results for LP-WUR measurement.

Proposal 1: As baseline, RAN4 to use -3dB SINR as side condition of accuracy requirements for both OOK based and OFDM based LR.

The simulation results show that

- For LP-SS, the measurement accuracy with the assumed LP-SS pattern cannot meet the target accuracy of 3.5 dB with single sample in TDL-C channel at -3dB SINR.

- For SSB, the measurement accuracy cannot meet the target accuracy of 3.5 dB with single sample in TDL-C channel at -3dB SINR.

Reference

- R4-2416861, WF on RRM requirements for NR_LPWUS, Moderator (vivo)