3GPP TSG-RAN WG4 Meeting #113	R4-2419146

Orlando, US, 18 – 22 November, 2024

Title: 	Discussion on RRM requirements for LP-WUR

Source: 	Huawei, HiSilicon

Agenda item:	7.23.4.2

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#112-bis, and the outcomes are captured in WF [1]. Based on [1] the following issues need further discussion.

- General aspects

- LP-WUR measurement

- MR relaxation

- LP-WUR at CONNECTED mode

In this paper, we will provide our views on RRM requirements related to LP-WUR.

Discussion

General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

We do not support to consider Case #2.

- There is no clear necessity to require MR to measure the serving cell since LR is already measuring the serving cell.

  - In particular, the MR measurement will be with relaxation, so it is much more infrequent compared to the LR measurement.

  - Some companies commented that LR measurement may be less accurate than that of MR. In our view, however, it is both necessary and feasible to keep similar accuracy between LR and MR.

  - Some companies commented that MR measurement may be used by UE to assist LR measurement. We believe this can be left to UE implementation rather than mandated by requirements.

- There is additional power consumption due to MR measurement even with relaxation.

- It will complicate the spec since another threshold (coverage level) is needed to switch between Case #1 and Case #2.

Proposal 1: RAN4 not to support Case #2 for MR and LR measurement.

Issue 1-1-2: Measurement requirements to be specified for LP-WUR

RAN1#118-bis reached following agreements related to LP-SS.

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

Measurement of LP-SS by OFDM based LR is clearly supported according to the RAN1 agreement, so we support to define following requirements for different LR architectures.

- OOK based LR measurement on LP-SS

- OFDM based LR measurement on LP-SS

- OFDM based LR measurement on SSB

OFDM based LR can measure both LP-SS and SSB, so if only one of 2) and 3) is defined, it means that UE with OFDM based LR is mandated to measure a certain signal, which unnecessarily limits the flexibility of OFDM based LR.

One may ask if OFDM based LR can measure SSB, why does it need to measure LP-SS. In our view, LP-SS can be assumed at the same center frequency as LP-WUS, and it is very likely to be outside the bandwidth of initial DL BWP containing SSB, in order to avoid reducing the scheduling resources for common channels. Using LP-SS allows OFDM based LR to perform sync and RRM measurement without RF re-tuning. Also, the measurement from LP-SS can better reflect the channel quality for LP-WUS reception compared to measurement from SSB which may be far away from LP-WUS in frequency domain.

Based on above discussions, we suggest RAN4 to define requirements for 2). From the RAN1 agreement, the design of overlaid sequence(s) doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS, so it is reasonable to assume same requirements for LP-SS based measurement by OOK based LR and OFDM based LR. This will also help to save the RAN4 workload to introduce another set of requirements specific to overlaid sequences.

Proposal 2: RAN4 to define requirements for OFDM based LR measurement based on LP-SS, by re-using the requirements for OOK based LR measurement based on LP-SS.

Issue 1-1-4: Threshold for switch between case 1 and case 3

Since RAN4 agreed to support Case #1 and #3, we need to discuss how UE switches between the two cases. In our view, the switch depends on whether neighbour cell measurement is triggered. Currently, start or stop of neighbour cell measurement is based on evaluation of serving cell against threshold SIntraSearchP, SIntraSearchQ, SnonIntraSearchP or SnonIntraSearchQ, which are thresholds for MR measurement.

For Case #1 to Case #3, only LR measures serving cell in Case #1, so LR measurement should be used to evaluate whether the neighbour cell measurement start criterion is met. LR measurement based on SSB is directly comparable against the existing thresholds, but LR measurement based on LP-SS is not. One option is to define new threshold for LP-RSRP and LP-RSRQ, and another option is to use the existing threshold with an offset. We suggest RAN4 to further discuss the two options.

Proposal 3: RAN4 to discuss following options for switch from Case #1 to Case #3 based on LR measurement.

- Option 1: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ

- Option 2: Use existing thresholds for SS-RSRP and SS-RSRQ, and define offsets for LP-RSRP and LP-RSRQ

For Case #3 to Case #1, both LR and MR measure serving cell in Case #3, so RAN4 needs to further discuss which measurement(s) is used for the evaluation. In our view, LR measurement should be at least considered because it is much more frequent than MR. On the other hand, stopping neighbour cell measurement is not time critical from mobility perspective, so MR measurement can also be included. In this case, UE should stop neighbor cell measurement only when both measurements are below their corresponding thresholds, as otherwise it may cause ping-pong. We suggest RAN4 to further discuss the following options.

Proposal 4: RAN4 to discuss following options for switch from Case #3 to Case #1 based on LR or both LR and MR measurements.

- Option 1: LR measurement only

- Option 2: both LR and MR measurements, and UE stops neighbor cell measurement when both measurements are below their corresponding thresholds

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

In RAN2#126, the following agreement was reached regarding serving cell RRM measurement offloading.

- For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

RAN2 will focus on specifying the offloading criterion. To avoid duplicated discussion, we suggest RAN4 not to further discuss the entry/exit condition for serving cell RRM measurement offloading to LR, unless triggered by other WGs.

In RAN2#126, the following agreement was reached regarding MR measurement relaxation.

- RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

RAN2 will further discuss the neighbour cell measurement relaxation criteria. To avoid duplicated discussion, we suggest RAN4 not to further discuss the entry/exit condition for MR RRM measurement relaxation, unless triggered by other WGs.

Proposal 5: RAN4 not to further discuss the entry/exit condition for measurement offloading to LR or MR RRM measurement relaxation, unless triggered by other WGs.

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

RAN1/2 has agreed on the baseline for the entry/exit condition for LP-WUS monitoring, and it is based on serving cell quality, i.e. RSRP and RSRQ. In our view, serving cell quality mainly reflects the coverage level, and another factor that needs to considered for determining whether UE should enter or exit LP-WUS monitoring is the inter-carrier interference level.

As the dynamic range of LR is smaller compared to MR, e.g. the bitmap for ADC is assumed to be 4 or 8 bits, it is more likely to get blocked by the inter-carrier interference. The issue was discussed in RF session, where the conclusion was that “RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR”.

In our view, interference level that LR can tolerate is smaller than that for MR. On the other hand, the actual interference level does not change depending on whether UE is using LR or MR in another carrier which may be used by another operator. Therefore, even UE is close to the serving gNB, i.e. the serving cell RSRP and RSRQ are high, the LR may not be usable because of high inter-carrier interference. In this case, the UE should not use LR as the LP-WUS detection performance cannot be guaranteed.

We understand the key question is whether RSRP and RSRQ measurement from LR can indicates the inter-carrier interference level. If so, we may not need additional entry/exit condition; if not, additional entry/exit condition on inter-carrier interference level besides the serving cell quality should be considered.

Proposal 6: For LP-WUS monitoring, RAN4 to discuss whether RSRP and RSRQ measurement from LR can reflect the impact of the inter-carrier interference.

Issue 1-1-7: On jointly consideration on issue 1-1-5, 1-1-6 and 1-1-7

In our view, the exact entry/exit conditions for LP-WUS monitoring, serving cell RRM measurement offloading and MR RRM measurement relaxation are to be discussed in RAN2. If RAN4 is expected to provide some high-level suggestion to RAN2, we suggest that the 3 events are based on same entry/exit condition.

- This will allow meaningful UE power saving, e.g. if UE skips PO monitoring but the measurement still relies on MR without relaxation, MR cannot enter ultra-deep sleep mode and there is not much power saving gain compared to the case when UE still monitors PO.

- This will simplify the spec and NW/UE implementation. For example, NW can configure a single threshold to enable meaningful UE power saving rather than manipulating multiple thresholds which results in many combinations most of which are artificial and not very meaningful for UE power saving.

When entry condition of serving cell RRM measurement offloading and MR RRM measurement relaxation is met, LR will perform serving cell measurement. Depending on whether neighbour cell measurement is triggered or not (configuration of threshold SIntraSearchP, SIntraSearchQ, SnonIntraSearchP or SnonIntraSearchQ), UE may be in Case 1 (no MR measurement) or Case 3 (relaxed MR measurement). When exit condition serving cell RRM measurement offloading and MR RRM measurement relaxation is met, LR may not perform serving cell measurement, and MR should perform serving cell measurement in legacy manner, i.e. UE will exit Case 1 or Case 3. As discussed above, entry/exit of Case 1/3 should happen at same time as LP-WUS monitoring.

Proposal 7: RAN4 to suggest RAN2 to define same entry/exit condition for LP-WUS monitoring, serving cell RRM measurement offloading and MR RRM measurement relaxation.

Issue 1-1-8: LP-WUR operating carrier frequency

Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. Issue for further discussion based on contribution driven: - For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

We support to define RRM requirements based on the assumption that LR and MR are operating on the same carrier frequency. We observe following issues when they are on different carrier frequencies, and we understand supporting such a scenario requires big efforts in RAN1/2/4.

- LR needs to be always ON before UE enters LP-WUS monitoring. This is because MR measurement cannot provide any information regarding the radio condition LR may experience.

- Time/frequency sync of LR may be different from (worse than) the agreed simulation assumption as MR cannot facilitate LR sync.

- UE may lose its MR serving cell when it is monitoring LP-WUS. This is because during the period when LR condition is good, MR condition may have changed a lot on another frequency. The existing IDLE mode mobility framework based on cell reselection is not working.

- When UE detects LP-WUS or when exit condition for LP-WUS monitoring is met, MR may need to do cell selection before it can monitor PO, and this will impact how NW determines where to send paging and how to configure the offset between PO and LO.

In last meeting, some companies mentioned another scenario where LR only performs LP-WUS monitoring on a different frequency from MR, but not the serving cell measurement. In our view, the motivation of this scenario is not clear because there is no power saving gain for the UE (MR needs to perform serving cell measurement in legacy manner without relaxation to maintain the mobility). Also, when LR is not doing the measurement, it may be difficult for LR to obtain time/frequency sync for LP-WUS monitoring. We suggest RAN4 not to consider defining requirements for this scenario before the motivation and feasibility are clarified.

For defining RRM requirements, we suggest to focus on the most basic and essential scenario in R19.

Proposal 8: RAN4 to define RRM requirements based on the assumption that LR and MR are operating on the same carrier frequency.

Issue 1-1-9: LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS

In our understanding, the LR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring is depending on the entry condition of LP-WUS monitoring.

In RAN2#127 the following working assumption was made.

- Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

In RAN2#127-bis, the following agreement was made.

- If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements.

This means when NW configures LR threshold for the entry condition of LP-WUS monitoring, LR needs to be turned ON to perform serving cell measurement. Considering this is a transient period, we suggest RAN4 not to define requirements for this time period.

Proposal 9: RAN4 not to define LR measurement requirements before entering or after exiting LP-WUS monitoring or LR measurement offloading.

Issue 1-1-10: Considerations on higher priority frequency layer

In our view, UE should always perform measurement on higher priority carriers, if configured. Configuring higher property carriers is an important tool from NW perspective to manage the mobility of UEs in RRC_IDLE or RRC_INACTIVE, and it should not be totally compromised due to power saving. Otherwise, it may impact the usage of the LP-WUR feature, i.e. NW may not enable LP-WUR feature when it wants to manage the mobility of UEs in RRC_IDLE or RRC_INACTIVE.

In current spec, the requirements for higher priority carriers depends on whether serving cell condition:

- If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, higher priority carriers are measured according to requirements in clause 4.2.2.7, i.e. every (60 * Nlayers) s. This is already a relaxed requirement compared to the case below. We are open to discuss whether the requirements should be further relaxed when LR measurement is ON (i.e. Case #4).

- If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, higher priority carriers are measured together with other carriers according to requirements in clause 4.2.2.4. When LR measurement is ON, the measurement should be relaxed based on agreement of Case #3 in Issue 1-1-1.

Proposal 10: UE should perform measurement for higher priority frequency layers, if configured

- If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements

- If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed.

Issue 1-1-12: RRM requirements for FR2

We support to prioritize defining RRM requirements for FR1 in this WI. Compared to FR1, an outstanding impact on RRM requirements of FR2 is Rx beamforming. It may consume a lot of RAN4 efforts to discuss the Rx beam sweeping factor, the Rx beamforming gain for a completely new Rx, etc. Considering the RAN4 workload, we suggest RAN4 to focus on defining RRM requirements for FR1 first.

Proposal 11: RAN4 to prioritize defining RRM requirements for FR1 in this WI.

Issue 1-1-13: LP-SS frequency domain relation with SSB

In our view, FDM between LP-SS and SSB is a valid NW configuration and should be supported. RAN4 may need to check whether there is any impact to RRM requirements from this configuration, e.g. whether UE can meet the measurement requirements for both MR and LR in Case 3. This depends on how SSB and LP-SS are configured. Since RAN1 has not concluded on supported LP-SS periodicity, we suggest RAN4 to further discuss the impact based on RAN1 conclusion.

Proposal 12: RAN4 to further discuss whether there is RRM impact due to the configuration of FDMed SSB and LP-SS.

LP-WUR measurement

Issue 1-2-1: Accuracy requirements

We support to define accuracy requirements for LR measurement in the same way as cell reselection requirements, where accuracy is implicitly defined as margin (highlighted in yellow in the copied requirements). So far, dedicated accuracy requirement in clause 10 are all for reported measurement in RRC_CONNECTED, and we prefer to keep same principle for this WI.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that: when rangeToBestCell is not configured: - the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

Proposal 13: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement, and reflect the accuracy performance as a margin in the core requirement.

Issue 1-2-2: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

In our view, RAN4 needs to discuss evaluation requirements for the entry and the exit condition separately.

- Exit is more time critical because the longer UE takes to exit LP-WUS monitoring, the more likely UE is to miss paging. Missing paging has negative impact on both NW and UE side, and we understand it should have more weigh than power saving. We support to define evaluation requirements for the exit condition based on single measurement period.

- Entry is not as time critical as exit, and to make sure there is no ping-pong between entry and exit, we support to define evaluation requirements for the entry condition based on evaluation period which includes multiple measurement period, which is similar to the existing requirements for the MR serving cell measurement for evaluating the S criterion.

In addition, to make sure the decision is not based on short term fast channel fading, we suggest each measurement should be based on filtering of at least 2 samples. This is also same as existing requirements.

Proposal 14: On the requirements for evaluation of entry/exit condition,

- Define evaluation requirements for the exit condition based on single measurement period,

- Define evaluation requirements for the entry condition based on evaluation period which includes multiple measurement period

Each measurement should be based on filtering of at least 2 samples.

Issue 1-2-2-1: Detail on LP-WUR/MR evaluation/measurement requirement

In our view, whether MR measurement, LR measurement or both MR and LR measurement is used for evaluating a particular entry or exit condition is discussed in separate issues e.g. 1-1-5, 1-1-6 and 1-1-7, and the decision may be done in other WGs. Also, it may depend on NW configuration, e.g. the entry condition of LP-WUS monitoring as RAN2#127-bis agreed.

On the other hand, for RAN4 we believe the requirements for LR and MR measurement period should be defined separately, regardless of how they are going to be used. For example, entry of LP-WUS monitoring may require both MR and LR measurement to be above certain threshold. In this case, the evaluation requirements of MR and LR apply independently, and UE enters LP-WUS monitoring when the later of the two evaluation periods ends.

Proposal 15: RAN4 to define LR and MR measurement period requirements separately.

Issue 1-2-2-2: LR measurement metrics used for entry/exit threshold evaluation for LP-WUS monitoring/RRM relaxation

We understand the issue is part of entry/exit condition as discussed in 1-1-5, 1-1-6 and 1-1-7. Since RAN2 is discussing the issue and has made agreement for LP-WUS monitoring, we do not see the need for RAN4 to discuss the same issue, unless the discussion is triggered by other WG.

Proposal 16: RAN4 not to discuss measurement metrics used for entry/exit threshold evaluation for LP-WUS monitoring/RRM relaxation, unless triggered by other WG.

Issue 1-2-3: On interruption related requirements for LP-WUR

In RAN1#118 the following is agreed related to PO monitoring after UE receives LP-WUS.

Agreement For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring Agreement On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured. - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported) - (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives - Alt 1: it monitors the PO associated with the offset. - Alt 2: it monitors the first PO after its reported wake-up delay. - Other alternatives are not precluded Note: The PO mentioned above refers to legacy PO configured for the UE.

In RAN1#118-bis the following is agreed related to PO monitoring after UE receives LP-WUS.

Agreement From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if - it does not detect a LP-WUS on the monitored LO - or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE.

In our view, it may be too early to conclude whether RAN4 needs to define interruption requirements as it depends on how UE should monitor PO after it receives LP-WUS which has not been concluded in RAN1.

It is noted that if UE is not expected to receive any PO in a time period following RAN1 spec, e.g. during the time period between a LO and a reference PO/PF, then RAN4 does not need to define interruption for this time period.

Proposal 17: RAN4 to wait for more RAN1 progress on PO monitoring before discussing the possible interruption requirements related to LR.

Issue 1-2-3-1: UE measurement requirements after receiving LP-WUS signal

In our understanding, when UE receives LP-WUS, UE will wake up MR to monitor PO. The time period from when UE receives LP-WUS to when UE receives PO is a transition period where UE will do MR wake up and sync. As MR is not fully operating, we do not think RAN4 should define measurement requirements for MR during this time period.

After UE reads PO, there can be 2 cases

- If UE finds it is paged, then it becomes legacy scenario where UE should follow existing requirements for MR measurement

- If UE finds it is not paged, then UE goes back to LP-WUS monitoring and should follow requirements for LR and possibly also MR measurement as discussed for Case #1, #3 and #4.

In both cases, we do not see additional RRM requirements needed.

Proposal 18: RAN4 not to define MR measurement requirements for the time period from when UE receives LP-WUS to when UE receives PO.

Issue 1-2-4: On LP-WUR cell selection evaluation requirements for case 1

We do not see the need to define cell selection evaluation requirements for LR. In a reasonable deployment, when UE is close to cell edge where S criterion needs to be evaluated, UE should have already exited LR measurement, and MR measurement should have started.

Proposal 19: RAN4 not to define cell selection evaluation requirements for LR.

Issue 1-2-6: LP-SS based RRM requirements periodicity and filtering

RAN4 agreed the measurement interval for LR measuring LP-SS. What remains open is the measurement interval for OFDM based LR measuring SSB.

- We support option A when LP-SS periodicity is configured. In our understanding, LP-SS is serving the same purpose as LP-SS for LR, and we do not see the reason to define different, especially more relaxed, requirements just because the LR is measuring a different RS, i.e. SSB.

- We support option B when LP-SS periodicity is not configured. In our understanding, DRX is not for LR, and following DRX cycle may cause late exit of LP-WUS monitoring and late MR wake up, when DRX cycle is large.

Proposal 20: RAN4 to define measurement period requirements for LP-WUR based on SSB

- Based on LP-SS periodicity if LP-SS periodicity is configured,

- Based on a fixed interval of [320]ms when LP-SS periodicity is not configured.

Issue 1-2-7: Other considerations

In our view, there is no strong need to evaluate the entry condition of LP-WUS monitoring, LR measurement or MR measurement relaxation based on MR measurement in RRC_CONNECTED. Entering LR is anyway not a time critical task, and it is more reliable to use the measurements in RRC_IDLE or RRC_INACTIVE to evaluate the entry condition. Coupling the entry condition with MR measurement in RRC_CONNECTED will complicate UE implementation.

Proposal 21: RAN4 not to consider evaluation of entry conditions of LP-WUS monitoring, LR measurement or MR measurement relaxation based on MR measurement in RRC_CONNECTED.

MR relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

On the delay requirements for relaxed MR measurement, as the relaxation is aimed at power saving, we believe the power saving gain should be the main factor to determine the relaxation factor. Figure 1 shows our evaluation on the power saving gain from SI for MR relaxed measurement, with different relaxation factors.

Figure 1: Power saving gain for MR relaxed measurement with different relaxation factors

It can be seen that the power saving gain reduced obviously when relaxation factor becomes smaller. With 10 times relaxation, the gain is 20% in ideal case for paging, and to achieve meaningful gain in realistic case, the relaxation factor needs to be > 20. We suggest RAN4 to consider 16 as the smallest relaxation factor.

Proposal 22: For MR measurement with relaxation the relaxation factor should be >= 16.

Issue 1-3-2: On Neighbour cell and serving cell relaxation factor

In our view, serving and neighbor cell measurements should share the same scaling factor because the power consumption of the MR measurement is determined by the more frequent measurement among serving and neighbour cell, i.e. the smaller scaling factor among the two. Meaningful power saving gain is obtained only when both measurements are relaxed with relaxation factor >= 16.

Proposal 23: Same relaxation factor applies to serving and neighbor cell measurements.

Issue 1-3-3: Accuracy for relaxed MR measurement

We support to re-use the legacy MR accuracy requirements for MR measurement with relaxation. Accuracy requirements do not depend on the measurement interval, i.e. same accuracy requirements apply regardless of whether the measurement is performed once per DRX cycle or per N DRX cycles. Here, by legacy MR accuracy requirements we mean the accuracy in clause 10 for CA/DC Idle Mode Measurements. We are also open to discuss other existing accuracy numbers in RRC_IDLE or RRC_INACTIVE.

Proposal 24: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation.

LP-WUR at CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

Recommendations: Suspend discussion on this topic until more progress from other group.

It is true that the objective of LP-WUR at RRC_CONNECTED mode does not involve RAN4, but it may be too earlier to preclude any RRM impacts as the RAN1/2 discussions are ongoing, and there may be RRM impacts. RAN4 can first focus on the LP-WUR requirements in RRC_IDLE or RRC_INACTIVE, and postpone the discussion for RRC_CONNECTED mode until more RAN1/2 conclusions are available.

Proposal 25: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Conclusions

In this paper we provided our views on RRM requirements related to LP-WUR.

Proposal 1: RAN4 not to support Case #2 for MR and LR measurement.

Proposal 2: RAN4 to define requirements for OFDM based LR measurement based on LP-SS, by re-using the requirements for OOK based LR measurement based on LP-SS.

Proposal 3: RAN4 to discuss following options for switch from Case #1 to Case #3 based on LR measurement.

- Option 1: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ

- Option 2: Use existing thresholds for SS-RSRP and SS-RSRQ, and define offsets for LP-RSRP and LP-RSRQ

Proposal 4: RAN4 to discuss following options for switch from Case #3 to Case #1 based on LR or both LR and MR measurements.

- Option 1: LR measurement only

- Option 2: both LR and MR measurements, and UE stops neighbor cell measurement when both measurements are below their corresponding thresholds

Proposal 5: RAN4 not to further discuss the entry/exit condition for measurement offloading to LR or MR RRM measurement relaxation, unless triggered by other WGs.

Proposal 6: For LP-WUS monitoring, RAN4 to discuss whether RSRP and RSRQ measurement from LR can reflect the impact of the inter-carrier interference.

Proposal 7: RAN4 to suggest RAN2 to define same entry/exit condition for LP-WUS monitoring, serving cell RRM measurement offloading and MR RRM measurement relaxation.

Proposal 8: RAN4 to define RRM requirements based on the assumption that LR and MR are operating on the same carrier frequency.

Proposal 9: RAN4 not to define LR measurement requirements before entering or after exiting LP-WUS monitoring or LR measurement offloading.

Proposal 10: UE should perform measurement for higher priority frequency layers, if configured

- If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements

- If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed.

Proposal 11: RAN4 to prioritize defining RRM requirements for FR1 in this WI.

Proposal 12: RAN4 to further discuss whether there is RRM impact due to the configuration of FDMed SSB and LP-SS.

Proposal 13: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement, and reflect the accuracy performance as a margin in the core requirement.

Proposal 14: On the requirements for evaluation of entry/exit condition,

- Define evaluation requirements for the exit condition based on single measurement period,

- Define evaluation requirements for the entry condition based on evaluation period which includes multiple measurement period

Each measurement should be based on filtering of at least 2 samples.

Proposal 15: RAN4 to define LR and MR measurement period requirements separately.

Proposal 16: RAN4 not to discuss measurement metrics used for entry/exit threshold evaluation for LP-WUS monitoring/RRM relaxation, unless triggered by other WG.

Proposal 17: RAN4 to wait for more RAN1 progress on PO monitoring before discussing the possible interruption requirements related to LR.

Proposal 18: RAN4 not to define MR measurement requirements for the time period from when UE receives LP-WUS to when UE receives PO.

Proposal 19: RAN4 not to define cell selection evaluation requirements for LR.

Proposal 20: RAN4 to define measurement period requirements for LP-WUR based on SSB

- Based on LP-SS periodicity if LP-SS periodicity is configured,

- Based on a fixed interval of [320]ms when LP-SS periodicity is not configured.

Proposal 21: RAN4 not to consider evaluation of entry conditions of LP-WUS monitoring, LR measurement or MR measurement relaxation based on MR measurement in RRC_CONNECTED.

Proposal 22: For MR measurement with relaxation the relaxation factor should be >= 16.

Proposal 23: Same relaxation factor applies to serving and neighbor cell measurements.

Proposal 24: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation.

Proposal 25: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Reference

- R4-2416861, WF on RRM requirements for NR_LPWUS, Moderator (vivo)