3GPP TSG-RAN WG4 Meeting #113	R4-2419151

Orlando, US, 18 – 22 November, 2024

Title: 	Discussion on other RRM requirements for Rel-19 NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.26.4.2

Document for:	Discussion

Introduction

RRM requirements Rel-19 NTN are discussed in RAN4#112-bis, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the RRM requirements for following objectives.

- Downlink coverage enhancements

In this paper, we will provide our views on other RRM requirements for Rel-19 NTN.

Discussion

### Issue 1-1: SSB periodicity enhancement Agreement: - RAN4 to wait for RAN1’s definitive decision on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements. FFS on the detailed list of RRM requirements to be affected, e.g., - uplink timing requirements, RRM requirements during transmission between different SSB periodicities, default assumption on the SSB and SMTC periodicities in handover/satellite switch with re-sync/RRC re-establishment/RRC connection release with redirection, cell identification, measurements, etc.

In RAN1#118-bis the following is agreed related to DL coverage enhancement.

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

In RAN2#127 the following is agreed related to DL coverage enhancement.

- From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) - From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. - RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity - If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

It can be seen that extension of SSB periodicity for initial access will be supported by RAN1. The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms, and it is FFS whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms.

From RAN4 perspective, we understand the existing RRM requirements can be re-used as baseline with SSB periodicity extension, for the following reasons.

- There is no requirement on cell selection (initial access) in RAN4.

- Existing RRM requirements are based on configured SSB or SMTC period which can be up to 160ms. If SSB periodicity is extended beyond 160ms, RAN2 will extend the signaling range, and we also do not see direct impact from it, as we already have DRX requirements where measurement interval will follow DRX cycle which can be much larger than 160ms.

One particular impact of default SSB periodicity extension up to 160ms is the RRC re-establishment.

Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); Kmulti_SMTC * MAX (800 ms, 10 x TSMTC). < -8; FR1; N/A; k * 800 ms Note1, 3. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in clause 9.2C.5.1. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt..

It can be seen that for the case with Serving cell SSB Ês/Iot (dB) < -8, the delay requirements are not based on configured SMTC periodicity but a default SMTC periodicity of 20ms. When the default SSB periodicity is extended beyond 20ms, the assumption of default SMTC periodicity of 20ms is not reasonable, and the delay requirements also needs to be redefined based on extended default SMTC periodicity.

Proposal 1: For default SSB periodicity extension (no matter the maximum value is 160ms or 320ms), RAN4 to define new RRC re-establishment requirements for the case with Serving cell SSB Ês/Iot (dB) < -8.

If the default SSB periodicity is extended to 320ms, there would be other RRM impacts. Below are the impacts we identified.

- The timing accuracy requirements (Te) are defined based on the assumption that SSB is available at least every 160ms, and RAN4 needs to discuss whether the current requirements apply with 320ms, or some relaxation is needed considering timing drift due to frequency error.

The UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc.

- Another impacted requirement is the MG related if MGRP would be extended 320ms. The existing requirements are defined based on the assumption of maximum MGRP = 160ms. Additional MG pattern needs to be added, and the MG sharing rule may need to be revisited.

Table 9.1C.2-1: Gap Pattern Configurations Gap Pattern Id; Measurement Gap Length (MGL, ms); Measurement Gap Repetition Period (MGRP, ms). 0; 6; 40. 1; 6; 80. 2; 3; 40. 3; 3; 80. 4; 6; 20. 5; 6; 160. 6; 4; 20. 7; 4; 40. 8; 4; 80. 9; 4; 160. 10; 3; 20. 11; 3; 160. 12; 5.5; 20. 13; 5.5; 40. 14; 5.5; 80. 15; 5.5; 160. 16; 3.5; 20. 17; 3.5; 40. 18; 3.5; 80. 19; 3.5; 160. 20; 1.5; 20. 21; 1.5; 40. 22; 1.5; 80. 23; 1.5; 160. 24; 10; 80. 25; 20; 160. two colliding measurement gaps are fully overlapping with MGRP=160ms, in case of collision between two measurement gap occasions, the UE shall perform measurements in the occasion of one of the measurement gaps, and it is up to UE implementation which occasion is used for measurement, provided that UE can meet the requirements in clause 9.2C and 9.3C. If two measurement gaps are fully overlapping with MGRP<160ms, no measurement requirements apply.

Proposal 2: For default SSB periodicity extension with maximum value 320ms, RAN4 to discuss potential impacts on

- Timing requirements

- MG related requirements (MGRP, MG sharing)

We are also open to discuss potential impacts of beam hopping which is used together with SSB periodicity extension to improve DL coverage, e.g. cell DTX. Since this is pending on conclusion from other WGs, RAN4 can further wait before discussing the exact RRM impact.

Proposal 3: RAN4 to wait for more conclusion from RAN1/2 before discussing the RRM impacts of beam hopping e.g. cell DTX.

Conclusions

In this paper we provided provide our views on other RRM requirements for Rel-19 NTN.

Proposal 1: For default SSB periodicity extension (no matter the maximum value is 160ms or 320ms), RAN4 to define new RRC re-establishment requirements for the case with Serving cell SSB Ês/Iot (dB) < -8.

Proposal 2: For default SSB periodicity extension with maximum value 320ms, RAN4 to discuss potential impacts on

- Timing requirements

- MG related requirements (MGRP, MG sharing)

Proposal 3: RAN4 to wait for more conclusion from RAN1/2 before discussing the RRM impacts of beam hopping e.g. cell DTX.

Reference

- R4-2416866, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)