# 3GPP TSG-RAN WG4 Meeting #113                                                                R4-2419160

# Orlando, US, 18th – 22nd November, 2024

Title: 	Remaining discussions on UE capability for type 4 UEs

Source: 	Huawei, HiSilicon

Agenda item:	7.5.2.2

Document for:	Discussion

# Introduction

RAN4 112-bis meeting the WF [1] was agreed, in which some open points are left without a conclusion. In this document we are going to address them.

# Discussion

### < Issue 2-3: BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated) > ### Way Forward: ### Continue further discussion on BS signalling in the next meeting.

In Rel-18 switching type 2 to type1 was agreed, in order to enable more DL layers per CC (2 DL layers/CC for type 2, 4 DL layers/CC for type 1). As type 4 and type 1 have 4 DL layers/CC then such a swtich does not show its advantage. On the other hand by switching from type 4 (8 separated Rx chain) to type1 (4 partially shared Rx chain) the UE can use the remaining Rx chains for diversity, which can be helpful for a better quality of service in scenarios with coverage difficulities.

Proposal 1: Switching from type 4a/4b to type 1 4layers/CC can be beneficial due to increase reception diversity.

Proposal 2: To allow a switching from type 4a/b (default type) to type 1 with 4Rx, RAN2 should, either introduce a new NW IE or extend nonCollocatedTypeNR-CA/  nonCollocatedTypeMRDC to make it applicable for UEs configured with maxMIMO-Layers ≤ 4, too.

### < Issue 2-4: BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated > Way Forward: - Continue further discussion on BS signalling in the next meeting.

Such a switching would be interesting, because the operators can gain 2 or 4 more MIMO layers/CC when they switch from type 4 to type 1 with 6 or 8 Rx in a collocated scenario, respectively. For such a switching, RAN2 should, either introduce a new NW IE or extend nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC to make it applicable for UEs configured with maxMIMO-Layers ≤ 4, too. Moreover another way would be to use the legacy NW IE maxMIMO-Layers=6 or 8 to switch from type 4 to type 1 with 6 or 8 Rx. The risk of using this legacy IE would be that if one day RAN4 decides to introduce a new type with 6 or 8Rx/CC for non-collocated deployment, using maxMIMO-Layers =6 or 8 would cause confusion between the new UE type and type 1 with 6 or 8Rx/CC.

Proposal 3:

Option1: To allow a switching from type 4a/b (default type) to type 1 with 6Rx/8Rx, RAN2 should, either introduce a new NW IE or extend nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC to make it applicable for UEs configured with maxMIMO-Layers ≤ 4, too.

Option 2: If there is a certainty that in future releases, no new non-collocated UE type (such as type 5 and ...) with 6 or 8 Rx/CC is added, the legacy NW IE maxMIMO-Layers=6 or 8 to switch from type 4 to type 1 with 6 or 8 Rx.

### < Issue 2-6: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b > Way Forward: - Continue further discussion on BS signalling in the next meeting.

Proposal 4: The UE behavior during initial connection among type 1, 2 and 4a/4b depends directly on the issues 2.3 and 2.4 and the outcome of those discussions clarifies the UE behavior for switching from type 4a/4b to type1. On the other hand switching between type 4 to type 2 can happen with the RRC maxMIMO-Layers=2, as type 4 and type 2 both use the same UE architecture.

# Conclusions

Proposal 1: Switching from type 4a/4b to type 1 can be beneficial due to increase reception diversity.

Proposal 2: To allow a switching from type 4a/b (default type) to type 1 with 4Rx, RAN2 should, either introduce a new NW IE or extend nonCollocatedTypeNR-CA/  nonCollocatedTypeMRDC to make it applicable for UEs configured with maxMIMO-Layers ≤ 4, too.

Proposal 3:

Option1: To allow a switching from type 4a/b (default type) to type 1 with 6Rx/8Rx, RAN2 should, either introduce a new NW IE or extend nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC to make it applicable for UEs configured with maxMIMO-Layers ≤ 4, too.

Option 2: If there is a certainty that in future releases, no new non-collocated UE type (such as type 5 and ...) with 6 or 8 Rx/CC is added, the legacy NW IE maxMIMO-Layers=6 or 8 to switch from type 4 to type 1 with 6 or 8 Rx.

Proposal 4: The UE behavior during initial connection among type 1, 2 and 4a/4b depends directly on the issues 2.3 and 2.4 and the outcome of those discussions clarifies the UE behavior for switching from type 4a/4b to type1. On the other hand switching between type 4 to type 2 can happen with the RRC maxMIMO-Layers=2, as type 4 and type 2 both use the same UE architecture.

# Reference

[1]	“R4-2417115, WF on rest issues for intra-band non-collocated EN-DC/NR-CA, KDDI.”