# 3GPP TSG-RAN WG4 Meeting # 113                                    R4-2419166

# Orlando, US, 18th – 22nd November, 2024

# Title: 	Discussion on spatial channel model for demodulation performance requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.14.2

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of study on spatial channel model for demodulation performance was approved in [1], wherein the study item includes the following objectives :

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In this contribution, we give some initial discussion on the practical spatial channel modelling methodology and other analysis based on the last meeting conclusion [2].

# 2 Discussion

During the last meeting, RAN4 discussed how to construct the SCM channel model and some simulation cases. Regarding the construction of the SCM channel model, there are essentially three approaches, CDL channel model based on the TR 38.827, extending current TDL channel model and TX-RX beam steering approach, and the simulation scenarios predominantly revolve whether MU-MIMO test case needs to be considered and the selection of current TDL channel model.

Subcluster Correlation

Proposals: - Option 1: RAN4 should discuss and clarify whether the potential correlation between subclusters is an issue, and whether there should be mechanisms to avoid it.

In the process of modeling a 3D channel using the geometry based stochastic modeling (GBSM) method, it is necessary to employ ray tracing to statistically model the characteristics of all scattering multipath components within the simulation scenarios, thus calculating the channel impulse. The statistical properties of the multipath components are describe by four sets of parameters: delay spread (DS), line-of-sight gain (K factor), shadow fading (SF), and angular spread (AS) . The statistical cross-correlation matrix of multipath components is a matrix comprised of the cross- correlation coefficients between ant two of these parameters. If N parameters are required to describe the multipath components of the channel, the statistical cross-correlation matrix of multipath components is an N×N positive-definite matrix.

The statistical cross-correlation matrix of multipath components has the following properties:

- The correlation matrix is positive definite ;

- The elements on the diagonal of the correlation matrix are 1.

- In 2D channel modeling, the angular spread parameters only include the horizontal dimension’s two spread parameters (ASD,ASA, azimuth spread of departure angle / azimuth spread of arrival angle ), resulting in total of five parameters (DS/ASD/ASA/K/SF) for a 2D channel, and the cross- correlation matrix is a 5×5 positive definite matrix, as shown in following. Since the 3D channel models the vertical dimension’s angle, in addition to the horizontal angular spread parameters, the channel angular spread parameters should also include vertical spread parameters, ZSD/ZSA (azimuth spread of departure angle / azimuth spread of arrival angle for angle ZOA/ZOA), meaning a 3D channel requires seven parameters (DS/ASD/ASA/K/SF/ZSD/ZSA) to describe the statistical properties of the multipath components. The statistical cross-correlation matrix of multipath components is a 7×7 positive definite symmetric matrix. The order of the seven parameters in the matrix is not fixed and can be in any order; for example, ρDS-ZSD and ρZSD-DS have the same meaning, both indicating the cross-correlation coefficient between parameters DS and ZSD.

- 1ρDS−ASDρASD−DS1ρASA−DSρASA−ASDρDS−ASAρDS−KρDS−SFρASD−ASAρASD−KρASD−SF1ρASA−KρASA−SFρK−DSρK−ASDρSF−DSρSF−ASDρK−ASA1ρK−SFρSF−ASAρSF−K1

- The statistical cross-correlation matrix of multipath components for the 3D channel is an extension based on the 2D channel cross-correlation matrix in the WINNER+ channel model. Since the cross-correlation matrix is derived from statistical analysis of real measurement data, there is no theoretical basis to determine which approach is closer to the actual 3D channel. Therefore, 3GPP building upon the WINNER+ channel model, incorporated the empirical results from various companies and performed positive definiteness correction to obtain the final version of the statistical cross-correlation matrix of multipath components for the 3D channel as the following table.

Table 7.3-6: Channel model parameters for 3D-UMi and 3D-UMa

| Scenarios |  | 3D-UMi |  |  | 3D-UMa |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I | LOS | NLOS | O2I |
| Cross-Correlations | ASD vs DS | 0.5 | 0 | 0.4 | 0.4 | 0.4 | 0.4 |
|  | ASA vs DS | 0.8 | 0.4 | 0.4 | 0.8 | 0.6 | 0.4 |
|  | ASA vs SF | -0.4 | -0.4 | 0 | -0.5 | 0 | 0 |
|  | ASD vs SF | -0.5 | 0 | 0.2 | -0.5 | -0.6 | 0.2 |
|  | DS vs SF | -0.4 | -0.7 | -0.5 | -0.4 | -0.4 | -0.5 |
|  | ASD vs ASA | 0.4 | 0 | 0 | 0 | 0.4 | 0 |
|  | ASD vs K | -0.2 | N/A | N/A | 0 | N/A | N/A |
|  | ASA vs K | -0.3 | N/A | N/A | -0.2 | N/A | N/A |
|  | DS vs K | -0.7 | N/A | N/A | -0.4 | N/A | N/A |
|  | SF vs K | 0.5 | N/A | N/A | 0 | N/A | N/A |
|  | ZSD vs SF | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs SF | 0 | 0 | 0 | -0.8 | -0.4 | 0 |
|  | ZSD vs K | 0 | N/A | N/A | 0 | N/A | N/A |
|  | ZSA vs K | 0 | N/A | N/A | 0 | N/A | N/A |
|  | ZSD vs DS | 0 | -0.5 | -0.6 | -0.2 | -0.5 | -0.6 |
|  | ZSA vs DS | 0.2 | 0 | -0.2 | 0 | 0 | -0.2 |
|  | ZSD vs ASD | 0.5 | 0.5 | -0.2 | 0.5 | 0.5 | -0.2 |
|  | ZSA vs ASD | 0.3 | 0.5 | 0 | 0 | -0.1 | 0 |
|  | ZSD vs ASA | 0 | 0 | 0 | -0.3 | 0 | 0 |
|  | ZSA vs ASA | 0 | 0.2 | 0.5 | 0.4 | 0 | 0.5 |
|  | ZSD vs ZSA | 0 | 0 | 0.5 | 0 | 0 | 0.5 |

- As mentioned above, the cross-correlation matrix is obtained through statistical analysis of real measurements data for 3D channel model. In the CDL channel model, each delay path is assigned a specific power and Doppler shift. When considering the impact of large-scale fading, the power distribution of these paths varies with time and spatial location. If multiple delay paths undergo a similar large-scale fading process, their power fluctuations will exhibit a certain degree of cross-correlation Additionally, the large-scale fading factors also follow a statistical distribution.

- Regarding the sub-clusters, the rays originating from the strongest clusters are spread in delay to three sub-clusters with fixed delay offset. From TR 38.827, twenty rays of a cluster are mapped to three sub-clusters as presented in following table. From our understanding, given that sub-clusters are generated with a fixed delay offset, we can assume that there is a correlation among the rays with each sub-cluster. However, due to different delay offsets between sub-clusters, we can at least consider uncorrelated.

Table 7.5-5: Sub-cluster information for intra cluster delay spread clusters

| sub-cluster # | mapping to rays | Power | delay offset |
| --- | --- | --- | --- |
|  |  | 10/20 | 0 |
|  |  | 6/20 | 1.28 |
|  |  | 4/20 | 2.56 |

Observation 1. The cross-correlation matrix is derived from statistical analysis of real measurement data and adheres to certain statistical characteristics.

Observation 2. The sub-clusters can be generated from the strongest clusters with different fixed delay offset, without exhibiting any statistical characteristics.

Proposal 1. The correlation between sub-clusters does not need to be considered.

Angles of Departure and Angles of Arrival

Proposals: - Option 1: Replace the mean angle in the last position of departure and arrival angles generated equation by the desired angle as .

There is a description in TR 38.901 as follows.

The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to  and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation:

is the mean angle of the original channel model table in NLOS case in TR 38.827.

In TR 38.901, a clear analysis is provided about some of the potential issues with using mean angles, such as in PMI feedback. When we discussed the test cases in last meeting, PMI test case with single user is also considered. Therefore, we think that replace the mean angle by the desired angle is reasonable.

Proposal 2. To consider replacing the mean angle   in the last position of departure and arrival angles generated equation by the desired angle as

BS Antenna Virtualiser choice

Proposals: - Option 1: Down Tilt Virtualiser : -10 degrees - Option 2: Broadside - Option 3: Max Receive Power - Option 4: DFT Virtualiser according to TR 36.897 with application of panning and tilting angles (as per Table 7.8-1 of 38.901)

During the actual transmission process, multiple data streams are mapped into multiple antenna ports by means of digital baseband processing (precoding), and then the multiple antenna ports are mapped to antenna elements by means of analog beamforming. For the antenna ports to antenna elements mapping, 3GPP supports either a sub-array connected structure (each antenna port is connected to a unique array of antenna elements) or a fully connected structure (each antenna port is connected to all antenna elements). In analog beamforming, digital signals on the antennas (where all antennas process the same signal) are phase adjusted via phase shifters in the analog domain to generate beams in specific directions.

In legacy BS array antennas, uniform linear arrays with fix phase shifts between its M elements to obtain a beamtilt in vertical direction are modelled using complex weights for 1D virtualization under sub-array partition.

Meanwhile, for full connection model with 2D virtualization, using following formula to generate beamforming weight matrix.

From the formula, we need to model the coordinate positions of the antenna elements, such as the horizontal direction with a spacing dH and the vertical direction with a spacing dV, which is only applicable for URA antenna array. Also if we consider antenna array virtualization, the electronic downtilt of antennas is actually by modifying the internal electronic components to change the radiation direction of the antenna within a certain range. As shown in the above formula, for antenna elements with co-polarized within the same sub array, the beam direction can be altered by adjusting the phase shifts.

And regarding the max receiver power approach, like TR 38.827 mentioned, which is a general formula, supports any antenna array geometry, as shown in following.

α1,1…αMe,Ne=1MeNeexp−j2πrtx,max T ∙ dtx,me,neλ0∈ℂ1×MeNe,

where dtx,me,ne is the location vector of transmit antenna element, which applicable to various types of antenna array. and rtx,max is a spherical unit vector denoting the target beam direction. What’s more, we need to define the Tx beam direction in both elevation and azimuth, e.g. a code book of 60 fixed beams is constructed to a grid of five elevation angles from–20 to +20 with 10 steps and 12 azimuth angles from–80 to +80 with ~15 step for FR1. Therefore, we get the position of each antenna elements, and the beam direction, CSI-RS ports mapping to antenna elements can be achieved.

In general, both above methods are indispensable approaches to achieving the mapping of CSI-RS ports to antenna ports. As for electronic down tilt method, there are only applicable to URA or ULA structures. Meanwhile, the method in 827 is more universal, suitable for all antenna array structures. More importantly, the beam direction of the antenna can be defined based on both the elevation and azimuth angles.

Observation 3. For the down tilt approach, it is only applicable to URA or ULA antenna structures. Meanwhile, the method in 827 is more general, suitable for all antenna array structures.

Proposal 3. From evaluation purpose, maybe we can consider different approaches for BS virtualization, e.g. down tilt and the method in TR 38.827.

TDL and CDL Methodologies

Agreement: Interested companies to use the following candidate SCM options: - (Option A) CDL (as defined in R4-2415283) with Antenna Array Virtualisation (AAV) using subarray configuration - (M,N,P,Ms,Ns) = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X) - (M,N,P,Ms,Ns) = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X) - (M,N,P,Ms,Ns) = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y) - (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y) - (Option B) CDL (as defined in R4-2415283), i.e., with AAV without subarray (fully connected) - [(M,N,P) = (8,8,2) for 8Tx and 4Tx CSI-RS Ports] - (Option C, for interested companies) CDL (as defined in R4-2415283) without AAV with the following configuration: - [(M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports] - [(M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports] - (Option D, for interested companies) CDL from 38.901 with AAV using subarray - (M,N,P,Ms,Ns) = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X) - (M,N,P,Ms,Ns) = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X) - (M,N,P,Ms,Ns) = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y) - (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y) - (Option E, for interested companies) CDL from 38.901 with AAV without subarray (fully connected) - [(M,N,P) = (8,8,2) for 8Tx and 4Tx CSI-RS Ports] - (Option F, for interested companies) TDL extensions as defined in R4-2412762) - (Option G for interested companies) TDL extensions as defined in R4-2415382) - Where the BS antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays according to the following description: (M,N,P,Ms,Ns) - M is the number of antenna elements with the same polarization in each column (vertical size) - N is the number of antenna elements with the same polarization in each row (horizontal size) - P is the number of polarizations - Ms is the number of Subarray Elements with the same polarization in each column (vertical size) - Ns is the number of Subarray Elements with the same polarization in each row (horizontal size) - Options on BS antenna configuration for Options A, D: - Option X: Fixed array size 128 AE (8,8,2,8,Ns) - Option Y (where most companies views are aligned): Fixed sub-array size 16 AE (8,N,2,8,1) - Companies to clarify their assumptions used for radiation patterns of antenna elements.

During the last meeting, RAN4 discussed the antenna array virtualization (AAV) and the configurations for BS side antenna array. As shown in above, with AAV and without AAV are discussed. Regarding the BS side antenna pattern, the commonly used configuration at present is 192 elements, there are a total of 12 rows in the horizontal  direction and 8 columns of elements in the vertical direction, with 32 or 64 channels, as shown in following Fig 1, which depends on different companies implementation.

Fig 1. BS side Antenna array

And now, we discussed the antenna configuration, from our understanding, we have one aspects need to be considered, e.g. with AAV or without AAV. Firstly, for AAV, it is evident that there are two potential implementations: with or without AAV. Without AAV means there is a one to one correspondence between CSI-RS ports and antenna elements, eliminating the need to consider beamforming, as shown in Fig 2. It can also be assumed that the beamforming weight matrix is identify matrix 1. This is easily achieved in modelling and has no connection whatsoever to implementation.

Fig 2. 8 ports CSI-RS mapping

On the other hand, with AAV means we need to consider the beanforming weight matrix during the modelling process. Most crucially, whether the mapping of CSI-RS antenna ports to physical antennas is in a sub-array or full connection model, different manufacturers have their own implementations. During the last meeting, RAN4 discussed the antenna configuration for both sub-array and full connected model. From our understanding, different weight matrices can generate various beam directions, which is crucial for the CDL channel model. In the sub-array model, each antenna port is only associated with a portion of the physical antennas (i.e., subarray). This means that each CSI-RS signal may be transmitted by only a small subset of the antennas in the array, which can reduce mutual interference between signals while also lowering hardware complexity. Contrary to the subarray mapping, in the full connection model, each antenna port is connected to all physical antennas. This means that every CSI-RS signal is transmitted by the entire antenna array, thereby, providing strong signal strength and better coverage. However, the hardware complexity and energy consumption of this mode are relatively higher.

Since RAN4 is still in the evaluation phase, we have not seen a compelling reason to exclude either of the two candidate options. Given that RAN4 needs to study the CDL channel model under this WI, both potential implementations should as least be considered.

Proposal 4. Both sub-array and full connection models shall be considered at least in study item.

# 3 Conclusion

In this contribution, we give some discussions on spatial channel model for demodulation performance requirements, The conclusions are:

Observation 1. The cross-correlation matrix is derived from statistical analysis of real measurement data and adheres to certain statistical characteristics.

Observation 2. The sub-clusters can be generated from the strongest clusters with different fixed delay offset, without exhibiting any statistical characteristics.

Observation 3. For the electronic downtilt approach, it is only applicable to URA or ULA antenna structures. Meanwhile, the method in 827 is more general, suitable for all antenna array structures.

Proposal 1. The correlation between sub-clusters does not need to be considered.

Proposal 2. To consider replacing the mean angle   in the last position of departure and arrival angles generated equation by the desired angle as

Proposal 3. From evaluation purpose, maybe we can consider different approaches for BS virtualization, e.g. electronic downtilt and the method in TR 38.827.

Proposal 4. Both sub-array and full connection models shall be considered at least in study item.

# 4 References

- RP-241610, New WID: Study on spatial channel model for demodulation performance requirements, Nokia.

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM. Nokia.