# 3GPP TSG-RAN WG4 Meeting #113                                                                R4-2419175

# Orlando, US, 18th – 22nd November, 2024

Title: 	Clarification on 12 and 20 PRB transmission BW configurations and LS reply to R4-2417510

Source: 	Huawei, HiSilicon

Agenda item:	5.8.1

Document for:	Discussion and LS Reply

It was mentioned in RAN2 LS R4-2417510 to clarify how the IE carrierBandwidth needs to be configured in case of 12 and 20 PRB transmission BW configurations. It should be first acknowledged that the IE carrierBandwidth refers to transmission BW configuration as mentioned in 38.101-1 section 5.3.4 “The UE transmission bandwidth configuration is indicated by the higher layer parameter carrierBandwidth”.Based on RAN4 specification there could be two options for IE carrierBandwidth . The first one would be restricting the transmission bandwidth to 12 PRB or 20 PRB when the SSB is located on 920.73 MHz or 921.45 MHz, respectively, as shown in Table 1. This option is based on TS 38.101-1 Table 5.4.3.1-3, where 12 PRB and 20 PRB transmission bandwidth are considered within 3 MHz and 5 MHz CBW for band n100, respectively. This option means that the SSB and RF channel are locked-in as shown in Figure 1.

Table 5.4.3.1-3: Additional GSCN parameters for band n100 SS Block frequency position SSREF (MHz); GSCN; Note. 920.73; 41637; Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. 921.45; 41638; Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1..

Option 2 requires no restriction on the transmission bandwidth configuration and it can be set as 15 or 25 PRB as shown in Table 1 and Figure 1.

Table 1 SSB BW and transmission BW configuration

|  | sync raster points (TS 38.101-1) | Transmission BW (carrierBandwidth) (PRB) | SSB BW (PRB) |
| --- | --- | --- | --- |
| less than 5MHz (new) sync raster | Table 5.4.3.1-2 | 15 | 12 |
|  | Table 5.4.3.1-3 (920.73 MHz) | 12 (option1) | 12 |
|  |  | 15 (option2) |  |
| 5 MHz | Table 5.4.3.1-3 (921.45 MHz) | 20 (option1) | 20 |
|  |  | 25 (option2) |  |

Figure 1 SSB and transmission BW configuration

In the following sections we will present what needs to be changed if either options is confirmed.

## Option 1: 12 or 20 PRB PBCH and 12, 20 PRB transmission BW configuration

The issue mentioned in the LS R4-2417510 comes from the fact that when in a cell IE carrierBandwidth < NRB, the UE shall consider that cell as barred and shall not continue decoding the rest of the SIB1. However for the band n100, there can be a case where the carrierBandwidth=12 or 20 but the NRB= 15 or 25, respectively, which could lead to having the cell barred.

Therefore we propose to consider 12 and 20 PRB as maximum transmission bandwidth configurations exceptionally for band n100 only by adding a note in Table 5.3.2-1 of TS 38.101-1 as shown below, which will prevent the cell from being barred because carrierBandwidth = NRB = 12 or 20 PRB.

TS 38.101-1 Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 151 | 251 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| NOTE 1: for 3 and 5 MHz CBW the maximum transmission bandwidth configurations, could be reduced to 12 and 20 PRB for n100. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1.1: update the Table 5.3.2-1 of TS 38.101-1 as shown above.

Proposal 1.2: send the following LS to RAN2 as a reply to R4-2417510 (Option 1)

## Option 2: 12 or 20 PRB PBCH and 15, 25 PRB transmission BW configuration

As mentioned in Table and Figure 1, the 2nd option is to avoid having any restriction on transmission BW configuration and specify only the SSB (or PBCH) size for the sync rasters at 920.73 MHz and 921.45 MHz. therefore the TS 38.101-1 Table 5.4.3.1-3 needs to be updated as shown below:

TS 38.101-1 Table 5.4.3.1-3: Additional GSCN parameters for band n100

| SS Block frequency position SSREF (MHz) | GSCN | Note |
| --- | --- | --- |
| 920.73 | 41637 | Only applicable for 12 PRB punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1., specifically to 3 MHz channel |
| 921.45 | 41638 | Only applicable for 20 PRB unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1, specifically to 5 MHz channel |

With these changes the transmission bandwidth configuration (IE carrierBandwidth ) will no more be restricted to 12 or 20 PRB.

Proposal 2.1: update the TS 38.101-1 Table 5.4.3.1-3 as shown above.

Proposal 2.2: send the following LS to RAN2 as a reply to R4-2417510 (Option 2)

## CORESET0 center frequency for SSB located at 920.73 MHz

Per RAN1 agreement R1-2308587, the 12 PRB CORESET0 is aligned with the 12 PRB SSB (KSSB=0), as shown in Figure 1. On the other hand the CORSET0 has to happen on the channel raster, and 920.73 MHz is not on the channel raster. Therefore it would be reasonable to change the sync raster to 920.7MHz. It should be mentioned with such a shift, an offset of 20 KHz will still be left to the CEPT allocation limitations (919.6 MHz), as shown in Figure 2.

Figure 2 Shifted sync raster to align the CORESET0 on a Channel raster

Proposal 3: To align the 12 PRB CORSET0 with the channel raster, shift the sync raster by 30KHz from 920.73 to 920.7MHz.

# Conclusion

Consider either Proposal 1 or 2:

Proposal 1: consider 12 or 20 PRB PBCH and 12, 20 PRB transmission BW configurations for 3 and 5 MHz, respectively.

Proposal 1.1: update the Table 5.3.2-1 of TS 38.101-1 as shown above.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 151 | 251 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| NOTE 1: for 3 and 5 MHz CBW the maximum transmission bandwidth configurations, could be reduced to 12 and 20 PRB for n100. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1.2: send the following LS to RAN2 as a reply to R4-2417510 (Option 1)

Proposal 2: consider 12 or 20 PRB PBCH and 15, 25 PRB transmission BW configurations for 3 and 5 MHz, respectively.

Proposal 2.1: update the TS 38.101-1 Table 5.4.3.1-3 as:

TS 38.101-1 Table 5.4.3.1-3: Additional GSCN parameters for band n100

| SS Block frequency position SSREF (MHz) | GSCN | Note |
| --- | --- | --- |
| 920.73 | 41637 | Only applicable for 12 PRB punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1., specifically to 3 MHz channel |
| 921.45 | 41638 | Only applicable for 20 PRB unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1, specifically to 5 MHz channel |

Proposal 2.2: send the following LS to RAN2 as a reply to R4-2417510 (Option 2).

Proposal 3: To align the 12 PRB CORSET0 with the channel raster, shift the sync raster by 30KHz from 920.73 to 920.7MHz.

# Option 1 Reply to LS R4-2417510

Title:	Reply to LS on carrierBandwidth configuration for less-than-5MHz carriers

Response to:	R4-2417510

Release:	Rel-18

Work Item:	NR_FR1_lessthan_5MHz_BW

Source:	RAN4

To:	RAN2

Cc: 	RAN1

Contact Person:

Name:	Mohammad Abdi Abyaneh

Email:		mohammad.abdi.abyaneh@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR XXXXX

1. Overall Description:

RAN4 would like to thank RAN2 for the questions.

RAN4 agreed to associate carrierBandwidth to maximum transmission bandwidth configuration. Moreover it was agreed to consider 12 and 20 PRB as valid maximum transmission bandwidth configurations for 3 MHz and 5 MHz CBWs in n100, exceptionally. These agreements are captured in CR XXXXX.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to consider the aforementioned points.

# Dates of next TSG RAN WG4 meetings

RAN4#114	      February 17 – 21, 2025		Athens, Greece

RAN4#114bis	April 7 – 11, 2025		         TBC, China

# Option 2 Reply to LS R4-2417510

Title:	Reply to LS on carrierBandwidth configuration for less-than-5MHz carriers

Response to:	R4-2417510

Release:	Rel-18

Work Item:	NR_FR1_lessthan_5MHz_BW

Source:	RAN4

To:	RAN2

Cc: 	RAN1

Contact Person:

Name:	Mohammad Abdi Abyaneh

Email:		mohammad.abdi.abyaneh@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR XXXXX

1. Overall Description:

RAN4 would like to thank RAN2 for the questions.

RAN4 agreed update the TS 38.101-1 Table 5.4.3.1-3, CR XXXXX, to clearly distinguish between PBCH size and the transmission bandwidth configuration. How to configure IE carrierBandwidth, when the sync rasters at

920.73, 921.45 MHz is given in Table 1:

Table 1 SSB BW and transmission BW configuration

|  | sync raster points (TS 38.101-1) | Transmission BW (carrierBandwidth) (PRB) | SSB BW (PRB) |
| --- | --- | --- | --- |
| 3 MHz | Table 5.4.3.1-3 (920.73 MHz) | 15 | 12 |
| 5 MHz | Table 5.4.3.1-3 (921.45 MHz) | 25 | 20 |

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to consider the aforementioned points.

# Dates of next TSG RAN WG4 meetings

RAN4#114	      February 17 – 21, 2025		Athens, Greece

RAN4#114bis	April 7 – 11, 2025		         TBC, China