3GPP TSG-RAN WG4 Meeting #113	R4-2419176

Orlando, US, 18th – 22nd November, 2024

Source:	Nokia

Title:	On General Aspects of AIML Mobility

Agenda item:	7.18.1

Document for:	Discussion

Introduction

RAN4#112bis meeting was the first one where the discussion of FS_NR_AIML_air took place. At that meeting it was agreed that the study will start with the RRM measurement prediction use-case [1]:

Issue 1-1-1: Priorities of different use cases Agreement: - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting. Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

This is also inline with the RAN4 working plan [2] that was also agree at RAN4#112bis.

In addition to the agreement on the prioritization of RRM measurement prediction use-case, the priority of scenarios and sub use-case was discussed at RAN4#112bis. Three of the scenarios that have the highest priority in RAN2 were selected to start the study in RAN4. Additionally, each scenario can be discussed for the three possible types of prediction (referred as sub-use-cases):

Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Agreement: - Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

In this contribution, we elaborate what scenarios should be still studied in RAN4 and whether it is possible to limit the number of use-case per scenario to streamline the analysis.

Discussion

On sub-use-case priorities

The high-level measurements model is described in Figure 9.2.4-1 of TS 38.300, presented in Figure 1 below.

Figure 1: High-level measurement model from TS 38.300.

The sub-use-cases 1-3 that define the possible approaches for the measurements prediction model can be mapped to the following points in Figure 1:

- Sub-use case 1: Inputs to the prediction model are L1 beam-level measurements, i.e., point A or A1, the outputs are also L1 beam-level predictions that are used to generate L3 cell-level results (point C) that follow currently specified beam consolidation and L3 filtering procedure.

- Sub-use case 2: Inputs and outputs of the model are the cell-level L3 measurements results, i.e., at point C. Therefore, the inputs to the model are generated following currently specified procedure for the derivation of L3 cell-level measurements.

- Sub-use case 3: Inputs to the model are actual L1 beam-level measurements, i.e., A or A1, and the outputs are directly the L3 cell-level measurements, i.e., at point C. Therefore, the procedure that include beam consolidation and L3 filtering are substituted by the model.

The objective of the study item [3] in respect of the AI/ML based RRM measurement prediction are the following:

- AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

It can be seen that the primary objective if the SI is to study cell-level measurement prediction, but beam-level measurement prediction might be still in the scope for inter-cell scenario. On the other hand, at RAN2#127, RAN2 agreed to consider L3 filtered beam-level predictions (i.e., point E) with lower priority:

Agreements - Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results. - If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A

RAN2 agreed that L3 filtered beam level prediction cases are lower priority.

RAN4 to focus initially the prediction of L3 cell-level measurement results and consider L3 filtered beam level prediction following RAN2 progress.

The goal of all high-priority sub-use-cases is prediction of L3 cell-level measurement result. From RAN4 point of view it could be an implementation issue how these L3 cell-level measurements are generated because the requirements in RAN4 will ultimately depend on the measurements that the device is performing and on the reporting that it is providing. In practice, in any of the sub-use-cases the device will need to perform L1 measurements of the beam(s) present in the testing setup and reporting on L3 cell-level measurements that can be observed outside of the device. If all possible implementation options need to be considered, i.e., 3 scenarios and 3 sub-use-cases per each scenario, then we get 9 cases in total. It could be challenging to consider all these combinations in the SI, and further prioritization is beneficial.

Moreover, even with legacy measurements reporting procedure it may not be known how UE is deriving L1 measurements that are up to implementation, and it could be that that are already some implicit reductions in measurements at this stage.

For cell-level predictions, the device will use beam-level measurements configured in the test even when the prediction model uses L3 measurements as an input. Then, only L3 cell-level measurements will be reported from the device even when L1 measurements are predicted. In this respect all sub-use-cases are similar, and it will be beneficial to down-scope the number of (scenarios x sub-use-cases) combinations based on better performance.

RAN4 to consider the sub-use case 2 (L3 Cell-level measurement result(s) are predicted based on actual L3 cell-level measurement result(s)) as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction.

On spatial domain prediction

In addition to the scenarios 2, 3, and 4 that are considered in RAN2 with high priority, there is still one more scenario 6, that got middle priority in RAN2, following the Table 5.2.1-2 from the draft TR 38.744:

scenario number; Priority; Evaluation scenario; Target study goal; Methodology. 6; Middle; FR2 to FR2 intra-frequency spatial domain; 1st goal; Intra-cell.

In RAN2#126, the following was agreed:

10 Methodology of Intra_F_C_S: Intra-frequency intra-cell spatial domain prediction is done by measuring sub set of configured SSB as input to the model to predict L3 cell level measurements for every instance of the same cell. It is only evaluted for FR2 intra-frequency scenario and RRM sub case 1 and 3. Several measurement reduction rates should be aligned among company without defining detail pattern. The detail rate values are FFS.

This scenario is notable due to the following two reasons:

- The spatial domain predictions are conceptually different from the time-domain predictions.

  - Time-domain predictions will require a certain number of measurements to perform the inference, i.e., an observation window. Moreover, the prediction in time domain is provided for one or several time instance(s) in the future, i.e., for the prediction window. Whereases, spatial domain predictions are done for every instance of the same cell.

  - Therefore, from the RAN4 point of view, time-domain and spatial-domain predictions may have different requirements.

- As it can be observed in our accompanying RAN2 paper [6], the Intra_F_C_S scenario is capable of providing good accuracy accompanied by measurement reduction. This can be achieved without introducing very complex complex models.

Intra-frequency intra-cell spatial domain prediction scenario is expected to have different RAN2 requirements from time domain prediction scenarios. This scenario provides high prediction accuracy without introducing complex models.

RAN4 to include FR2 to FR2 intra-frequency spatial domain scenario 6 into the scope of the Rel-19 study.

Conclusion

In this paper we discuss whether it is possible to limit the number of sub-use-cases per scenario and the importance of considering FR2 intra-frequency spatial domain scenario 6 in the study.

The following observations and proposals are made:

- RAN2 agreed that L3 filtered beam level prediction cases are lower priority.

- RAN4 to focus initially the prediction of L3 cell-level measurement results and consider L3 filtered beam level prediction following RAN2 progress.

For cell-level predictions, the device will use beam-level measurements configured in the test even when the prediction model uses L3 measurements as an input. Then, only L3 cell-level measurements will be reported from the device even when L1 measurements are predicted. In this respect all sub-use-cases are similar, and it will be beneficial to down-scope the number of (scenarios x sub-use-cases) combinations based on better performance.

RAN4 to consider the sub-use case 2 (L3 Cell-level measurement result(s) are predicted based on actual L3 cell-level measurement result(s)) as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction.

Intra-frequency intra-cell spatial domain prediction scenario is expected to have different RAN2 requirements from time domain prediction scenarios. This scenario provides high prediction accuracy without introducing complex models.

RAN4 to include FR2 to FR2 intra-frequency spatial domain scenario 6 into the scope of the Rel-19 study.

- References

- R4-2416855, WF on RRM requirements for FS_NR_AIML_Mob, Nokia, RAN4#112-bis, RAN4#112-bis, Hefei, China, 14th –18th October 2024.

- R4-2415968, Work Plan for RAN4 Discussions on AI/ML for Mobility in NR, Nokia, OPPO, Mediatek, RAN4#112-bis, Hefei, China, 14th –18th October 2024.

- RP-242393, Revised SID on AIML for mobility in NR , OPPO, 3GPP TSG RAN Plenary #105, Melbourne, Australia, September 9-12, 2024.

- R4- 2416449, On AI/ML Mobility Scenarios Evaluation and RRM Requirements Impacts, Nokia, RAN4#112-bis, Hefei, China, 14th –18th October 2024.

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Release 19)

- R2-2410796, On the RRM measurement prediction aspects, Nokia, RAN2#128, Orlando, USA, 18 – 22 November 2024.