3GPP TSG-RAN WG4 Meeting #113	R4-2419178

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	On AI/ML Based CSI Compression

Agenda item:	7.17.2.1

Document for:	Discussion

Introduction

This contribution is devoted to the discussion of requirements, testability, and interoperability aspects of AI/ML-enabled CSI reporting use-case.

The discussion of the approaches to define and derive reference and test encoder/decoder models for two-sided CSI compression has continued at RAN4#112bis [1]. It was generally agreed that the sufficient level of alignment was achieved based on the parameters agreed in at RAN4#111 [3]. For this meeting, following the agreement on AI/ML model and dataset logistics [4], we share our decoder and encoder models as described in mode details in our accompanying contribution [5].
Moreover, we check the performance of the trained models on the datasets provided by other companies (OPPO and CIATC) and mixed dataset. The new datasets can be different from the one used for training.

In this paper, we discuss the next steps of the study of Option 3 (fully specified test decoder) we elaborate on the derivation/selection of the test decoder and reference encoder following Option 4 (i.e., when the exact models are not expected to be specified). Finally, the question of verification for the test decoder is also treated.

Finally, additional data for better alignment of the datasets and SGCS results are shared in our accompanying contribution [6]:

- Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

Discussion

Results of decoder/encoder training

According to the agreements from the previous meeting RAN4#112 [2], the following CNN based reference encoder and decoder model parameters were agreed:

- For the results with quantization, to use a sigmoid function in the encoder for normalization

- Sigmoid in the decoder is optional so it was not used by us

- Not to use drop-out layers

- Use (1-SGCS) as loss function as reference

- Use Adam optimizer

- Fixed learning rate of 10^-4

- Batch Size: 256

- 150 epochs

- Stopping Criterion: No improvement over 25 epochs.

- Weight initialization: Default

- Kernel Parameters: Default

With these agreed parameter settings also summarized in Table 1, we have updated the CNN based reference encoder and decoder model performance, as shown in Table 2. The encoder and decoder were trained following Type 1 procedure (i.e., joint training) on a 630K sample dataset with 80% used for training, 10% for validation, and 10% for testing. The total trainable parameters are about 1.93M, with achieved mean training and testing SGCS performance of 0.721 and 0.705 at 64 bits overhead, respectively. It is worth nothing that no dropout layer is used so the training and testing SGCS performance have some slight difference. Compared to the Rel16 eTypeII codebook (0.691 mean SGCS for parameter combination 1), 2% SGCS gain is observed.

Table 1: Agreed Hyperparameters used during Type I joint training.

| Hyperparameter | Value |
| --- | --- |
| Optimizer | Adam |
| No of Epochs | 150 |
| Batch Size | 256 |
| Learning rate | 10-4 |
| Validation patience | 25 |
| Loss function | 1-SGCS |
| L2 regularization coefficient (λ) | 0 |
| Uniform Scalar Quantization | 2 bits |

The same model, with ideal hyperparameter settings, is also trained on datasets publicly shared from OPPO and CAICT, containing 600K and 100K samples, respectively. The training and testing SGCS performances are detailed in Table 2. Given that the validation patience is set to 25, the actual number of training epochs is less than 150 epochs for all three datasets. Table 3 presents the training and testing SGCS performances for the same model, with ideal hyperparameter settings, but trained without quantization to eliminate the impact of quantization.

Table 2: Trained AI/ML encoder-decoder pair SGCS performance with 2-bit quantization and 64-bits overhead.

| Dataset | Samples | Training SGCS | Testing SGCS | Epoch | Model Parameters | Rel16 SGCS | SGCS gain Rel 16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NOKIA | 630K | 0.721 | 0.705 | 99 | 1.93M | 0.691 | 2.0% |
| OPPO | 600K | 0.728 | 0.703 | 138 |  | - | - |
| CAICT | 100K | 0.715 | 0.676 | 75 |  | - | - |
| Mixed | 300K | 0.710 | 0.686 | 73 |  | - | - |

Table 3: SGCS of Reference CNN model without Quantization.

| Dataset | Samples | Training SGCS | Testing SGCS | Epoch | Model Parameters |
| --- | --- | --- | --- | --- | --- |
| NOKIA | 630K | 0.769 | 0.755 | 150 | 1.93M |
| OPPO | 600K | 0.775 | 0.752 | 150 |  |
| CAICT | 100K | 0.766 | 0.729 | 80 |  |
| Mixed | 300K | 0.761 | 0.718 | 143 |  |

Finally, to gain a comprehensive understanding of the differences between datasets, cross-validation was performed using models trained on the NOKIA dataset, OPPO dataset, CAICT dataset, and a Mixed dataset comprising 300,000 samples, with 100,000 samples from each individual dataset. Each model was tested on both its respective dataset and the mixed dataset. The results indicate that models trained on individual datasets do not generalize well across different datasets. However, the model trained on the mixed dataset demonstrates relatively equal performance across various datasets. The overall performance on Mixed dataset is limited to the limited number of samples of this dataset, i.e., 300K instead of ~600K.

Table 4: Cross-validation of encoder-decoder par with different datasets.

| Train on |  | Test on |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NOKIA | OPPO | CAICT | Mixed |
| NOKIA | 630K | 0.705 | 0.645 | 0.595 | 0.648 |
| OPPO | 600K | 0.626 | 0.707 | 0.639 | 0.657 |
| CAICT | 100K | 0.497 | 0.586 | 0.677 | 0.587 |
| Mixed | 300K | 0.680 | 0.689 | 0.688 | 0.686 |

As shown in Tables 2-4 above, the trained reference encoder-decoder pairs exhibited dataset dependency, indicating that a model trained on a single dataset may not perform well on other datasets, even under identical scenario settings. Training on a mixed dataset significantly enhances the model’s generalization ability, with larger mixed datasets yielding better performance.

Derivation of test decoder and encoder with Option 3

At RAN4#112 meeting the following steps were agreed to proceed with the study of test decoder derivation following Option 3, i.e., when the test decoder is expected to be fully specified in RAN4 including the architecture/backbone/parameters and the values of the weights [2]:

Issue 4-1: Option 3 next steps - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

As it can be seen, there is several steps in the plan that require further discussion.

The problem of training the test decoder by each of the companies individually is that it is harder to justify and explain the potential differences in performance. Moreover, quite probably, companies will use Type 1 training (i.e., joint training as a single entity) that could provide too optimistic performance results in comparison to Type 3 training (i.e., separate or sequential training) that is more practical when the encoder training is done after the test decoder is already trained.

Dataset aggregation can provide additional means of comparison and validation for the test decoder. The aggregated/mixed dataset can be used for objective comparison of encoder/decoder pair that was trained by different companies.

It is enough if such a verification dataset can contain only eigenvectors {V} that are coming to the input of the encoder. The same V from the dataset can be also used for the calculation of the SGCS after decoding.

An aggregated/mixed dataset {V} of channel eigenvectors can be used for objective performance comparison/validation of test decoders contributed by the companies.

The next question is how such aggregated dataset can be created. If different companies select different parts of the datasets from other companies for aggregation then it could be still variations in training and performance evaluation. Therefore, it makes sense to create such aggregated dataset following certain rules. One option could be to select only certain number of first samples from each of the dataset contributed by the companies and add those to the mixed dataset. The size of the sample should be defined by the number of contributing datasets, and the size of the aggregated dataset should be on the level agreed before, i.e., 600K samples. For example, if 6 companies are contributing to the aggregated dataset, then 100K first samples from each dataset will be used for aggregation.

- RAN4 to use an aggregated dataset of eigenvectors {V} for training and comparison of the performance of encoders/decoders provided by the companies.

Aggregated dataset should be created by taking A first sample from each of the contributing datasets, where A = 600 000 / n, and n is the number of contributing datasets.

- A single decoder demonstrating SGCS performance closest to be average across companies on aggregated dataset can be selected for the further study as a test decoder.

Following the procedure proposed in steps 3-5, a single decoder can be selected.

Next, in the Step (4), the companies can train their “own” encoders based on the known decoder. At this step, two main challenges can be present:

- Whether companies are allowed to use their own encoder architectures?

- Which dataset to use to train the encoder?

If we try to mimic the realistic scenario when companies train their own encoders on their own data. Additionally the architecture of the encoder does not mandatorily needs to be fixed and follow the CNN architecture anymore.

“Own encoders” should be trained on the company-specific dataset.

“Own encoders” trained for the selected test decoder should not follow the agreed CNN architecture internally. The output size and quantization should be kept the same.

Moreover, it should be verified that the performance of the own encoder is not worse than the performance of the CNN-based/reference encoder. This can be used as an additional criterion for the verification of Option 3, step 4:

It should be confirmed that the performance of “own” encoder is not worse than the performance of the CNN-based encoder.

The encoders are trained based on selected decoder, and it will be necessary to compare the performance of these encoders with selected decoder(s). The question is again what will be criteria for the comparison of the performance? The same issues as discussed above arises: the encoder trained and tested on one dataset may not show the same level of performance on a different dataset, e.g., on the channel generated in the testing setup.

The encoder trained based on known test decoder and company own dataset can demonstrate difference performance on different datasets/channel implementations coming from different test equipment vendors.

Therefore, to avoid unrealistic evaluation of performance based on the same dataset type that was use for training, the only way is to check the performance on the other datasets and compare the variation of performance on those. If a large degradation is not observed that would mean that the encoder/decoder couple can be sued in different testing setups.

- To conclude about the feasibility of Option 3, RAN4 needs to ensure that there is no significant performance (SGCS) variation in between “own” encoder – test decoder(s) in datasets from different companies.

Finally, since (relative) user throughput is usually considered in RAN4 in the requirements, some TPut-based metric should be studied in addition to SGCS-based.

- RAN4 would need to check the span of the throughput-based test metric in between the companies to conclude about the success of Option3 feasibility study.

Overall, we can summarize the feasibility evaluation steps for Option 3 in the following way:

- Check on performance alignment -> see simulation results from contributing companies

  - repeat simulations until good alignment is achieved

  - move to next step after alignment

- Share models(encoder or decoder or both)/datasets (training/testing/inference)

  - framework to share data among companies(dataset/model) to be discussed

- Create a fixed aggregated dataset based on companies’ contributions and train the decoder based on this dataset

  - Compare the performances and select single decoder, e.g., demonstrating average performance.

- Each company brings results for training of “own encoder” with selected decoder

  - Training is done using “own” dataset

  - It is verified that the performance of “own encoder” is not worse than for the agreed CNN-based encoder

- Conclude on overall feasibility of Option 3

  - The performance variation across the companies and datasets are within certain limits

  - The throughput-based performance metric variations are within certain limits

RAN4 to consider the steps 3-5 above to continue the feasibility check of Option 3.

On Option 4

At the previous RAN4#112 meeting, the following sub-options were agreed to proceed with the study of test decoder specification Option 4, i.e., when the test decoder is expected to be partially specified:

Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors - FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors - FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

At RAN4#112bis the following way forward was agreed:

Way forwards - Interested companies are invited to bring further proposals for the next steps for the feasibility study of Option 4(for any of the sub-options of Option 4) - Analysis of commonality with option 3(whether any steps or data generated for Option 3 can be reused) would be useful

In our contribution to RAN4#112bis [1], we proposed procedures to evaluate the feasibility of Options 4a and 4b, which are repeated here for reference:

Option 4a (Data-based) Proposal from Nokia in RAN4#112bis:

- Generate a dataset of channel/eigenvector realizations {V}.

- Create reference encoder/decoder.

- Generate dataset {V, c} – standardized, common dataset.

- Companies design their own test decoders (replicating the design process of TE vendors) using their architecture of choice, training and verifying them using the standardized {V, c}.

- Companies train their own encoder (replicating the UE vendor design process) using an architecture of their choice.

  - For training their encoders, companies could choose between two alternatives:
Alternative 1: Use the standardized dataset
Alternative 2: Use their own dataset

- Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders).

Option 4b (Reference encoder-based) Proposal from Nokia in RAN4#112bis:

- Generate a dataset of channel/eigenvector realizations {V}.

- Create reference encoder/decoder.

- Create a verification test dataset (common).

- Each company (mimicking a test equipment vendor) generates their own training dataset {V, c} using the common reference encoder to create the encoder output c from the encoder input V.

- Companies design their own test decoders (replicating the design process of TE vendors) using their architecture of choice, training them using the dataset {V, c} from the previous step and verifying them using the verification dataset from Step 3.

- Companies train their own encoder (mimicking a UE vendor) using an architecture of their choice, training the encoder using a dataset different from the dataset in the previous step.

- Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders).

We now consider the ongoing feasibility study for Option 3 in the same light as these proposed procedures in order to understand the commonalities and differences in the procedures so that the feasibility study is a reasonable representation of the expected procedure when used among network, UE, and test equipment vendors.  The Option 3 feasibility study procedure can be represented as:

- Generate a dataset of channel/eigenvector realizations {V}.

- Create reference encoder/decoder. The test decoder is the reference decoder.

- Create a verification test dataset (common) to be used as a basis for evaluation of the company-specific encoders.

- Companies train their own encoder (replicating the UE vendor design process) using an architecture of their choice.

  - For training their encoders, companies could choose between two alternatives:
Alternative 1: Use a common, shared dataset
Alternative 2: Use their own dataset

- Perform testing of all the UE encoders using the test/reference decoder (replicating the process of testing UE’s against RAN4 requirements the decoder shared by TE vendor decoders).

Steps 1 and 2 are currently in process in the Option 3 feasibility study. The study is currently doing performance alignment on the fully specified reference encoder/decoder pair where we are learning the consequences of shared, mixed, or individual datasets for {V}. The study is also preparing to select a test decoder in step 2 by selecting one decoder from among those submitted by different companies. At the end of step 2, the reference encoder and decoder will have been selected. At that point, the test decoder is identical to the reference decoder and is shared across the test equipment vendors. A common verification test dataset is then determined for later performance tests. Companies will then, acting as UE vendors, design and train their own encoders using either a common, shared dataset or using their own dataset. In either case, the dataset contains target CSI for the encoder input V and encoder output c determined using the reference encoder. The UE encoders from Step 4 are then tested using the common verification test dataset.

When the feasibility procedures are compared for Options 3, 4a, and 4b, we see some commonality. The first two steps of the process are the same since all procedures need a reference encoder and decoder. Therefore, these steps can be shared between the studies, noting that the difference lies in the purposes of the reference models. For Option 3, it is important to note that the reference decoder is the specified test decoder, common to all test equipment vendors. For Option 4a, the reference encoder and decoder are used as a means to obtain the standardized dataset for test decoder training.  For Option 4b, the reference encoder is specified for use in developing individual test decoders. The third step of all three procedures also contains some commonality. If the first two steps are shared, as we are proposing, then the common verification test dataset can also be shared across procedures. Finally, the last two steps of the three procedures, where the UE encoders are developed and tested, are the same.  However, while the processes for training and testing the UE encoders are the same in each procedure, the encoders themselves are different since they are developed under different specification assumptions.  So, the shared aspect for these two steps is the process which is carried out, but not the encoders which are developed and tests.

The first two steps of the feasibility study procedures for Options 3, 4a, and 4b can be shared, utilizing the same reference encoder and decoder.

The reference encoder and decoder obtained in the first two steps of the feasibility study procedures for Options 3, 4a, and 4b are utilized for different purposes in each of the procedures.

There are common processes used in the development and evaluation of the UE encoders across the feasibility study procedures for Options 3, 4a, and 4b.

Share the agreed reference encoder and decoder across the feasibility study procedures for Options 3, 4a, and 4b, utilizing those in a manner appropriate for each option.

  - In Option 4a, CNN-based encoder and test decoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated dataset

  - In Option 4b, CNN-based encoder matching the selected test decoder from Option 3 can be used.

Conclusion

In this contribution we discuss different approach to derive test decoder for two-sided AIML CSI compression model.

The following Observations and Proposals were made:

- As shown in Tables 2-4 above, the trained reference encoder-decoder pairs exhibited dataset dependency, indicating that a model trained on a single dataset may not perform well on other datasets, even under identical scenario settings. Training on a mixed dataset significantly enhances the model’s generalization ability, with larger mixed datasets yielding better performance.

On Option 3:

- RAN4 to use an aggregated dataset of eigenvectors {V} for training and comparison of the performance of encoders/decoders provided by the companies.

Aggregated dataset should be created by taking A first sample from each of the contributing datasets, where A = 600 000 / n, and n is the number of contributing datasets.

- A single decoder demonstrating SGCS performance closest to be average across companies on aggregated dataset can be selected for the further study as a test decoder.

- “Own encoders” should be trained on the company-specific dataset.

- “Own encoders” trained for the selected test decoder should not follow the agreed CNN architecture internally. The output size and quantization should be kept the same.

- It should be confirmed that the performance of “own” encoder is not worse than the performance of the CNN-based encoder.

The encoder trained based on known test decoder and company own dataset can demonstrate difference performance on different datasets/channel implementations coming from different test equipment vendors.

- To conclude about the feasibility of Option 3, RAN4 needs to ensure that there is no significant performance (SGCS) variation in between “own” encoder – test decoder(s) in datasets from different companies.

- RAN4 would need to check the span of the throughput-based test metric in between the companies to conclude about the success of Option3 feasibility study.

- Create a fixed aggregated dataset based on companies’ contributions and train the decoder based on this dataset

  - Compare the performances and select single decoder, e.g., demonstrating average performance.

- Each company brings results for training of “own encoder” with selected decoder

  - Training is done using “own” dataset

  - It is verified that the performance of “own encoder” is not worse than for the agreed CNN-based encoder

- Conclude on overall feasibility of Option 3

  - The performance variation across the companies and datasets are within certain limits

  - The throughput-based performance metric variations are within certain limits

RAN4 to consider the steps 3-5 above to continue the feasibility check of Option 3.

On Option 4:

The first two steps of the feasibility study procedures for Options 3, 4a, and 4b can be shared, utilizing the same reference encoder and decoder.

The reference encoder and decoder obtained in the first two steps of the feasibility study procedures for Options 3, 4a, and 4b are utilized for different purposes in each of the procedures.

There are common processes used in the development and evaluation of the UE encoders across the feasibility study procedures for Options 3, 4a, and 4b.

Share the agreed reference encoder and decoder across the feasibility study procedures for Options 3, 4a, and 4b, utilizing those in a manner appropriate for each option.

  - In Option 4a, CNN-based encoder and test decoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated dataset

  - In Option 4b, CNN-based encoder matching the selected test decoder from Option 3 can be used.

- References

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm, RAN4#112-bis, Hefei, Anhui, China, 14th – 18th October, 2024.

- R4-2414323, WF on AI/ML, Qualcomm, RAN4#112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024.

- R4-2414447, Alignment of model training and data set parameters for AI/ML-based CSI compression, Nokia, RAN4#112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024.

- R4-2417207, WF on data sharing for AI/ML air interface, Qualcomm, RAN4#112-bis, Hefei, Anhui, China, 14th – 18th October, 2024.

- R4-2419180, On AI/ML Two-Sided Model Logistical Issues, Nokia, RAN4#113, Orlando, US, Nov 18th – Nov 22nd, 2024.

- R4-2419179, Additional Data for Alignment for CSI Compression Use-Case Study, Nokia, RAN4#113, Orlando, US, Nov 18th – Nov 22nd, 2024.