3GPP TSG-RAN WG4 Meeting #113	R4-2419180

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	On AI/ML Two-Sided Model Logistical Issues

Agenda item:	7.17.2.2

Document for:	Discussion

Introduction

At the previous RAN4#112bis meeting the general principles of model and dataset sharing were agreed for the continuation of the study of AI/ML enabled CSI compression use-case in RAN4 [1].

In this contribution, we share our view on the retention period of data, elaborate on the FTP connection speed and describe the data we shared.

Discussion

On retention period of shared models and datasets

In addition to the type of shared data there another important aspects that need to be treated: The retention period of shared data, i.e., for how long the models and the datasets need to be stored. It will be necessary to distinguish

- whether the model/dataset is shared in between the companies during the study, i.e., only for the study purposes,

- or if it needs to be captured as a part of the TR or potentially even of the TS.

In the first case, it will be sufficient to store data only during the timeline of the SI or WI, i.e., for 18 -24 month.
However, in the second case, the same retention policy as for regular TDocs should be applied. It will be reasonable to assume that the upper bound of the storage needed for such data is not more than listed above for one meeting for all companies, i.e., 160 GB for dataset(s) and 1.6 GB for models.

The retention period of the models/datasets shared only for the study purposes can be the length of the 3GPP release, i.e., 18-24 month.

The retention policy of the models/datasets agreed to be captured in the outcome of the study or as a part of the TR can be the same as for regular TDocs/TRs.

Based on the preliminary discussions with MCC, it became clear that the current practice of uploading 3GPP TDocs is not very appropriate for the AIML models and datasets used for study purposes, at least because of the TDoc retention policy (i.e., all uploaded TDocs needs to be stored for backed-up for a very long time) and due to the size of the files, especially datasets. On the other hand, we understood that FTP will stay as the file exchange platform. FTP over TLS (FTPS) and HTTP interface are also considered as possible enhancements by MCC. We also would like to suggest enabling Large File Support (LFS) to support large file transfers (files larger than 2GB).

Due to the large size of the files to be exchanged (especially for the datasets) the sufficient upload/download speeds need to be ensured and Large File Support (LFS) for the FTP server would be beneficial.

Shared models and dataset

At RAN4#112bis it was agreed to share the AIML-based CSI encoder and decoder models and datasets [2]:

Issue 4-1: Simulation results discussion for Option 3 Agreement: Next step: - Interested companies to share decoder and encoder ML models and dataset in next meeting (RAN4#113) - Model and dataset used to generate the data submitted to RAN4#112-bis Files to be shared Agreement: Files to be shared, to be decided based on option under study - Dataset containing encoder input per subband (as described in Section 5-3) - Encoder and/or decoder model (as described in Section 5-1)

Our dataset is shared in

Directory Listing /ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Datasets/R4_113

And our models are shared in

Directory Listing /ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Datasets/R4_113

The dimensions of the array stored in the file “N0KIR4_113dsei.npy” are N (samples) × 2 (IQ) × nSB (number of sub-bands) × nPorts (number of CSI-RS ports) × nLayers (number of layers) with specific values of 630000 × 2 × 13 × 32. All the elements of the dataset are float32 real numbers. The Network parameters used in generating this data are as follows:

| Parameter Name | Parameter value |
| --- | --- |
| Simulation Scenario | UMa |
| Pathloss Model | NR UMa |
| The 'shape' of the macrocell layout | hexagonal |
| Number of cell-sites | 7 |
| BS antenna height in meters | 25 |
| The number of UE's to drop per macrocell sector | 100 |
| Distribution of UEs (outdoor pedestrian, indoor, outdoor in-car) | [ 0.2 0.8 0 ] |
| UE antenna height in meters above the floor level in meters | 1.5 |
| Minimum UE distance to macrocell in meters | 35 |
| Macrocell inter-site distance in meters | 200 |
| Number of macro sectors per site | 3 |
| Initial sector azimuth (the azimuth of the first sector at a macrocell) in degrees | 90 |
| The height of a building floor in meters | 3 |
| Maximum indoor distance to assign to a UE in meters | 25 |
| building Penetration Loss | low-->50%, high-->50% |
| range of building heights in floors | [4, 8] |
| Carrier Freqquency | 2 GHz |
| UE antenna element configurations | (1,2,2,1,1,1,2) |
| UE virtualized antenna element configurations | 1*2*2 (virtual rows * virtual columns * polarizations) |
| UE antenna Element spacing | 0.5 lambda between columns |
| gNB antenna element configurations | (8,8,2,1,1,2,8) |
| gNB virtualized antenna port configuration | 2*8*2 (virtual rows * virtual columns * polarizations) |
| gNB antenna Element spacing | 0.5 lambda between columns and 0.8 lambda between rows |

The eigenvectors are normalized over each individual sub-band so that the coefficient of the first "port" of each sub-band is always a positive real number. Each eigenvector coefficient within a sub-band is multiplied by exp(-1i * θ), where θ is the angle component of the coefficient of the first port of the first PRB in each sub-band. Example: v_s = [ v1s v2s v3s ... vPs ].'  where s is the sub-band index, v_s is the unnormalized eigenvector for sub-band s, and P is the number of antenna ports.  Then θ_s = angle(v1s) and the phase normalized vectors are:  vn_s = exp(-1j* θ_s)*v_s  for s = 1,...,Nsub-bands

We have uploaded our trained reference encoder and decoder model in ONNX format as agreed during the previous RAN4#112bis meeting. The files are named ‘N0KIR4_113mlec.onnx’ (encoder) and ‘N0KIR4_113mldc.onnx’ (decoder). There might be some errors in converting the trained models from PyTorch to ONNX formats, which is due to version compatibility issues. For our setup, we used PyTorch version 2.4.1 and ONNX version 1.16.2, which worked well with the agreed CNN-based reference autoencoder model.

The encoder takes an input tensor with dimensions (N_batch, 2, 13, 32) and outputs a quantized latent vector of 32 dimensions, specifically (N_sub, 32). This vector uses 2-bit scalar quantization, where each of the 32 elements takes one of four distinct values: -0.75, -0.25, 0.25, or 0.75. Each of these values corresponds to 2 digital bits. The decoder model then takes this quantized latent vector and reconstructs an output tensor of dimensions (N_batch, 2, 13, 32).

RAN4 to consider shared encoder and decoder models, and the dataset containing encoder input per subband for the further study of CSI compression use-case and test decoder derivation.

Conclusion

In this contribution we shared our view on model and dataset retention time, recommended to increase the upload/download speed for the dataset, and described the models and the dataset we shared.

The following Observations and Proposals were made:

- The retention period of the models/datasets shared only for the study purposes can be the length of the 3GPP release, i.e., 18-24 month.

The retention policy of the models/datasets agreed to be captured in the outcome of the study or as a part of the TR can be the same as for regular TDocs/TRs.

Due to the large size of the files to be exchanged (especially for the datasets) the sufficient upload/download speeds need to be ensured and Large File Support (LFS) for the FTP server would be beneficial.

RAN4 to consider shared encoder and decoder models, and the dataset containing encoder input per subband for the further study of CSI compression use-case and test decoder derivation.

- References

- R4-2417207, WF on data sharing for AI/ML air interface, Qualcomm, RAN4#112-bis, Hefei, Anhui, China, 14th – 18th October, 2024.

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm, RAN4#112-bis, Hefei, Anhui, China, 14th – 18th October, 2024.