3GPP TSG-RAN WG4 Meeting #113	R4-2419187

Orlando, US, 18th – 22nd November, 2024

Source:	Nokia

Title:	Testability and Interoperability Issues for AIML Mobility

Agenda item:	7.18.3

Document for:	Discussion

Introduction

RAN4 discussion on the AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR (FS_NR_AIML_Mob study item) started from RAN4#112bis. The recent RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In RAN4#113 meeting, following aspects are to be discussed during this meeting.

- General Aspects of AI/ML Mobility

- RRM Requirements of AI/ML Mobility

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution we provide our views and discuss different aspects of Testability and Interoperability issues in RAN4 for AI/ML Mobility based on the RAN2 framework.

Discussion

Interoperability and testability within the 3GPP framework are crucial for the success deployment of AI/ML solutions. Specially, when the part of AI/ML functionality are at UE side, the NW needs to properly validate the functionality. Interoperability allows AI/ML based mobility-driven solutions such as measurement prediction, measurement event prediction, and RLF prediction operate within diverse set of networks environments, including devices from different vendors. This enables improved network optimization, radio resource management, and predictive maintenance that results in seamless, consistent, and enhanced user experience.

Based on the current RAN2 agreements, following scenarios/use cases have been potentially considered for AI/ML mobility.

- Measurement Prediction

- Handover and Radio Link Failure Prediction, and

- Measurement Event Prediction

Following agreement was achieved in RAN4#112bis meeting on the priorities of different use cases:

Agreement: - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting.

For measurement event prediction use-case and RLF use case, following agreement was achieved in RAN4#112bis meeting:

Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

Therefore, in this contribution, we will focus on the testability and interoperability aspects of RRM measurement prediction use case. In below, for RRM measurement prediction use case, we provide some our views on issues related to testability and interoperability aspects, as listed below:

- Testing goals

- New Testability aspects

- Principle of FR2 testing and new FR2 testing aspects

- Consistency between different Carriers

RRM core requirements related topics are discussed in our accompanying paper [2].

Testing goals

As it is recommended in the study item description that AI/ML mobility study should leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability, RAN4 should consider reusing testing goal definition from AI/ML or NR air interface WI.

Based on the recent agreements in RAN1 and RAN2, AI/ML-enabled Feature, functionality (to be interpreted here as indicating the set of parameters and their values as supported for a particular feature configuration based on the UE capabilities), Associated ID and applicability of the functionality can be interpreted in the following manner:

- Parameters that are defined by the feature functionality/configuration based on the UE capabilities

- Parameters that are defined by the Associated IDs (NW side additional conditions, i.e., NW-related parameters) that help the UE to determine/check the functionality applicability and enable consistency between training and inference conditions for the UE-side ML functionalities.

- Other/additional parameters that cannot be defined neither by the feature functionality/configuration nor include in the Associated ID e.g., propagation conditions/channel model, UE-side configurations/additional conditions, etc.

In RAN4, as part of conformance testing, the minimum performance should be ensured by specific sub-set of application configurations (may also be referred as matching functionalities).

Each specified AI/ML-enabled Feature shall have RAN4 requirements for minimum performance and UE needs to pass at least one test for specific sub-set of applicable configurations (matching functionalities) as part of conformance testing.

A minimum set of test configurations (matching Associated ID(s)) shall be defined as mandatory for testing of AI/ML-enabled Feature.

New Testability Aspects

In RAN4#112bis meeting, following agreement was achieved on the new testability aspects for AI/ML mobility use cases in comparison of AI/ML NR for air interface WI.

- Agreement - According to the SID, RAN4 to leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - RAN4 will need to consider at least the following testability aspects for AI/ML-enabled mobility SI: - Testing in FR1 - Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction (for scenario 3) - Testing cell-level L3-RSRP measurements - Consideration of serving and neighbor cells

In addition of above agreed aspects, there are other new aspects to be considered for testability and interoperability of AI/ML mobility SI. For instance, due to the presence of inter-cell inter-frequency scenarios (scenario 3) in measurement prediction use case, network assistant information may be different for BM use case of AI/ML NR for air interface and AI/ML mobility use case. In contrast to BM use case, AI/ML mobility use case may have multiple target cells/sites in the same or different frequency layers.

Furthermore, the moving UE trajectory in case of AI/ML will cause time-varying received power and SINR. RAN4 will need to study how to model different time-varying characteristics per cell/site as well as how to model different variation per test iteration.

For AI/ML mobility, RAN4 should consider different NW side additional conditions (i.e., NW-related parameters) for multiple target cells/sites in the same or different frequency layers.

RAN4 should also study how to model different time-varying characteristics per cell/site due to moving UE trajectories, and how to model different variations per test iteration.

One of the considerations for RAN4 in establishing RRM requirements and defining test cases for measurement prediction is the effect of L3 filtering and the number of beams utilized within test environments. Currently, a majority of legacy test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero). Additionally, to streamline test configurations, only a limited number of beams are typically simulated in the test setup.

Figure 1: 3GPP TS 38.300 Measurement Model

As illustrated in Figure 1, when L3 filtering is absent, the L1 and L3 beam measurement levels at point A1 align. Furthermore, for L3 cell-level RSRP predictions at Point C, the predicted values may correspond to the L1 and L3 beam-level predictions when a single beam is employed for prediction purposes. This approach may lead to inefficient or inaccurate test design and UE performance evaluation in predicting L1/L3/cell level measurement as the lack of filtering and limited beam emulation can result in an oversimplified model that fails to capture realistic multi-beam scenarios, thereby impacting the reliability of performance assessments.

- Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup.

- Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E.

- RAN4 to consider L3 filtering and multi-beam scenarios when studying test cases for AI/ML mobility uses cases.

Considering measurement prediction use case within AI/ML mobility, the testing will involve either L1 or L3 measurements or both, either measured or predicted, on a preselected set of cells / beams (either intra or inter frequency). The considered trained ML model outputs predicted measurements on a preselected time horizon. It is therefore necessary to map the measured values to the predicted value with the selected prediction horizon, accounting for the standardized filtering procedure when estimating L3 measurements based on L1 or L3 measurements.

RAN4 should define test setups considering L1 and L3 measurements from serving and neighbouring cells as well as AI/ML model with related configuration (inputs, prediction horizon).

Principle of FR2 testing and new FR2 testing aspects

Similar to AI/ML BM use case in WI AI/ML NR for air interface, OTA testing should be considered for FR2 in case of AI/ML mobility use cases. The testing setup that is under study in AI/ML BM use case should also be used as a baseline for AI/ML mobility testing.

For FR2, the under-study testing setup in AI/ML BM use case should be reused as a baseline for measurement prediction use case testing.

However, RAN4 should also consider new aspects of FR2 testing for AI/ML mobility use case. Some of the new aspects are listed below:

- Which channel model to be considered CDL or TDL

- Number of AoAs may be different from BM use case due to higher number of cells and beams requirements in AI/ML mobility use case.

- Maximum number of cells and Tx beams to be considered

- Number of probes in the test chamber

RAN4 should consider new aspects for FR2 testing in AI/ML mobility such as channel model, number of AoAs, maximum number of cells and Tx beams and number of probes required in the test chamber.

Consistency between different carriers

In AI/ML Mobility, in order to train and infer, AI/ML functionalities need a dataset/environment that guarantees certain consistency between primary and secondary carrier components. This constraint is not present in the AI/ML BM use case and therefore is not considered in the scope of ongoing WI. Therefore, in the scope of mobility SI, RAN4 should consider the methods to guarantee the consistency between different carriers.

RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to testability and interoperability of AI/ML Mobility use cases. Specifically, we cover following aspects:

- Testing goals

- New Testability aspects

- Principle of FR2 testing and new FR2 testing aspects

- Consistency between different Carriers

In the paper, the following Observations and Proposals were made:

Proposal 1: Each specified AI/ML-enabled Feature shall have RAN4 requirements for minimum performance and UE needs to pass at least one test for specific sub-set of applicable configurations (matching functionalities) as part of conformance testing.

Proposal 2: A minimum set of test configurations (matching Associated ID(s)) shall be defined as mandatory for testing of AI/ML-enabled Feature.

Proposal 3: For AI/ML mobility, RAN4 should consider different NW side additional conditions (i.e., NW-related parameters) for multiple target cells/sites in the same or different frequency layers.

Proposal 4:  RAN4 should also study how to model different time-varying characteristics per cell/site due to moving UE trajectories, and how to model different variations per test iteration.

Observation 1: Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup.

Observation 2: Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E.

Proposal 5: RAN4 to consider L3 filtering and multi-beam scenarios when studying test cases for AI/ML mobility uses cases.

Proposal 6: RAN4 should define test setups considering L1 and L3 measurements from serving and neighbouring cells as well as AI/ML model with related configuration (inputs, prediction horizon).

Proposal 7: For FR2, the under-study testing setup in AI/ML BM use case should be reused as a baseline for measurement prediction use case testing.

Proposal 8: RAN4 should consider new aspects for FR2 testing in AI/ML mobility such as channel model, number of AoAs, maximum number of cells and Tx beams and number of probes required in the test chamber.

Proposal 9: RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI.

- References

- RP-242393, Revised SID on AIML for mobility in NR , OPPO, 3GPP TSG RAN Plenary #105, Melbourne, Australia, September 9-12, 2024.

- R4- 2419186, On AI/ML Mobility Scenarios Evaluation and RRM Requirements Impacts, Nokia, RAN4#113, Orlando, US, 18 –22 November 2024.