3GPP TSG-RAN WG4 Meeting#113                              R4-2419202

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:	7.21.3.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement of Intermediate node (UE)

Document for:	Approval

Introduction

In the RAN#102 meeting, Study on solutions for Ambient IoT (Internet of Things) in NR has been approved and it’s expected to start the normative work from this April meeting. In this contribution, we would to share some initial views on the RF requirement for A-IoT intermediate note (UE).

Issue 2-2-4: Spectrum usage for R2D in D2T2 Agreement: - Use FDD UL spectrum for R2D in D2T2. Issue 2-2-5: Spectrum usage for CW transmission in D2T2 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. · Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum · Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum · Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering - Use UL spectrum as the starting point for co-existence evaluation. - It won’t preclude the use of DL for backscattering transmission. - FFS on the minimum distance between the intermediate UE and A-IoT device Issue 2-5-3: Evaluation cases for D2T2 for device 1 and 2a between NR and AIOT Agreement: - FFS on evaluation cases for D2T2 for device 1 and 2a between NR and AIOT. Corresponding evaluation cases are listed for information. Note that some duplicated cases are omitted in the table. Deployment scenario and topology; spectrum; aggressor; victim. Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum; R2D: UL CW2D and D2R: UL; CW and/or device; NR UL. NR UL; Device and/or reader. reader; NR UL. NR UL; device. \ Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum Self interference cancelation is needed for reader; R2D: UL CW2D and D2R: UL; CW and/or device; NR UL. NR UL; Device and/or reader. reader; NR UL. NR UL; device. Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Self interference cancelation is needed for reader; R2D: UL CW2D and D2R: UL; CW and/or device; NR UL. NR UL; Device and/or reader. reader; NR UL. NR UL; device. R2D: UL CW2D and D2R: DL; CW and/or device; NR DL. NR DL; device and/or reader.

Discussion

- 2.1. General aspects for Ambient intermediate node

- In the topology 2, intermediate node is used to transmit the inventory command towards the A-IoT device and also decoding the backscattering signal from the A-IoT device. Quite similar as A-IoT BS, we also need to consider the Tx/Rx requirement for intermediate UE;

- Proposal 1: specify the Tx and Rx requirement for intermediate UE in addition to RF requirements for the legacy Uu link.

- 2.2. RF requirement for Ambient intermediate node

- In the following section, we would like to share some initial understandings on the RF requirements for A-IoT device.

- Table 1. The initial analysis for RF requirement of A-IoT intermediate node in D2T2

| Tx part |  |
| --- | --- |
| Maximum output power | In last RAN4 meeting, it was agreed to use the UL spectrum for R2D transmission, therefore from our understanding, we could use the FDD PC3 as starting point for Ambient intermediate node transmission by reusing the same LPAMid module within UE. In addition, for CW transmission, it’s still not clear to use DL or UL spectrum for transmission, its maximum transmission power could be further discussed later on. |
| Output power dynamics | Some power boosting for OOK signal might be needed for in-band/guard band operation of A-IoT service if A-IoT intermediate node share the same hardware with legacy eMBB transmission in other PRBs e.g. sharing the same LPAMid module; |
| Transmit ON/OFF power | The transmit ON-OFF power is only limited for TDD bands, however A-IoT intermediate node might need to send the R2D signal and CW signal in the sequential way, then some transition period might be needed for the switch between R2D signal transmission and CW transmission in D2T2. or switch between R2D signal transmission, CW transmission and other eMBB transmission in in-band/guard band scenario under the shared RF architecture. |
| Transmitted signal quality | The legacy UE transmit frequency error requirement could be reused for A-IoT intermediate node. EVM for OOK signal is not needed since OOK signal is not mapped by the legacy constellation, instead it’s reflected in the envelope level as following. Tag also don’t need to have channel estimation and equalization to decode the OOK signal, instead of the power stability or power accuracy for OOK ON signal and OOK OFF signal is important and also power difference between OOK ON and OOK OFF is essential to ensure the tag OOK detection performance. Figure 2.4-1. time domain waveform for OOK-4 In addition, as pointed out in [3], RFID uses ASK modulation with PIE data encoding. The ASK modulation mask was defined as shown in Figure 2.4-2 below. The pulsewidth (PW), Modulation Depth ((A–B)/A), RF Envelope Ripple (M), RF envelope Rise time (tr,10–90), and RF envelope fall time (tf,10–90) are the major requirements. Figure 2-4-2: Interrogator-to-Tag RF envelope For A-IoT BS EVM requirement, we propose to define the EVM requirement similar as RFID with OOK signal considered. The specific EVM requirement could be finalized at WI phase. |
| OBW | The legacy UE OBW requirement could be reused for A-IoT intermediate node; |
| Tx intermodulation | The legacy UE transmitter intermodulation requirement is somehow agnostic to certain bands, therefore from our understanding, the legacy Tx intermodulation requirement is still applicable for Ambient intermediate node which is somehow different from A-IoT BS and A-IoT device. |
| ACLR | This depends on further coexistence study. |
| Operating band unwanted emissions | This depends on outcome of coexistence evaluation. |
| Transmitter spurious emissions | The legacy transmitter spurious emission requirement could be reused since this is coming from the ITU regulatory definition. |
| Rx part |  |
| REFSENS | The REFSENS requirement might be not based on the throughput metric and it should be dependent on the miss detection ratio and false alarm detection ratio instead if without any HARQ-ACK feedback. In addition, the impacts on CW signal transmission should be also taken into account especially for D2T2. For D2T2-A2 deployment scenarios, some self interference on Ambient intermediate node should be taken into account. |
| Maximum input power | Similar analysis for backscattering signal should be specified with measurement metric as miss detection ratio or false alarm detection ratio. |
| ACS | This depends on further coexistence study. |
| Blocking requirement | This depends on further coexistence study. |
| OOBB | Don’t see the reason not to reuse the -15dBm CW signal as interference signal of OOBB requirement. For f_OOBB requirement, this could be further discussed once we have more clear assumption on A-IoT intermediate node. |
| RX IMD | This is somehow similar as Tx intermodulation requirement. |
| Rx spurious emission | The legacy UE receiver spurious emission requirement could be applicable. |
| Receiver spurious response | This might be needed for D2R reception only. |

Conclusions

In this contribution, we shared some initial views on the RF requirement for A-IoT intermediate node and proposals are made as following:

- Proposal 1: specify the Tx and Rx requirement for intermediate UE in addition to RF requirements for the legacy Uu link.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

Annex

- Topology 2 includes the following cases e..g. Case 2-2, Case 2-3 and Case 2-4. The deployment layout is depicted in the following figure 4-6 according to the latest agreement reached in last RAN1 meeting.

NOTE 3:  This topology includes the possibility that the intermediate node transmitting to the Ambient IoT device is a different from the intermediate node receiving from the Ambient IoT device

- NOTE 4: the DL/UL spectrum used for inventory command is not decided yet.

- Figure 1. the illustration of Case 2-2 topology and its transmission signal

- Figure 2. the illustration of Case 2-3 topology and its transmission signal

- Figure 3. the illustration of Case 2-4 topology and its transmission signal