3GPP RAN WG4 Meeting #113		R4-2419245

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	On modification of existing RX RF requirements for SBFD

Agenda Item:	7.20.2.3

Document for:	Approval

Introduction

During RAN4#112, BS RF requirements for SBFD have been discussed [4]. In this contribution, we would like to share our views regarding the open issues.

Discussion

2.1 Reference sensitivity and OTA sensitivity

During Rel-18 SI, value range [0.5~1] dB has been considered for the allowed sensitivity degradation due to self-interference. RAN4 may need to continue the work from e.g. whether to consider interferences other than self-interference for OTA sensitivity.

Per WF in [4], the following WF was reached.

WF:

- FFS on the followings:

  - Option 1: 1.0 dB degradation

  - Option 2: 0.5 dB degradation

For the value range, ~1 dB sensitivity degradation was used in the SI in general. If considering overall impact to system performance, self-interference, inter-site interference, and inter-sector interference all will contribute on the sensitivity degradation. Considering there may be higher IoT in the real network, e.g. we define 20 dB dynamic range requirement to consider co-channel interference. And other receiver requirements, such as ACS and in-band blocking, the reference sensitivity degradation is normally defined as 6 dB for NR RX. Compared to these values, 1 dB is still a relative low value and then should be applied. It is proposed to add 1 dB addition to reference sensitivity due to self-interference. For example, the OTA sensitivity for SBFD slot can be defined as below,

Table 10.3.2-1: Wide Area BS reference sensitivity levels for SBFD slot

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -94.6 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 10.3.2-2: Medium Range BS reference sensitivity levels for SBFD slot

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -89.6 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 10.3.2-3: Local Area BS reference sensitivity levels for SBFD slot

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -86.6 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Proposal 1: for OTA reference sensitivity, it is proposed to add 1 dB addition to reference sensitivity due to self-interference.

2.2 Dynamic range requirement

Per WF in [4],

- FFS on the followings:

  - Option 1: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work.

  - Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations.

  - Option 3: Dynamic range of FR1/FR2-1 indoor scenario can reuse the existing requirement.

During Rel-18 SI, it is agreed that dynamic range is still applicable for SBFD capable BS. Receiver dynamic range requirement is to ensure that the base station can receive high throughput also in the presence of increased interference levels. For FR1 20 dB interference over thermal noise is used for NR and LTE. From a scenario of view, the case of co-channel interference from uplink signals is the same as normal NR and LTE. One question is whether BS2BS CLI signal shall be included. 20 dB IOT level is high enough to reflect an acceptable coverage. Hence there is no need to consider higher dynamic range.

Proposal 2: For FR1 BS 20 dB IoT level assumption is reused for dynamic range requirement.

For FR2, dynamic range requirement for range2 NR BS is not defined. Per TR 38.817-02, the absolute levels for demodulation requirements shall be well above (e.g. at least ~5dB) the AWGN noise floor, thus the small dynamic range shall be handled via demodulation requirements. Hence it needs to perform simulation to check whether larger dynamic range is needed for FR2 SBFD.

Proposal 3: it is proposed to perform simulation to check whether larger dynamic range is needed for FR2 SBFD.

2.3 ACS requirement

Per WF in [4],

Agreement:

- The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement.

WF:

- FFS on the followings:

  - If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed.

  - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

For ACS and in-band blocking, both requirements are defined out of the BS channel bandwidth instead of uplink subband bandwidth. ACS and in-band blocking are defined with different interferer frequency offsets at adjacent channel(s). For channel bandwidth wider than 20 MHz, ACS interferer frequency offset is ~9.5 MHz and in-band blocking interferer frequency offset is 30 MHz, using 20 MHz NR interfering signal. And value of the interferer level is 9 dB offset. Hence, we propose to consider the two requirements together.

Proposal 4: the definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level.

2.4 In-band blocking

Per WF in [4],

Agreement: - For FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements. WF: - FFS on the followings: - For FR1 WA BS, RAN4 need to discuss whether a unified in-band blocking requirement is defined? - Option 1: a unified blocking requirement to consider both scenarios: coexistence with legacy TDD system in adjacent channel, and coexistence with new SBFD system in adjacent channel. - Option 2: separate requirements are defined, - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - For FR1 WA BS co-existence study, FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - In-band blocking requirement for FR1 MR SBFD BS - In-band blocking requirement for FR2-1 WA and MR SBFD BS - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements.

In order to guarantee feasible co-existence performance between different BSs operating in adjacent frequency, in-band blocking requirement for SBFD should be studied since the blocker outside of carrier could be DL transmission from other BS, which is other than the UL transmission from other UEs had been considered for legacy requirement.

For Wide Area and MR BS, companies’ proposals in last RAN4#112 meeting are diverging on the blocking levels, e.g. from 5 dBm to -30 dBm. The requirement is to guarantee feasible co-existence performance between different BSs operating in adjacent frequency. We look at the consequence to co-existence for these values. The aggressor is TDD and the victim is SBFD UL. For legacy TDD ACLR, 50 dB typical value is adopted. From Table 2.4-1, IOT is about 20 dB when the power of adjacent NR interferer is -20 dBm due to the legacy TDD ACLR interference. It will not help the co-existence between a SBFD BS with capability to handle strong blocker and a legacy TDD BS with existing ACLR capability.

Table 2.4-1: IOT due to the legacy TDD ACLR

| Blocking level (dBm/100MHz) | Legacy TDD ACLR (dB) | Noise floor (dBm/40MHz) | IOT (dB) |
| --- | --- | --- | --- |
| 5 | 50 | -93 | 44 |
| 0 | 50 | -93 | 39 |
| -10 | 50 | -93 | 29 |
| -20 | 50 | -93 | 19 |
| -30 | 50 | -93 | 9 |
| -40 | 50 | -93 | -1 |

Observation 1: without improving the legacy TDD unwanted emission, only defining high blocking receiver requirement cannot guarantee the adjacent channel co-existence between a SBFD BS and a legacy TDD BS.

It may be not possible to change the OBUE requirement for legacy TDD BS. While for a new deployment, adjacent channel co-existence between a SBFD BS with another SBFD would be feasible by defining additional requirements both transmitter side and receiver side. As discussed in TX part, additional attenuation is achievable by using sub-band RF filter.

Table 2.4-3: example of IOT due to SBFD additional OBUE leakage

| Blocking level (dBm/100MHz) | SBFD additional OBUE leakage ratio (with 35 dB attenuation filtering) | Noise floor (dBm/40MHz) | IOT (dB) |
| --- | --- | --- | --- |
| 5 | 85 | -93 | 9 |
| 0 | 85 | -93 | 4 |
| -10 | 85 | -93 | -6 |
| -20 | 85 | -93 | -16 |
| -30 | 85 | -93 | -26 |
| -40 | 85 | -93 | -36 |

Observation 2: to guarantee the adjacent channel co-existence between SBFD BSs, both blocking level at receiver side and OBUE leakage at transmitter side should be improved.

As discussed in [R4-2413284], some simulation result is shown in table 2.4-2 for information with following assumption.

  - The scenarios to be studied are Scenario 1, defined in TR 38.858

  - The case to be studied is Case 3

  - absolute power level of interference from the DL adjacent channel interference

  - the reference point: before array gain

  - Grid-shift: Option-1: 10%, 20%, 50%, 100%

  - Percentage values: 99%

Table 2.4-4: blocking level @ 99%CDF

| Blocking level(dBm) |  |
| --- | --- |
| Grid-shift | 99% CDF |
| 10% | -5.1 |
| 20% | -12.4 |
| 50% | -22.4 |
| 100% | -25.8 |

In this case a new classification has to be introduced because the adjacent channel interference is generated in a different manner. The classification, based on the amount of BS-to-BS interference, the most destructive interference due to the presence of a not synchronised operator, is the following:

-	worst case scenario: 0 m base station shifting (co-siting);

-	intermediate case scenario: 577/2 m base station shifting;

-	best case scenario: 577 m base station shifting.

(Text in TS 25.942)

To answer the question on whether two set of requirements are needed or not, it is worth discussing which grid shift is used for the in-band blocking evaluation. In the simulation 10% grid shift means 100% SBFD BS would suffer the blocker from the adjacent BS at only 28.8 meters, and 20% grid shift means 57.6 meters. Meanwhile it cannot reflect real deployments. In our statistics on the dense urban macro deployments for C band, it is about 45% BSs which there is another operator’s BSs deployed less than 50 meters away, including co-location cases. If removing the co-location cases, the percentage will decrease further. So it is observed that using 10% grid shift as simulation assumption will result over stringent requirement. As a reference in 3G system, 50% grid shift (i.e. intermediate case scenario) is adopted to derive co-existence requirements for BS-BS interference.

Observation 3: 10% grid shift cannot reflect real deployments. And in 3G system, 50% grid shift (i.e. intermediate case scenario) is adopted to derive co-existence requirements for BS-BS interference.

Proposal 5: it is proposed to consider 20%-50% grid shift for the simulation.

The simulation results from companies is summarized in the following table for FR1cases [5]. To take the average for the results of 20% grid shift and 50% grid shift so far we can get the average value -17 dBm.

| Scenario | Grid shift | ZTE | Huawei | Samsung | Ericsson case1 | Ericsson case2 | Nokia | Qualcomm | CATT | Tejas Networks[baseline] | Tejas Networks[Reduced blocker] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10% | -1.5067 | -5.1 | -4.39 | -4.7 | -0.5 | -2.5 |  | 0.33 | -0.82 | -36.09 |
|  | 20% | -7.4879 | -12.4 | -11.01 |  |  |  |  |  |  |  |
|  | 50% | -17.4928 | -22.4 | -21.48 |  |  |  | -26.6 |  |  |  |
|  | 100% | -25.3301 | -25.8 | -27.76 | -23.4 | -20.4 |  |  | -25.17 | -27.47 |  |
| 3 | NA | -42.4091 |  |  | -40.3 |  | -36.1 |  | -34.42 |  |  |
| 5 | 10% | -15.4155 |  |  | -10 | -7.5 | -4.3 |  | -17.24 |  |  |
|  | 20% | -22.3199 |  |  |  |  |  |  |  |  |  |
|  | 50% | -32.3749 |  |  |  |  |  |  |  |  |  |
|  | 100% | -38.9889 |  |  | -39.4 | -31.5 |  |  | -32.19 |  |  |

2.5 Receiver intermodulation

Per WF in [4],

Necessity of New RX intermodulation requirement with 1 interfering signal WF: - FFS on the followings: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to study further RX intermodulation scenario with 1 interfering signal. Necessity of New RX intermodulation requirement with 2 interfering signals WF: - FFS on the following: - Option 1: - For the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - For receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: New IMD requirement with 2 interfering signal scenario is not needed. - Option 3: RAN4 to study further

The motivation of the two new RX intermodulation requirements are to consider BS2BS cross link interference. For general in-band blocking case as discussed in clause 2.4, the stronger intermodulation product from closer in-band blocking blocker and self sub-band DL TX is already covered. Hence there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s).

Proposal 6: there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s). The two new requirement is implicitly captured by the SBFD RX blocking requirement.

Conclusion

In this contribution we discussed on the BS RX requirement for SBFD. We have the following observations and proposals:

Proposal 1: for OTA reference sensitivity, it is proposed to add 1 dB addition to reference sensitivity due to self-interference.

Proposal 2: For FR1 BS 20 dB IoT level assumption is reused for dynamic range requirement.

Proposal 3: it is proposed to perform simulation to check whether larger dynamic range is needed for FR2 SBFD.

Proposal 4: the definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level.

Observation 1: without improving the legacy TDD unwanted emission, only defining high blocking receiver requirement cannot guarantee the adjacent channel co-existence between a SBFD BS and a legacy TDD BS.

Observation 2: to guarantee the adjacent channel co-existence between SBFD BSs, both blocking level at receiver side and OBUE leakage at transmitter side should be improved.

Observation 3: 10% grid shift cannot reflect real deployments. And in 3G system, 50% grid shift (i.e. intermediate case scenario) is adopted to derive co-existence requirements for BS-BS interference.

Proposal 5: it is proposed to consider 20%-50% grid shift for the simulation.

Proposal 6: there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s). The two new requirement is implicitly captured by the SBFD RX blocking requirement.

- References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958, Way Forward for [111][313] NR_duplex_evo,	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416528, Topic summary for [112bis][307] NR_duplex_evo_BSRF, Moderator (Huawei)